| Change Request |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Document | ORAN-WG3.E2SM-RC | ver | 01.03.01 | CR | QCM-0001 | rev | 1 |  |

| Title: | E1AP Spec Number Reference Correction |  |  |
| --- | --- | --- | --- |
| Source to WG: | Qualcomm |  |  |
| Target WG : | WG3 |  |  |
| Category: | D | CR Creation Date | 2022.09.08 |
|  | Use one of the following categories: A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) F (correction)’ Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |

| Reason for Change: | 3GPP transferred the technical contents of E1AP 38.463 to 37.483, effective for release 17 |
| --- | --- |
| Summary of change: | Change to ref [21] |
| Consequences if not aproved: | Ref [21] no longer valid |

| Clauses affected: | 2, 8.1, 8.4, 8.5 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | N |  |  |  |
| Other specs |  | Y | Other core specifications: | <fill in related CRs if “Y”> |  |
| affected: |  | x | Test specifications: | <fill in related CRs if “Y”> |  |
| (show related CRs) |  | X | O&M Specifications: | <fill in related CRs if “Y”> |  |
| Supporting material: |  |  |  |  |  |
| Other comments: | xxx |  |  |  |  |

| Status: | Open | CR Closed Date: | <CR closed date> |
| --- | --- | --- | --- |
| Outcome: | Approved or Rejected or Deferred | Duplication: | <dupl. CR number> |
| Outcome explanation: | <document editor provides the explanation of the outcome, especially justifying a “rejected” outcome (duplicate CR is a valid reason for rejection) > |  |  |

********** FIRST MODIFIED SECTION **********

# 2	References

[1]	3GPP TR 21.905: “Vocabulary for 3GPP Specifications”.

[2]	O-RAN Working Group 3, Near-Real-time RAN Intelligent Controller, Architecture & E2 General Aspects and Principles (E2GAP)

[3]	ORAN Working Group 3, Near-Real-time RAN Intelligent Controller, E2 Application Protocol (E2AP).

[4]	O-RAN Working Group 3, Near-Real-time RAN Intelligent Controller, E2 Service Model (E2SM)

[5]	3GPP TR 25.921: "Guidelines and principles for protocol description and error handling".

[6]	ITU-T Recommendation X.680 (2002-07): "Information technology – Abstract Syntax Notation One (ASN.1): Specification of basic notation".

[7]	ITU-T Recommendation X.681 (2002-07): "Information technology – Abstract Syntax Notation One (ASN.1): Information object specification".

[8]	ITU-T Recommendation X.691 (2002-07): "Information technology - ASN.1 encoding rules - Specification of Packed Encoding Rules (PER) "

[9]	Reserved

[10]	TS 38.410: “NG-RAN; NG general aspects and principles”.

[11]	TS 38.413: “NG-RAN; NG Application Protocol (NGAP)”.

[12]	Reserved

[13]	TS 36.413: “E-UTRAN; S1 Application Protocol (S1AP)”.

[14]	TS 38.420: “NG-RAN; Xn general aspects and principles”.

[15]	TS 38.423: “NG-RAN; Xn Application Protocol (XnAP)”.

[16]	TS 36.420: “E-UTRAN; X2 general aspects and principles”.

[17]	TS 36.423: “E-UTRAN; X2 Application Protocol (X2AP)”.

[18]	TS 38.470: “NG-RAN; F1 general aspects and principles”.

[19] 	TS 38.473: “NG-RAN; F1 Application Protocol (F1AP)”.

[20]	TS 38.460: “NG-RAN; E1 general aspects and principles”.

[21]	TS 37.483: “NG-RAN; E1 Application Protocol (E1AP)”.

[22]	TS 38.331: “NR; Radio Resource Control (RRC) Protocol Specification”.

[23]		TS 36.331: “E-UTRA; Radio Resource Control (RRC) Protocol Specification”.

[24]	TS 38.323: “NR; Packet Data Convergence Protocol (PDCP) specification”.

[25]	TS 38.322: “NR; Radio Link Control (RLC) protocol specification”.

[26]	TS 38.321: “NR; Medium Access Control (MAC) protocol specification”.

[27]	TS 25.321: “Medium Access Control (MAC) protocol specification”.

[28]	TS 28.552: “Management and orchestration 5G performance measurements”.

[29]	Reserved

[30]	IETF RFC 5905 (2010-06): “Network Time Protocol Version 4: Protocol and Algorithms Specification”

[31]	TS 37.340: “E-UTRA and NR Multiconnectivity Stage 2”.

********** END OF MODIFIED SECTION**********

********** NEXT MODIFIED SECTION **********

## 8.1	RAN parameters for Event Trigger

The RAN Parameters pertaining to “Event Trigger” that are used across multiple service styles are listed here. All RAN Parameters defined in section 8.1 may also be used to define Policy Condition, see also section 8.5.

    - 8.1.1	Common RAN Parameters

This section contains the common set of RAN parameters that can be accessed via RAN parameters defined in each service style across multiple services.

#### 8.1.1.5	NG-RAN Data Radio Bearer

The following RAN Parameters are associated with the NG-RAN data radio bearer.

| RAN Parameter ID | RAN Parameter | RAN Parameter Value Type | Key Flag | RAN Parameter Definition | Semantics Description |
| --- | --- | --- | --- | --- | --- |
| 14001 | 5QI | ELEMENT | FALSE | 5QI IE in TS 37.483 [21] Section 9.3.1.27 or TS 37.483 [21] Section 9.3.1.28 |  |
| 14002 | Packet Delay Budget | ELEMENT | FALSE | Packet Delay Budget IE in TS 37.483 [21] Section 9.3.1.47 |  |
| 14003 | Packet Error Rate | ELEMENT | FALSE | Packet Error Rate IE in TS 37.483 [21] Section 9.3.1.48 |  |
| 14004 | NG-RAN DRB Allocation and Retention Priority | STRUCTURE |  |  | NG-RAN Allocation and Retention Priority IE in TS 37.483 [21] Section 9.3.1.29 |
| 14005 | >Priority Level | ELEMENT | FALSE | Priority Level IE in TS 37.483 [21] Section 9.3.1.29 |  |
| 14006 | >Pre-emption Capability | ELEMENT | FALSE | Pre-emption Capability IE in TS 37.483 [21] Section 9.3.1.29 |  |
| 14007 | >Pre-emption Vulnerability | ELEMENT | FALSE | Pre-emption Vulnerability IE in TS 37.483 [21] Section 9.3.1.29 |  |
| 14008 | Priority Level of the mapped QoS flows | ELEMENT | FALSE | Priority Level IE in TS 37.483 [21] Section 9.3.1.51 |  |
| 14009 | QoS parameters for GBR flows in NG-RAN Bearer | STRUCTURE |  |  | GBR QoS Flow Information IE in TS 37.483 [21] Section 9.3.1.30 |
| 14010 | >Maximum Flow Bit Rate Downlink | ELEMENT | FALSE | Bit Rate IE in TS 37.483 [21] Section 9.3.1.30 |  |
| 14011 | >Guaranteed Flow Bit Rate Downlink | ELEMENT | FALSE | Bit Rate IE in TS 37.483 [21] Section 9.3.1.30 |  |
| 14012 | >Maximum Packet Loss Rate Downlink | ELEMENT | FALSE | Packet Loss Rate IE in TS 37.483 [21] Section 9.3.1.30 |  |
| 14013 | >Maximum Flow Bit Rate Uplink | ELEMENT | FALSE | Bit Rate IE in TS 37.483 [21] Section 9.3.1.30 |  |
| 14014 | >Guaranteed Flow Bit Rate Uplink | ELEMENT | FALSE | Bit Rate IE in TS 37.483 [21] Section 9.3.1.30 |  |
| 14015 | >Maximum Packet Loss Rate Uplink | ELEMENT | FALSE | Packet Loss Rate IE in TS 37.483 [21] Section 9.3.1.30 |  |
| 14016 | QoS Monitoring Enable Request | ELEMENT | FALSE | QoS Monitoring Request IE in TS 37.483 [21] Section 9.3.1.26 |  |
| 14017 | QoS Monitoring Reporting Frequency | ELEMENT | FALSE | QoS Monitoring Reporting Frequency IE in TS 37.483 [21] Section 9.3.1.26 |  |
| 14018 | QoS Monitoring Disabled | ELEMENT | FALSE | QoS Monitoring Disabled IE in TS 37.483 [21] Section 9.3.1.26 |  |
| 14019 | Reflective QoS Mapping | ELEMENT | FALSE | RDI IE in TS 37.483 [21] Section 9.3.1.26 |  |
| 14101 | List of cell groups to be added | LIST |  |  | Cell Group To Add IE in TS 37.483 [21] Section 9.3.3.11 |
| 14102 | >Cell group item | STRUCTURE |  |  | Cell Group Item IE in TS 37.483 [21] Section 9.3.1.11 |
| 14103 | >>Cell Group ID | ELEMENT | TRUE | Cell Group ID IE in TS 37.483 [21] Section 9.3.1.11 |  |
| 14104 | >>Cell Group | STRUCTURE |  | 8.1.1.7 |  |
| 14201 | SDAP Configuration | STRUCTURE |  |  | SDAP Configuration IE in TS 37.483 [21] Section 9.3.1.39 |
| 14202 | >PDU Session ID | ELEMENT | TRUE | PDU-SessionID IE in TS 38.331 [22] Sec 6 |  |
| 14203 | >Default DRB | ELEMENT | FALSE | Default DRB IE in TS 37.483 [21] Section 9.3.1.39 |  |
| 14210 | >PDU Session | STRUCTURE |  | 8.1.1.16 |  |
| 14301 | PDCP Configuration | STRUCTURE |  |  | PDCP Configuration IE in TS 37.483 [21] Section 9.3.1.38 |
| 14302 | >RLC mode | ELEMENT | FALSE | RLC mode IE in TS 37.483 [21] Section 9.3.1.38 |  |
| 14303 | >PDCP Duplication | ELEMENT | FALSE | PDCP Duplication IE in TS 37.483 [21] Section 9.3.1.38 |  |
| 14304 | >UL Data Split Threshold | ELEMENT | FALSE | UL Data Split Threshold IE in TS 37.483 [21] Section 9.3.1.43 |  |
| 14305 | >PDCP Re-establishment | ELEMENT | FALSE | PDCP Re-establishment IE in TS 37.483 [21] Section 9.3.1.38 |  |
| 14306 | >PDCP Data Recovery | ELEMENT | FALSE | PDCP Data Recovery IE in TS 37.483 [21] Section 9.3.1.38 |  |
| 14307 | >Out-of-Order Delivery | ELEMENT | FALSE | Out of Order Delivery IE in TS 37.483 [21] Section 9.3.1.38 |  |
| 14308 | >PDCP Status Report Indication | ELEMENT | FALSE | PDCP Status Report Indication IE in TS 37.483 [21] Section 9.3.1.38 |  |
| 14309 | >Number of PDCP duplication | ELEMENT | FALSE | Additional PDCP duplication Information IE in TS 37.483 [21] Section 9.3.1.38 |  |
| 14310 | >UL More than one RLC | STRUCTURE |  |  | moreThanOneRLC IE in TS 38.331 [22] Section 6 |
| 14311 | >>Primary Path | STRUCTURE |  |  | primaryPath IE in TS 38.331 [22] Section 6 |
| 14312 | >>>Cell Group ID | ELEMENT | TRUE | cellGroup IE in TS 38.331 [22] Section 6 |  |
| 14313 | >>>Logical Channel ID | ELEMENT | FALSE | logicalChannelIdentity IE in TS 38.331 [22] Sec 6 |  |
| 14314 | >UL More than two RLC | STRUCTURE |  |  | moreThanTwoRLC-DRB-r16 IE in TS 38.331 [22] Section 6 |
| 14315 | >>Split Secondary Path | ELEMENT | FALSE | splitSecondaryPath IE in TS 38.331 [22] Sec 6 |  |
| 14316 | >>Duplication State | ELEMENT | FALSE | duplicationState IE in TS 38.331 [22] Sec 6 |  |

#### 8.1.1.6	QoS flow

The following RAN Parameters are associated with the NG-RAN QoS flow.

| RAN Parameter ID | RAN Parameter | RAN Parameter Value Type | Key Flag | RAN Parameter Definition | Semantics Description |
| --- | --- | --- | --- | --- | --- |
| 15001 | 5QI | ELEMENT | FALSE | 5QI IE in TS 37.483 [21] Section 9.3.1.27 or TS 37.483 [21] Section 9.3.1.28 |  |
| 15002 | Packet Delay Budget | ELEMENT | FALSE | Packet Delay Budget IE in TS 37.483 [21] Section 9.3.1.47 |  |
| 15003 | Packet Error Rate | ELEMENT | FALSE | Packet Error Rate IE in TS 37.483 [21] Section 9.3.1.48 |  |
| 15004 | NG-RAN DRB Allocation and Retention Priority | STRUCTURE |  |  | NG-RAN Allocation and Retention Priority IE in TS 37.483 [21] Section 9.3.1.29 |
| 15005 | >Priority Level | ELEMENT | FALSE | Priority Level IE in TS 37.483 [21] Section 9.3.1.29 |  |
| 15006 | >Pre-emption Capability | ELEMENT | FALSE | Pre-emption Capability IE in TS 37.483 [21] Section 9.3.1.29 |  |
| 15007 | >Pre-emption Vulnerability | ELEMENT | FALSE | Pre-emption Vulnerability IE in TS 37.483 [21] Section 9.3.1.29 |  |
| 15008 | Priority Level of the mapped QoS flows | ELEMENT | FALSE | Priority Level IE in TS 37.483 [21] Section 9.3.1.51 |  |
| 15009 | QoS parameters for GBR flows in NG-RAN Bearer | STRUCTURE |  |  | GBR QoS Flow Information IE in TS 37.483 [21] Section 9.3.1.30 |
| 15010 | >Maximum Flow Bit Rate Downlink | ELEMENT | FALSE | Bit Rate IE in TS 37.483 [21] Section 9.3.1.30 |  |
| 15011 | >Guaranteed Flow Bit Rate Downlink | ELEMENT | FALSE | Bit Rate IE in TS 37.483 [21] Section 9.3.1.30 |  |
| 15012 | >Maximum Packet Loss Rate Downlink | ELEMENT | FALSE | Packet Loss Rate IE in TS 37.483 [21] Section 9.3.1.30 |  |
| 15013 | Maximum Flow Bit Rate Uplink | ELEMENT | FALSE | Bit Rate IE in TS 37.483 [21] Section 9.3.1.30 |  |
| 15014 | >Guaranteed Flow Bit Rate Uplink | ELEMENT | FALSE | Bit Rate IE in TS 37.483 [21] Section 9.3.1.30 |  |
| 15015 | >Maximum Packet Loss Rate Uplink | ELEMENT | FALSE | Packet Loss Rate IE in TS 37.483 [21] Section 9.3.1.30 |  |
| 15016 | QoS Monitoring Enable Request | ELEMENT | FALSE | QoS Monitoring Request IE in TS 37.483 [21] Section 9.3.1.26 |  |
| 15017 | QoS Monitoring Reporting Frequency | ELEMENT | FALSE | QoS Monitoring Reporting Frequency IE in TS 37.483 [21] Section 9.3.1.26 |  |
| 15018 | QoS Monitoring Disabled | ELEMENT | FALSE | QoS Monitoring Disabled IE in TS 37.483 [21] Section 9.3.1.26 |  |
| 15019 | Reflective QoS Mapping | ELEMENT | FALSE | RDI IE in TS 37.483 [21] Section 9.3.1.26 |  |
| 15020 | Redundant QoS Flow Indicator | ELEMENT | FALSE | Redundant QoS Flow Indicator IE in TS 37.483 [21] Section 9.3.1.74 |  |

#### 8.1.1.7	Cell Group

The following RAN Parameters are associated with the NR cell group item.

| RAN Parameter ID | RAN Parameter | RAN Parameter Value Type | Key Flag | RAN Parameter Definition | Semantics Description |
| --- | --- | --- | --- | --- | --- |
| 15502 | UL Configuration | ELEMENT | FALSE | UL Configuration IE in TS 37.483 [21] Section 9.3.1.11 |  |
| 15503 | RAT Type | ELEMENT | FALSE | RAT Type IE in TS 37.483 [21] Section 9.3.1.11 |  |
| 15504 | Number of Tunnels | ELEMENT | FALSE | Number of Tunnels IE in TS 37.483 [21] Section 9.3.1.11 |  |

#### 8.1.1.11	E2 Node

The following RAN Parameters are associated with the E2 node.

| RAN Parameter ID | RAN Parameter | RAN Parameter Value Type | Key Flag | RAN Parameter Definition | Semantics Description |
| --- | --- | --- | --- | --- | --- |
| 17001 | CHOICE E2 Node Component Type | STRUCTURE |  |  | O-RAN WG3 E2AP Section 9.2.26 |
| 17010 | >NG-RAN gNB | STRUCTURE |  |  | O-RAN WG3 E2AP Section 9.2.27 |
| 17011 | >>Global gNB ID | STRUCTURE |  |  | TS 38.413 [11] Section 9.3.1.6 |
| 17012 | >>>PLMN Identity | ELEMENT | FALSE | PLMN Identity IE in TS 38.413 [11] Section 9.3.3.5 |  |
| 17013 | >>>CHOICE gNB ID | STRUCTURE |  |  | gNB ID IE in TS 38.413 [11] Section 9.3.1.6 |
| 17014 | >>>>gNB ID | STRUCTURE |  |  | gNB ID IE in TS 38.413 [11] Section 9.3.1.6 |
| 17015 | >>>>>gNB ID | ELEMENT | FALSE | gNB ID IE in TS 38.413 [11] Section 9.3.1.6 |  |
| 17016 | >>List of NR served cells | LIST |  |  | List of Served Cells NR IE in TS 38.423 [15] Section 9.1.3.1 |
| 17017 | >>>Served NR cell item | STRUCTURE |  |  | Individual cell item in the List of Served Cells NR |
| 17028 | >>>>Served NR cell | STRUCTURE |  | NR Cell IE in 8.1.1.1 |  |
| 17018 | >>Count of list of NR served cells | ELEMENT | FALSE | INTEGER (1..16384) |  |
| 17019 | >>List of NR neighbor cells | LIST |  |  | Neighbor Information IE in TS 38.423 [15] Section 9.2.2.13 |
| 17020 | >>>NR neighbor cell item | STRUCTURE |  |  | Individual cell item in the Neighor Information IE NR |
| 17031 | >>>>Neighbor NR cell | STRUCTURE |  | NR Cell IE in 8.1.1.1 |  |
| 17021 | >>Number of RRC connections | ELEMENT | FALSE | Number of RRC connections IE in TS 38.423 [15] Section 9.2.2.57 |  |
| 17022 | >>gNB Performance Measurements | STRUCTURE |  | 8.1.1.15 |  |
| 17050 | >en-gNB | STRUCTURE |  |  | O-RAN WG3 E2AP Section 9.2.27 |
| 17051 | >>Global en-gNB ID | STRUCTURE |  |  | TS 36.423 [17] Section 9.2.112 |
| 17052 | >>>PLMN Identity | ELEMENT | FALSE | PLMN Identity IE in TS 36.423 [17] Section 9.2.4 |  |
| 17053 | >>>CHOICE en-gNB ID | STRUCTURE |  | en-gNB ID IE in TS 36.423 [17] Section 9.2.112 |  |
| 17054 | >>>>en-gNB ID | STRUCTURE |  | en-gNB ID IE in TS 36.423 [17] Section 9.2.112 |  |
| 17055 | >>>>>en-gNB ID | ELEMENT | FALSE | en-gNB ID IE in TS 36.423 [17] Section 9.2.112 |  |
| 17056 | >>List of served NR cells | LIST |  |  | List of Served Cells NR IE in TS 38.423 [15] Section 9.1.3.1 |
| 17057 | >>>Served NR cell item | STRUCTURE |  |  | Individual cell item in the List of Served Cells NR IE |
| 17068 | >>>>Served NR cell | STRUCTURE |  | NR Cell IE in 8.1.1.1 |  |
| 17058 | >>Count of list of NR served cells | ELEMENT | FALSE | INTEGER (1..16384) |  |
| 17059 | >>List of NR neighbour cells | LIST |  |  | Neighbor Information IE in TS 38.423 [15] Section 9.2.2.13 |
| 17060 | >>>NR neighbor cell item | STRUCTURE |  |  | Individual cell item in Neighbor Information IE |
| 17065 | >>>>NR neighbor cell | STRUCTURE |  | NR Cell IE in 8.1.1.1 |  |
| 17061 | >>gNB Performance Measurements | STRUCTURE |  | 8.1.1.15 | gNB Measurements IE in Sec 8.1.1.15 |
| 17070 | >ng-eNB | STRUCTURE |  |  | O-RAN WG3 E2AP Section 9.2.27 |
| 17071 | >>ng-eNB ID | STRUCTURE |  |  | ng-eNB ID IE TS 38.413 [11] Section 9.2.2.2 |
| 17072 | >>>PLMN Identity | ELEMENT | FALSE | PLMN Identity IE in TS 38.413 [11] Section 9.3.1.6 |  |
| 17073 | >>>CHOICE ng-eNB ID | STRUCTURE |  |  | Ng-eNB ID IE in TS 38.413 [11] Section 9.2.2.2 |
| 17074 | >>>>Macro ng-eNB ID | STRUCTURE |  |  | Macro ng-eNB ID IE in TS 38.413 [11] Section 9.2.2.2 |
| 17075 | >>>>>Macro ng-eNB ID | ELEMENT | FALSE | Macro ng-eNB ID IE in TS 38.413 [11] Section 9.2.2.2 |  |
| 17076 | >>>>Short Macro ng-eNB ID | STRUCTURE |  |  | Short Macro ng-eNB ID IE in TS 38.413 [11] Section 9.2.2.2 |
| 17077 | >>>>>Short Macro ng-eNB ID | ELEMENT | FALSE | Short Macro ng-eNB ID IE in TS 38.413 [11] Section 9.2.2.2 |  |
| 17078 | >>>>Long Macro ng-eNB ID | STRUCTURE |  |  | Long Macro ng-eNB ID IE in TS 38.413 [11] Section 9.2.2.2 |
| 17079 | >>>>>Long Macro ng-eNB ID | ELEMENT | FALSE | Long Macro ng-eNB ID IE in TS 38.413 [11] Section 9.2.2.2 |  |
| 17080 | >>List of E-UTRA served cells | LIST |  |  | List of Served Cells E-UTRA IE in TS 38.423 [15] Section 9.1.3.1 |
| 17081 | >>>Served E-UTRA cell item | STRUCTURE |  |  | Individual cell item in List of Served Cells E-UTRA IE |
| 17092 | >>>>Served E-UTRA cell | STRUCTURE |  | 8.1.1.2 |  |
| 17082 | >>Count of list of E-UTRA served cells | ELEMENT | FALSE | INTEGER (0..16384) |  |
| 17083 | >>List of E-UTRA neighbor cells | LIST |  |  | Neighbor Information E-UTRA IE in TS 38.423 [15] Section 9.2.2.14 |
| 17084 | >>>E-UTRA neighbor cell item | STRUCTURE |  |  | Individual cell item in Neighbor Information E-UTRA IE |
| 17095 | >>>>E-UTRA neighbor cell | STRUCTURE |  | 8.1.1.2 |  |
| 17085 | >>Number of RRC connections | ELEMENT | FALSE | Number of RRC connections IE in TS 38.423 [15] Section 9.2.2.57 |  |
| 17086 | >>eNB Performance Measurements | STRUCTURE |  | 8.1.1.18 |  |
| 17100 | >NG-RAN gNB CU-CP | STRUCTURE |  |  | O-RAN WG3 E2AP Section 9.2.27 |
| 17101 | >>Global gNB CU-CP ID | STRUCTURE |  |  | TS 38.413 [11] Section 9.3.1.6 |
| 17102 | >>>PLMN Identity | ELEMENT | FALSE | PLMN Identity IE in TS 38.413 [11] Section 9.3.3.5 |  |
| 17103 | >>>CHOICE gNB ID | STRUCTURE |  |  | gNB ID IE in TS 38.413 [11] Section 9.3.1.6 |
| 17104 | >>>>gNB ID | STRUCTURE |  |  | gNB ID IE in TS 38.413 [11] Section 9.3.1.6 |
| 17105 | >>>>>gNB ID | ELEMENT | FALSE | gNB ID IE in TS 38.413 [11] Section 9.3.1.6 |  |
| 17107 | >>List of activated NR cells | LIST |  |  | Cells to be Activated List IE in TS 38.473 [19] Section 9.2.1.10 |
| 17108 | >>>NR cell item | STRUCTURE |  |  | Cells to be Activated Item IE in TS 38.473 [19] Section 9.2.1.10 |
| 17109 | >>>>NR cell | STRUCTURE |  | NR Cell IE in 8.1.1.1 |  |
| 17110 | >>>>Available PLMN List | LIST |  |  | Available PLMN List IE in TS 38.473 [19] Section 9.2.1.10 |
| 17111 | >>>>>PLMN Item | STRUCTURE |  |  | Available PLMN Item IEs IE in TS 38.473 [19] Section 9.3.1.65 |
| 17112 | >>>>>>PLMN ID | ELEMENT | FALSE | PLMN Identity IE in TS 38.413 [11] Section 9.3.3.5 |  |
| 17113 | >>List of de-activated NR cells | LIST |  |  | Cells to be Deactivated List IE in TS 38.473 [19] Section 9.2.1.10 |
| 17114 | >>>NR cell item | STRUCTURE |  |  | Individual cell item in Cells to be Deactivated List IE |
| 17515 | >>>>NR cell | STRUCTURE |  | NR Cell IE in 8.1.1.1 |  |
| 17115 | >>List of barred NR cells | LIST |  |  | Cells to be barred List IE in TS 38.473 [19] Section 9.2.1.10 |
| 17116 | >>>NR cell item | STRUCTURE |  |  | Individual cell item in Cells to be barred List IE |
| 17517 | >>>>NR cell | STRUCTURE |  | NR Cell IE in 8.1.1.1 |  |
| 17117 | >>Count of list of NR served cells | ELEMENT | FALSE | INTEGER (0..16384) |  |
| 17118 | >>List of NR neighbor cells | LIST |  |  | Neighbour Cell Information List IE in TS 38.473 [19] Section 9.2.1.10 |
| 17119 | >>>NR neighbor cell item | STRUCTURE |  |  | Individual Cell Item in Neighbour Cell Information List IE |
| 17520 | >>>>NR neighbor cell | STRUCTURE |  | NR Cell IE in 8.1.1.1 |  |
| 17120 | >>Number of RRC connected UEs | ELEMENT | FALSE | Number of RRC connections IE in TS 38.423 [15] Section 9.2.2.57 |  |
| 17121 | >>gNB CU-CP Performance Measurements | STRUCTURE |  | 8.1.1.12 |  |
| 17130 | >NG-RAN gNB DU | STRUCTURE |  |  | O-RAN WG3 E2AP Section 9.2.27 |
| 17132 | >>gNB-DU ID | ELEMENT | FALSE | gNB-DU ID IE in TS 38.473 [19] Section 9.3.1.9 |  |
| 17133 | >>List of NR served cells | LIST |  |  | List of Served Cells NR IE in TS 38.423 [15] Section 9.1.3.1 |
| 17134 | >>>Served NR cell item | STRUCTURE |  |  | Individual cell item in List of Served Cells NR IE |
| 17535 | >>>>Served NR cell | STRUCTURE |  | NR Cell IE in 8.1.1.1 |  |
| 17135 | >>gNB DU Measurements | STRUCTURE |  | 8.1.1.14 |  |
| 17140 | >NG-RAN gNB CU-UP | STRUCTURE |  |  | O-RAN WG3 E2AP Section 9.2.27 |
| 17141 | >>gNB CU-UP ID | ELEMENT | TRUE | gNB-CU-UP ID IE in TS 37.483 [21] Section 9.3.1.15 |  |
| 17142 | >>gNB CU-UP Capacity | ELEMENT | FALSE | gNB-CU-UP Capacity IE in TS 37.483 [21] Section 9.3.1.56 |  |
| 17143 | >>gNB CU-UP Performance Measurements | STRUCTURE |  | 8.1.1.13 |  |
| 17150 | >eNB | STRUCTURE |  |  | O-RAN WG3 E2AP Section 9.2.27 |
| 17151 | >>eNB ID | STRUCTURE |  |  | eNB ID IE TS 36.423 [17] Section 9.2.2.2 |
| 17152 | >>>PLMN Identity | ELEMENT | FALSE | PLMN Identity IE in TS 36.423 [17] Section 9.2.4 |  |
| 17153 | >>>CHOICE eNB ID | STRUCTURE |  |  | Global eNB ID IE in TS 36.423 [17] Section 9.2.22 |
| 17154 | >>>>Macro eNB ID | STRUCTURE |  |  | Macro eNB ID IE in TS 36.423 [17] Section 9.2.22 |
| 17155 | >>>>>Macro eNB ID | ELEMENT | FALSE | Macro eNB ID IE in TS 36.423 [17] Section 9.2.22 |  |
| 17156 | >>>>Home eNB ID | STRUCTURE |  |  | Home eNB ID IE in TS 36.423 [17] Section 9.2.22 |
| 17157 | >>>>>Home eNB ID | ELEMENT | FALSE | Home eNB ID IE in TS 36.423 [17] Section 9.2.22 |  |
| 17158 | >>>>Short Macro eNB ID | STRUCTURE |  |  | Short Macro eNB ID IE in TS 36.423 [17] Section 9.2.22 |
| 17159 | >>>>>Short Macro eNB ID | ELEMENT | FALSE | Short Macro eNB ID IE in TS 36.423 [17] Section 9.2.22 |  |
| 17160 | >>>>Long Macro eNB ID | STRUCTURE |  |  | Long Macro eNB ID IE in TS 36.423 [17] Section 9.2.22 |
| 17161 | >>>>>Long Macro eNB ID | ELEMENT | FALSE | Long Macro eNB ID IE in TS 36.423 [17] Section 9.2.22 |  |
| 17162 | >>List of E-UTRA served cells | LIST |  |  | List of Served Cells E-UTRA IE in TS 38.423 [15] Section 9.1.3.1 |
| 17163 | >>>Served E-UTRA cell item | STRUCTURE |  |  | Individual cell item in List of Served Cells E-UTRA IE |
| 17564 | >>>>Served E-UTRA cell | STRUCTURE |  | 8.1.1.2 |  |
| 17164 | >>Count of list of E-UTRA served cells | ELEMENT | FALSE | INTEGER (0..16384) |  |
| 17165 | >>List of E-UTRA neighbor cells | LIST |  |  | Neighbor Information E-UTRA IE in TS 38.423 [15] Section 9.2.2.14 |
| 17166 | >>>E-UTRA neighbor cell item | STRUCTURE |  |  | Individual cell item in Neighbor Information E-UTRA IE |
| 17567 | >>>>E-UTRA neighbor cell | STRUCTURE |  | 8.1.1.2 |  |
| 17167 | >>Number of RRC connected UEs | ELEMENT | FALSE | Number of RRC connections IE in TS 38.423 [15] Section 9.2.2.57 |  |
| 17168 | >>List of NR neighbor cells | LIST |  |  | Neighbor Information IE in TS 38.423 [15] Section 9.2.2.13 |
| 17169 | >>>NR neighbor cell item | STRUCTURE |  |  | Individual cell item in Neighbor Information IE |
| 17570 | >>>>NR neighbor cell | STRUCTURE |  | 8.1.1.1 |  |
| 17170 | >>eNB Performance Measurements | STRUCTURE |  | 8.1.1.18 |  |
| 17200 | TAI Support List | LIST |  |  | TAI Support List IE in TS 38.423 [15] Section 9.2.3.20 |
| 17201 | >TAI Support Item | STRUCTURE |  |  | TAI Support Item IE in TS 38.423 [15] Section 9.2.3.20 |
| 17202 | >>TAC | ELEMENT | FALSE | TAC IE in TS 38.423 [15] Section 9.2.2.5 |  |
| 17203 | >>List of Broadcast PLMNs | LIST |  |  | Broadcast PLMNs IE in TS 38.423 [15] Section 9.2.3.20 |
| 17204 | >>>Broadcast PLMN Item | STRUCTURE |  |  | Broadcast PLMNs IE in TS 38.423 [15] Section 9.2.3.20 |
| 17205 | >>>PLMN Identity | ELEMENT | FALSE | PLMN Identity IE in TS 38.413 [11] Section 9.3.3.5 |  |
| 17206 | >>>TAI Slice Support List | LIST |  |  | TAI Slice Support List IE in TS 38.423 [15] Section 9.2.3.20 |
| 17207 | >>>>Slice Support Item | STRUCTURE |  |  | Slice Support Item IE in TS 38.423 [15] Section 9.2.3.20 |
| 17208 | >>>>>S-NSSAI | STRUCTURE |  |  | S-NSSAI IE in TS 38.473 [19] Section 9.3.1.38 |
| 17209 | >>>>>>SST | ELEMENT | TRUE | SST IE in TS 38.473 [19] Section 9.3.1.38 |  |
| 17210 | >>>>>>SD | ELEMENT | FALSE | SD IE in TS 38.473 [19] Section 9.3.1.38 |  |
| 17211 | >>>Count of TAI Slice Support List | ELEMENT | FALSE | INTEGER (0..1024) |  |
| 17212 | >>>NR CGI Support List | LIST |  |  | List of Served Cells NR IE in TS 38.423 [15] Section 9.1.3.1 |
| 17213 | >>>>NR CGI Cell Support Item | STRUCTURE |  |  | Individual cell item in List of Served Cells NR IE |
| 17614 | >>>>>NR CGI Cell | STRUCTURE |  | NR Cell IE in 8.1.1.1 |  |
| 17214 | >>>E-UTRA CGI Support List | LIST |  |  | List of Served Cells E-UTRA IE in TS 38.423 [15] Section 9.1.3.1 |
| 17215 | >>>>E-UTRA CGI Cell Support Item | STRUCTURE |  |  | Individual cell item in List of Served Cells E-UTRA IE |
| 17616 | >>>>>E-UTRA CGI | STRUCTURE |  | E-UTRA Cell IE in 8.1.1.2 |  |
| 17216 | >>>QoS Parameters Support List | LIST |  |  | QoS Flow List IE in TS 37.483 [21] Section 9.3.1.12 |
| 17217 | >>>>QoS Parameters Support Item | STRUCTURE |  |  | QoS Flow Item IE in TS 37.483 [21] Section 9.3.1.12 |
| 17218 | >>>>>CHOICE QoS Parameters Support Item | STRUCTURE |  |  |  |
| 17219 | >>>>>>NG-RAN QoS Support Item | STRUCTURE |  | 8.1.1.6 |  |
| 17221 | >>>>>>E-UTRA QoS Support Item | STRUCTURE |  | 8.1.1.9 |  |
| 17223 | >>Count of list of broadcast PLMNs | ELEMENT | FALSE | INTEGER (0..12) |  |
| 17224 | Count of TAI Support List | ELEMENT | FALSE | INTEGER (0..256) |  |

#### 8.1.1.17	UE Context Information

| RAN Parameter ID | RAN Parameter | RAN Parameter Value Type | Key Flag | RAN Parameter Definition | Semantics Description |
| --- | --- | --- | --- | --- | --- |
| 21501 | Master Node | STRUCTURE |  | 8.1.1.11 |  |
| 21502 | >gNB Measurements | STRUCTURE |  | 8.1.1.15 |  |
| 21503 | CHOICE Primary Cell of MCG | STRUCTURE |  |  |  |
| 21504 | >NR Cell | STRUCTURE |  | 8.1.1.1 | For NR SpCell |
| 21505 | >E-UTRA Cell | STRUCTURE |  | 8.1.1.2 | For E-UTRA PCell |
| 21506 | List of Secondary Cells of MCG | LIST |  |  | Scell To Be Setup List IE in TS 38.473 [19] clause 9.2.2.1 |
| 21507 | >SCell Item | STRUCTURE |  |  | Scell To Be Setup Item IEs IE in TS 38.473 [19] clause 9.2.2.1 |
| 21508 | >>CHOICE SCell | STRUCTURE |  |  |  |
| 21509 | >>>NR Cell | STRUCTURE |  | 8.1.1.1 | For NR SCell |
| 21510 | >>>E-UTRA Cell | STRUCTURE |  | 8.1.1.2 | For E-UTRA SCell |
| 21511 | Secondary Node | STRUCTURE |  | 8.1.1.11 |  |
| 21512 | >gNB Measurements | STRUCTURE |  | 8.1.1.15 |  |
| 21513 | CHOICE Primary Cell of SCG | STRUCTURE |  |  | PSCell IE as defined in TS 38.331 [22] or the structure defined in TS 38.473 [19] clause 9.2.2.1 |
| 21514 | >NR Cell | STRUCTURE |  | 8.1.1.1 |  |
| 21515 | >E-UTRA Cell | STRUCTURE |  | 8.1.1.2 |  |
| 21516 | List of Secondary Cells of SCG | LIST |  |  | Scell To Be Setup List IE in TS 38.473 [19] clause 9.2.2.1 |
| 21517 | >SCell Item | STRUCTURE |  |  | Scell To Be Setup Item IEs IE in TS 38.473 [19] clause 9.2.2.1 |
| 21518 | >>CHOICE SCell | STRUCTURE |  |  |  |
| 21519 | >>>NR Cell | STRUCTURE |  | 8.1.1.1 |  |
| 21520 | >>>E-UTRA Cell | STRUCTURE |  | 8.1.1.2 |  |
| 21521 | List of PDU Sessions | LIST |  |  | PDU Session Resources To Be Setup List IE in TS 38.423 [15] Section 9.2.1.1 |
| 21522 | >PDU Session Item | STRUCTURE |  |  | PDU Session Resources To Be Setup Item IE in TS 38.423 [15] Section 9.2.1.1 |
| 21543 | >>PDU Session ID | ELEMENT | TRUE | PDU Session ID IE in TS 38.413 [11] Section 9.3.1.50 |  |
| 21523 | >>PDU Session | STRUCTURE |  | 8.1.1.16 |  |
| 21524 | >>List of DRBs | LIST |  |  | DRB to Be Setup List IE in TS 38.473 [19] Section 9.2.2.1 |
| 21525 | >>>DRB Item | STRUCTURE |  |  |  |
| 21546 | >>>>DRB ID | ELEMENT | TRUE | DRB ID IE in TS 37.483 [21] clause 9.3.1.16 |  |
| 21547 | >>>>DRB | STRUCTURE |  | 8.1.1.5 |  |
| 21526 | >>>>List of QoS flows mapped to DRB | LIST |  |  | QoS Flows Information To Be Setup IE in TS 37.483 [21] Section 9.3.3.2 |
| 21527 | >>>>>QoS Flow Item | STRUCTURE |  |  |  |
| 21548 | >>>>>>QoS Flow Identifier | ELEMENT | TRUE | QoS Flow Identifier IE in TS 38.413 [11] Section 9.3.1.51 |  |
| 21549 | >>>>>>QoS Flow | STRUCTURE |  | 8.1.1.6 |  |
| 21528 | List of Neighbor cells | LIST |  |  | measResultNeighCells IE in TS 38.331 [22] |  |
| 21529 | >Neighbor Cell Item | STRUCTURE |  |  |  |  |
| 21530 | >>CHOICE Neighbor Cell | STRUCTURE |  |  |  |  |
| 21531 | >>>NR Cell | STRUCTURE |  | 8.1.1.1 | MeasResultNR IE in TS 38.331 [22] |  |
| 21532 | >>>E-UTRA Cell | STRUCTURE |  | 8.1.1.2 | MeasResultEUTRA IE in TS 38.331 [22] |  |

#### 8.1.1.18	eNB Measurements

Editor’s Note: FFS

### 8.1.2	RAN Parameters for Call Process Breakpoint

#### 8.1.2.0	Call process type IDs

The RAN Parameters for the Event Trigger style 2 are defined based on the following call process type.

| Call Process Type ID | Call Process Type | RAN Parameters |
| --- | --- | --- |
| 1 | UE Context Management | Defined in Section 8.1.2.1 |
| 2 | Bearer Context Management | Defined in Section 8.1.2.2 |
| 3 | Mobility Management | Defined in Section 8.1.2.3 |
| 4 | Multi-RAT Dual Connectivity Management | Defined in Section 8.1.2.4 |
| 5 | Radio Resource Control Management | Defined in Section 8.1.2.5 |
| 6 | PDU Session Management | Defined in Section 8.1.2.6 |

#### 8.1.2.1	UE Context Management

The RAN Parameters for the call process type of “UE Context Management” are defined as follows.

##### 8.1.2.1.1	UE Context Setup

| RAN Parameter xID | RAN Parameter | RAN Parameter Value Type | Key Flag | RAN Parameter Definition | Semantics Description |
| --- | --- | --- | --- | --- | --- |
| 25001 | Primary Cell ID | STRUCTURE |  |  | This is for the primary serving cell of the UE. The structuring of this parameter is based on Target Cell Global ID IE in TS 38.423 [15] clause 9.2.3.25 |
| 25002 | >CHOICE Primary Cell | STRUCTURE |  |  | The primary cell could either be an NR primary cell or an LTE primary cell. The structuring is based onTarget Cell IE in TS 38.423 [15] clause 9.2.3.25 |
| 25003 | >>NR SpCell | STRUCTURE |  | 8.1.1.1 | NR Cell IE in TS 38.423 [15] clause 9.2.3.25 or SpCell ID IE in TS 38.473 [19] clause 9.2.2.1 |
| 25063 | >>E-UTRA PCell | STRUCTURE |  | 8.1.1.2 | E-UTRA Cell IE in TS 38.423 [15] clause 9.2.3.25 |
| 25004 | List of secondary cells to be setup | LIST |  |  | Scell To Be Setup List IE in TS 38.473 [19] clause 9.2.2.1 |
| 25005 | >Secondary cell to be setup Item | STRUCTURE |  |  | Scell To Be Setup Item IEs IE in TS 38.473 [19] clause 9.2.2.1 |
| 25006 | >>CHOICE Secondary cell | STRUCTURE |  |  | Scell To Be Setup Item IEs IE in TS 38.473 [19] clause 9.2.2.1 |
| 25007 | >>>NR SCell | STRUCTURE |  | 8.1.1.1 |  |
| 25008 | >>>E-UTRA SCell | STRUCTURE |  | 8.1.1.2 |  |
| 25009 | >>>Scell UL Configured | ELEMENT | FALSE | Cell UL Configured IE in TS 38.473 [19] clause 9.3.1.33 |  |
| 25010 | Number of secondary cells to be setup | ELEMENT | FALSE | INTEGER (1..32) | Count of list of secondary cells to be setup |
| 25011 | List of DRBs for setup | LIST |  |  | DRB to Be Setup List IE in TS 38.473 [19] clause 9.2.2.1 |
| 25012 | >DRB item for setup | STRUCTURE |  |  | DRB to Be Setup Item IE in TS 38.473 [19] clause 9.2.2.1 |
| 25033 | >>DRB ID | ELEMENT | TRUE | DRB ID IE in TS 37.483 [21] clause 9.3.1.16 |  |
| 25034 | >>CHOICE DRB Type | STRUCTURE |  |  | The individual DRB could either be an NG-RAN DRB or a E-UTRA DRB |
| 25035 | >>>NG-RAN DRB | STRUCTURE |  | 8.1.1.5 |  |
| 25036 | >>>E-UTRA DRB | STRUCTURE |  | 8.1.1.9 |  |
| 25013 | >List of QoS flows for setup | LIST |  |  | This is the list of QoS flows multiplexed to an NG-RAN DRB. The structuring is based on QoS Flow List IE in TS 37.483 [21] Section 9.3.1.12 |
| 25014 | >>QoS flow Item | STRUCTURE |  |  | QoS Flow Item IE in TS 37.483 [21] Section 9.3.1.12 |
| 25045 | >>>QoS flow Identifier | ELEMENT | TRUE | QoS Flow Identifier IE in TS 37.483 [21] Section 9.3.1.12 |  |
| 25046 | >>>QoS flow | STRUCTURE |  | 8.1.1.6 |  |
| 25015 | List of PDU sessions for setup | LIST |  |  | PDU Session Resource Setup Request List IE in TS 38.413 Section 9.2.1.1 |
| 25016 | >PDU Session Item | STRUCTURE |  |  | PDU Session Resource Setup Item IE in TS 38.413 Section 9.2.1.1 |
| 25057 | >>PDU Session ID | ELEMENT | TRUE | PDU Session ID IE in TS 38.413 [11] Section 9.3.1.50 |  |
| 25058 | >>PDU Session | STRUCTURE |  | 8.1.1.16 |  |
| 25017 | >>List of QoS flows for setup | LIST |  |  | QoS Flow Setup Request Item IE in TS 38.413 Section 9.3.4.1 |
| 25018 | >>>QoS flow Item | STRUCTURE |  |  | QoS Flow Setup Request Item IE in TS 38.413 Section 9.3.4.1 |
| 25019 | >>>>QoS flow Identifier | ELEMENT | TRUE | QoS Flow Identifier IE in TS 38.413 [11] Section 9.3.1.51 |  |
| 25020 | >>>>QoS flow | STRUCTURE |  | 8.1.1.6 |  |
| 25021 | Number of DRBs for setup | ELEMENT | FALSE | INTEGER (0..64) | Count of list of DRBs for setup |
| 25022 | UE Aggregate Maximum Bit Rate | STRUCTURE |  |  | UE Aggregate Maximum Bit Rate IE in TS 36.423 [17] Section 9.2.12 |
| 25023 | >UE Aggregate Maximum Bit Rate Downlink | ELEMENT | FALSE | UE Aggregate Maximum Bit Rate Downlink IE in TS 36.423 [17] clause 9.2.12 |  |
| 25024 | >UE Aggregate Maximum Bit Rate Uplink | ELEMENT | FALSE | UE Aggregate Maximum Bit Rate Uplink IE in TS 36.423 [17] clause 9.2.12 |  |

##### 8.1.2.1.2	UE Context Modification

| RAN Parameter ID | RAN Parameter |  | RAN Parameter Value Type | Key Flag | RAN Parameter Definition | Semantics Description |
| --- | --- | --- | --- | --- | --- | --- |
| 25101 | Primary Cell ID |  | STRUCTURE |  |  | This is for the primary serving cell of the UE. The structuring of this parameter is based on Target Cell Global ID IE in TS 38.423 [15] clause 9.2.3.25 |
| 25102 | >CHOICE Primary Cell |  | STRUCTURE |  |  | The primary cell could either be an NR primary cell or an LTE primary cell. The structuring is based on Target Cell IE in TS 38.423 [15] clause 9.2.3.25 |
| 25103 | >>NR SpCell |  | STRUCTURE |  | 8.1.1.1 | NR IE in TS 38.423 [15] clause 9.2.3.25 or SpCell ID IE in TS 38.473 [19] clause 9.2.2.1 |
| 25104 | >>E-UTRA PCell |  | STRUCTURE |  | 8.1.1.2 | E-UTRA IE in TS 38.423 [15] clause 9.2.3.25 |
| 25105 | List of secondary cells to be setup |  | LIST |  |  | Scell To Be Setup List IE in TS 38.473 [19] clause 9.2.2.1 |
| 25106 | >Secondary cell to be setup Item |  | STRUCTURE |  |  | Scell To Be Setup Item IEs IE in TS 38.473 [19] clause 9.2.2.1 |
| 25107 | >>CHOICE Secondary cell |  | STRUCTURE |  |  | Scell To Be Setup Item IEs IE in TS 38.473 [19] clause 9.2.2.1 |
| 25108 | >>>NR SCell |  | STRUCTURE |  | 8.1.1.1 |  |
| 25109 | >>>E-UTRA SCell |  | STRUCTURE |  | 8.1.1.2 |  |
| 25110 | >>>Scell UL Configured |  | ELEMENT | FALSE | Cell UL Configured IE in TS 38.473 [19] [Q] clause 9.3.1.33 |  |
| 25111 | Number of secondary cells to be setup |  | ELEMENT | FALSE | INTEGER (1..32) | Count of list of secondary cells to be setup |
| 25112 | List of Scells to be removed |  | LIST |  |  | SCell To Be Removed List IE in TS 38.473 [19] clause 9.2.2.7 |
| 25113 | >Scell to be removed Item |  | STRUCTURE |  |  | SCell to Be Removed Item IEs IE in TS 38.473 [19] clause 9.2.2.7 |
| 25114 | >>CHOICE Secondary cell |  | STRUCTURE |  |  | A UE can have either NR Cells as secondary cells or E-UTRA cells as secondary cells. |
| 25115 | >>>NR SCell |  | STRUCTURE |  | 8.1.1.1 |  |
| 25116 | >>>E-UTRA SCell |  | STRUCTURE |  | 8.1.1.2 |  |
| 25117 | >>>Scell UL Configured |  | ELEMENT | FALSE | Cell UL Configured IE in TS 38.473 [19] [Q] clause 9.3.1.33 |  |
| 25118 | Number of secondary cells to be removed |  | ELEMENT | FALSE | INTEGER (1..32) | Count of list of secondary cells to be removed |
| 25119 | List of DRBs for setup |  | LIST |  |  | DRB to Be Setup List IE in TS 38.473 [19] clause 9.2.2.1 |
| 25120 | >DRB item for setup |  | STRUCTURE |  |  | DRB to Be Setup Item IE in TS 38.473 [19] clause 9.2.2.1 |
| 25161 | >>DRB ID |  | ELEMENT | TRUE | DRB ID IE in TS 37.483 [21] clause 9.3.1.16 |  |
| 25162 | >>CHOICE DRB Type |  | STRUCTURE |  |  | DRB could either be an NG-RAN DRB or a E-UTRA DRB |
| 25163 | >>>NG-RAN DRB |  | STRUCTURE |  | 8.1.1.5 |  |
| 25164 | >>>E-UTRA DRB |  | STRUCTURE |  | 8.1.1.9 |  |
| 25121 | >>List of QoS flows for setup |  | LIST |  |  | This is the list of QoS flows multiplexed to an NG-RAN DRB for setup. The structuring is based on QoS Flow List IE in TS 37.483 [21] Section 9.3.1.12 |
| 25122 | >>>QoS flow Item |  | STRUCTURE |  |  | QoS Flow Item IE in TS 37.483 [21] Section 9.3.1.12 |
| 25166 | >>>>QoS flow Identifier |  | ELEMENT | TRUE | QoS Flow Identifier IE in TS 37.483 [21] Section 9.3.1.12 |  |
| 25167 | >>>>QoS flow |  | STRUCTURE |  | 8.1.1.6 |  |
| 25123 | List of PDU sessions for setup |  | LIST |  |  | PDU Session Resource Setup Request List IE in TS 38.413 Section 9.2.1.1 |
| 25124 | >PDU Session Item |  | STRUCTURE |  |  | PDU Session Resource Setup Item IE in TS 38.413 Section 9.2.1.1 |
| 25175 | >>PDU Session ID |  | ELEMENT | TRUE | PDU Session ID IE in TS 38.413 [11] Section 9.3.1.50 |  |
| 25176 | >>PDU Session |  | STRUCTURE |  | 8.1.1.16 |  |
| 25125 | >>List of QoS flows for setup |  | LIST |  |  | QoS Flow Setup Request Item IE in TS 38.413 Section 9.3.4.1 |
| 25126 | >>>QoS flow Item |  | STRUCTURE |  |  | QoS Flow Setup Request Item IE in TS 38.413 Section 9.3.4.1 |
| 25177 | >>>>QoS flow Identifier |  | ELEMENT | TRUE | QoS Flow Identifier IE in TS 38.413 [11] Section 9.3.1.51 |  |
| 25178 | >>>>QoS flow |  | STRUCTURE |  | 8.1.1.6 |  |
| 25127 | List of DRBs to be modified |  | LIST |  |  | DRB to Be Setup List IE in TS 38.473 [19] clause 9.2.2.1 |
| 25128 | >DRB item for modification |  | STRUCTURE |  | 8.1.1.5 | DRB to Be Setup Item IE in TS 38.473 [19] clause 9.2.2.1 |
| 25129 | >>List of QoS flows remapped |  | LIST |  |  | This is the list of QoS flows remapped to an NG-RAN DRB. The structuring is based on QoS Flow List IE in TS 37.483 [21] Section 9.3.1.12 |
| 25130 | >>>QoS flow Item |  | STRUCTURE |  |  | QoS Flow Item IE in TS 37.483 [21] Section 9.3.1.12 |
| 25181 | >>>>QoS flow Identifier |  | ELEMENT | TRUE | QoS Flow Identifier IE in TS 37.483 [12] Section 9.3.1.12 |  |
| 25182 | >>>>QoS flow |  | STRUCTURE |  | 8.1.1.6 |  |
| 25131 | List of PDU sessions for modification |  | LIST |  |  | For modification of PDU sessions. The structuring is based on PDU Session Resource Modify Request List IE in TS 38.413 Section 9.2.1.5 |
| 25132 | >PDU Session Item |  | STRUCTURE |  |  | PDU Session Resource Modify Request Item IE in TS 38.413 Section 9.2.1.5 |
| 25183 | >>PDU Session ID |  | ELEMENT | TRUE | PDU Session ID IE in TS 38.413 [11] Section 9.3.1.50 |  |
| 25184 | >>PDU Session |  | STRUCTURE |  | 8.1.1.16 |  |
| 25133 | >>List of QoS flows remapped |  | LIST |  |  | Modified list of QoS flows within the PDU Session. Structuring is based on QoS Flow Add or Modify Request List IE in TS 38,413 Section 9.3.4.3 |
| 25134 | >>>QoS flow Item |  | STRUCTURE |  |  | QoS Flow Add or Modify Request Item IE in TS 38.413 Section 9.3.4.3 |
| 25185 | >>>>QoS flow identifier |  | ELEMENT | TRUE | QoS Flow Identifier IE in TS 38.413 [11] Section 9.3.1.51 |  |
| 25186 | >>>>QoS flow |  | STRUCTURE |  | 8.1.1.6 |  |
| 25135 | Number of DRBs to be modified |  | ELEMENT | FALSE |  | Count of list of DRBs to be modified |
| 25136 | List of DRBs to be released |  | LIST |  |  | DRB To Be Released List IE in TS 38.473 [19] Section 9.2.2.7 |
| 25137 | >DRB item for release |  | STRUCTURE |  |  | DRB To Be Released Item IE in TS 38.473 [19] Section 9.2.2.7 |
| 25188 | >>DRB ID |  | ELEMENT | TRUE | DRB ID IE in TS 37.483 [21] clause 9.3.1.16 |  |
| 25189 | >>CHOICE DRB Type |  | STRUCTURE |  |  | DRB could either be an NG-RAN DRB or a E-UTRA DRB |
| 25190 | >>>NG-RAN DRB |  | STRUCTURE |  | 8.1.1.5 |  |
| 25191 | >>>E-UTRA DRB |  | STRUCTURE |  | 8.1.1.9 |  |
| 25138 | Number of DRBs to be released |  | ELEMENT | FALSE |  | Count of list of DRBs to be released |
| 25139 | UE Aggregate Maximum Bit Rate |  | STRUCTURE |  |  | UE Aggregate Maximum Bit Rate IE in TS 36.423 [17] Section 9.2.12 |
| 25140 | >UE Aggregate Maximum Bit Rate Downlink |  | ELEMENT | FALSE | UE Aggregate Maximum Bit Rate Downlink IE in TS 36.423 [17] clause 9.2.12 |  |
| 25141 | >UE Aggregate Maximum Bit Rate Uplink |  | ELEMENT | FALSE | UE Aggregate Maximum Bit Rate Uplink IE in TS 36.423 [17] clause 9.2.12 |  |
| 25151 |  | Target Cell ID | STRUCTURE |  |  | This is for the target cell of the UE for HO. The structuring of this parameter is based on Target Cell Global ID IE in TS 38.423 [15] clause 9.2.3.25 |
| 25152 |  | >CHOICE Target Cell | STRUCTURE |  |  | The target cell could either be a target NR primary cell or a target LTE primary cell. The structuring is based on Target Cell IE in TS 38.423 [15] clause 9.2.3.25 |
| 25153 |  | >>Target NR SpCell | STRUCTURE |  | 8.1.1.1 | NR IE in TS 38.423 [15] clause 9.2.3.25 |
| 25154 |  | >>Target E-UTRA PCell | STRUCTURE |  | 8.1.1.2 | E-UTRA IE in TS 38.423 [15] clause 9.2.3.25 |

##### 8.1.2.1.3	UE Context Release

| RAN Parameter ID | RAN Parameter | RAN Parameter Value Type | Key Flag | RAN Parameter Definition | Semantics Description |
| --- | --- | --- | --- | --- | --- |
| 25201 | List of candidate cells to be cancelled | LIST |  |  | Candidate Cells To Be Cancelled List IE in TS 38.473 [19] Section 9.2.2.4 |
| 25202 | >Candidate cell to be cancelled Item | STRUCTURE |  |  | Candidate Cell to be Cancelled Item IE in TS 38.473 [19] Section 9.2.2.4 |
| 25203 | >>CHOICE Candidate cell | STRUCTURE |  |  | Candidate Cell to be Cancelled Item IE in TS 38.473 [19] Section 9.2.2.4 |
| 25204 | >>>NR SCell | STRUCTURE |  | 8.1.1.1 |  |
| 25205 | >>>E-UTRA SCell | STRUCTURE |  | 8.1.1.2 |  |

#### 8.1.2.2	Bearer Context Management

The RAN Parameters for the call process type of “Bearer Context Management” are defined as follows.

##### 8.1.2.2.1	Bearer Context Setup

| RAN Parameter ID | RAN Parameter | RAN Parameter Value Type | Key Flag | RAN Parameter Definition | Semantics Description |
| --- | --- | --- | --- | --- | --- |
| 26101 | DRB ID | ELEMENT | TRUE | DRB ID IE in TS 37.483 [21] clause 9.3.1.16 |  |
| 26102 | CHOICE DRB Type | STRUCTURE |  |  | DRB could either be an NG-RAN DRB or a E-UTRA DRB |
| 26103 | >NG-RAN DRB | STRUCTURE |  | 8.1.1.5 |  |
| 26104 | >E-UTRA DRB | STRUCTURE |  | 8.1.1.9 |  |
| 26105 | List of QoS Flows to be setup in DRB | LIST |  |  | This is the list of QoS flows multiplexed to an NG-RAN DRB for setup. The structuring is based on QoS Flow List IE in TS 37.483 [21] Section 9.3.1.12 |
| 26106 | >QoS Flow Item | STRUCTURE |  |  | QoS Flow Item IE in TS 37.483 [21] clause 9.3.1.12 |
| 26107 | >>QoS Flow Identifier | ELEMENT | TRUE | QoS Flow Identifier IE in TS 37.483 [21] clause 9.3.1.24 |  |
| 26108 | >>QoS Flow | STRUCTURE |  | 8.1.1.6 |  |
| 26109 | Count of list of QoS Flows to be setup in DRB | ELEMENT | FALSE | INTEGER (0..64) |  |
| 26110 | Cell Group List to Add | STRUCTURE |  |  | CellGroupList IE in TS 37.483 [21] Section 9.3.1.11 |
| 26111 | >Cell Group Item | STRUCTURE |  |  |  |
| 26122 | >>Cell Group ID | ELEMENT | TRUE | cellGroup IE in TS 38.331 [22] Section 6 |  |
| 26123 | >>Cell Group | STRUCTURE |  | 8.1.1.7 |  |
| 26112 | Logical Channel ID | ELEMENT | FALSE | LogicalChannelIdentity IE in TS 38.331 [22] clause 6 |  |
| 26113 | Channel Access Priority | ELEMENT | FALSE | channelAccessPriority IE in TS 38.331 [22] |  |
| 26114 | L2 Bearer State Information | STRUCTURE |  | 8.1.1.8 |  |
| 26115 | Count of cell group list to be added | ELEMENT | FALSE | INTEGER (0..4) |  |

##### 8.1.2.2.2	Bearer Context Modification

| RAN Parameter ID | RAN Parameter | RAN Parameter Value Type | Key Flag |  | RAN Parameter Definition |  | Semantics Description |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 26201 | DRB ID | ELEMENT | TRUE |  | DRB ID IE in TS 37.483 [21] clause 9.3.1.16 |  |  |
| 26202 | CHOICE DRB Type | STRUCTURE |  |  |  |  | DRB could either be an NG-RAN DRB or a E-UTRA DRB |
| 26203 | >NG-RAN DRB | STRUCTURE |  |  | 8.1.1.1 |  |  |
| 26204 | >E-UTRA DRB | STRUCTURE |  |  | 8.1.1.2 |  |  |
| 26205 | Logical Channel ID | ELEMENT | TRUE |  | LogicalChannelIdentity IE in TS 38.331 [22] clause 6 |  |  |
| 26206 | CHOICE RLC-Config | STRUCTURE |  |  |  |  | RLC-Config IE in TS 38.331 [22] |
| 26207 | >AM | STRUCTURE |  |  |  |  | am IE in TS 38.331 [22] |
| 26208 | >>UL AM RLC | STRUCTURE |  |  |  |  | ul-AM-RLC IE in TS 38.331 [22] |
| 26209 | >>>Poll Retransmit | ELEMENT |  | FALSE |  | t-PollRetransmit IE in TS 38.331 [22] |  |
| 26210 | >>>Max Retransmission Threshold | ELEMENT |  | FALSE |  | maxRetxThreshold IE in TS 38.331 [22] |  |
| 26211 | >>>Poll PDU | ELEMENT |  | FALSE |  | pollPDU IE in TS 38.331 [22] |  |
| 26212 | >>>Poll Byte | ELEMENT |  | FALSE |  | pollByte IE in TS 38.331 [22] |  |
| 26213 | >>DL AM RLC | STRUCTURE |  |  |  |  | dl-AM-RLC IE in TS 38.331 [22] |
| 26214 | >>>Reassembly | ELEMENT |  | FALSE |  | t-Reassembly IE in TS 38.331 [22] |  |
| 26215 | >>>Status Prohibit | ELEMENT |  | FALSE |  | t-StatusProhibit IE in TS 38.331 [22] |  |
| 26216 | >UM Bi-directional | STRUCTURE |  |  |  |  | um-Bi-Directional IE in TS 38.331 [22] |
| 26217 | >>UL UM RLC | ELEMENT |  | FALSE |  |  | UL-UM-RLC IE in TS 38.331 [22] |
| 26218 | >>DL UM RLC | STRUCTURE |  |  |  |  | DL-UM-RLC IE in TS 38.331 [22] |
| 26219 | >>>Reassembly | ELEMENT |  | FALSE |  | t-Reassembly IE in TS 38.331 [22] |  |
| 26220 | >UM Uni-directional UL | STRUCTURE |  |  |  |  | um-Uni-Directional-UL IE in TS 38.331 [22] |
| 26221 | >>UL UM RLC | ELEMENT |  | FALSE |  | ul-UM-RLC IE in TS 38.331 [22] |  |
| 26222 | >UM Uni-directional DL | ELEMENT |  | FALSE |  |  | um-Uni-Directional-DL IE in TS 38.331 [22] |
| 26223 | >DL UM RLC | STRUCTURE |  |  |  |  | dl-UM-RLC IE in TS 38.331 [22] |
| 26224 | >>Reassembly | ELEMENT |  | FALSE |  | t-Reassembly IE in TS 38.331 [22] |  |
| 26225 | List of QoS Flows to be setup in DRB | LIST |  |  |  |  | Flow Mapping Information IE in TS 37.483 [21] clause 9.3.1.26 |
| 26226 | >QoS Flow Item | STRUCTURE |  |  |  |  | QoS Flow Item IE in TS 37.483 [21] clause 9.3.1.12 |
| 26227 | >>QoS Flow Identifier | ELEMENT | TRUE |  | QoS Flow Identifier IE in TS 37.483 [21] clause 9.3.1.24 |  |  |
| 26228 | >>QoS Flow | STRUCTURE |  |  | 8.1.1.6 |  |  |
| 26229 | List of QoS Flows to be modified in DRB | LIST |  |  |  |  | Flow Mapping Information IE in TS 37.483 [21] clause 9.3.1.26 |
| 26230 | >QoS Flow Item | STRUCTURE |  |  |  |  | QoS Flow Item IE in TS 37.483 [21] clause 9.3.1.12 |
| 26231 | >>QoS Flow Identifier | ELEMENT | TRUE |  | QoS Flow Identifier IE in TS 37.483 [21] clause 9.3.1.24 |  |  |
| 26232 | >>QoS Flow | STRUCTURE |  |  | 8.1.1.6 |  |  |
| 26233 | List of Cell Groups to Add | LIST |  |  |  |  | Cell Group List IE in TS 37.483 [21] Section 9.3.1.11 |
| 26234 | >Cell Group Item | STRUCTURE |  |  |  |  |  |
| 26255 | >>Cell Group ID | ELEMENT | TRUE |  | cellGroup IE in TS 38.331 [22] Section 6 |  |  |
| 26256 | >>Cell Group | STRUCTURE |  |  | 8.1.1.7 |  |  |
| 26235 | List of Cell Groups To Modify/Update | LIST |  |  |  |  | Cell Group List IE in TS 37.483 [21] Section 9.3.1.11 |
| 26236 | >Cell Group Item | STRUCTURE |  |  |  |  |  |
| 26257 | >>Cell Group ID | ELEMENT | TRUE |  | cellGroup IE in TS 38.331 [22] Section 6 |  |  |
| 26258 | >>Cell Group | STRUCTURE |  |  | 8.1.1.7 |  |  |
| 26237 | List of Cell Groups To Delete | LIST |  |  |  |  | Cell Group List IE in TS 37.483 [21] Section 9.3.1.11 |
| 26238 | >Cell Group Item | STRUCTURE |  |  |  |  |  |
| 26259 | >>Cell Group ID | ELEMENT | TRUE |  | cellGroup IE in TS 38.331 [22] Section 6 |  |  |
| 26260 | >>Cell Group | STRUCTURE |  |  | 8.1.1.7 |  |  |
| 26239 | Logical Channel ID | ELEMENT | FALSE |  | LogicalChannelIdentity IE in TS 38.331 [22] clause 6 |  |  |
| 26240 | Channel Access Priority | ELEMENT | FALSE |  | channelAccessPriority IE in TS 38.331 [22] |  |  |
| 26241 | L2 State Information | STRUCTURE |  |  | 8.1.1.8 |  |  |
| 26242 | Data Report Usage List | LIST |  |  |  |  | Data Usage Report List IE in TS 37.483 [21] Section 9.3.1.44 |
| 26243 | >Data Report Usage Item | STRUCTURE |  |  |  |  | Data Report Usage Item IE in TS 37.483 [21] Section 9.3.1.44 |
| 26244 | >>Start Timestamp | ELEMENT | FALSE |  | Start timestamp IE in TS 37.483 [21] Section 9.3.1.44 |  |  |
| 26245 | >>End Timestamp | ELEMENT | FALSE |  | End timestamp IE in TS 37.483 [21] Section 9.3.1.44 |  |  |
| 26246 | >>Usage Count DL | ELEMENT | FALSE |  | Usage count DL IE in TS 37.483 [21] Section 9.3.1.44 |  |  |
| 26247 | >>Usage Count UL | ELEMENT | FALSE |  | Usage count DL IE in TS 37.483 [21] Section 9.3.1.44 |  |  |

##### 8.1.2.2.3	Bearer Context Release

| RAN Parameter ID | RAN Parameter | RAN Parameter Value Type | Key Flag | RAN Parameter Definition | Semantics Description |
| --- | --- | --- | --- | --- | --- |
| 26301 | DRB ID | ELEMENT | TRUE | DRB ID IE in TS 37.483 [21] clause 9.3.1.16 |  |
| 26302 | PDCP DL Count | ELEMENT | FALSE | PDCP DL Count IE in TS 37.483 [21] Section 9.2.2.11 |  |
| 26303 | PDCP UL Count | ELEMENT | FALSE | PDCP UL Count IE in TS 37.483 [21] Section 9.2.2.11 |  |
| 26304 | Release Cause | STRUCTURE |  |  | Cause IE in TS 37.483 [21] Section 9.3.1.2 |
| 26305 | >CHOICE Cause Group | STRUCTURE |  |  | Cause Group IE in TS 37.483 [21] Section 9.3.1.2 |
| 26306 | >>Radio Network Layer | STRUCTURE |  |  | Radio Network Layer IE in TS 37.483 [21] Section 9.3.1.2 |
| 26307 | >>>Radio Network Layer Cause | ELEMENT | FALSE | Radio Network Layer Cause IE in TS 37.483 [21] Section 9.3.1.2 |  |
| 26308 | >>Transport Layer | STRUCTURE |  |  | Transport Layer IE in TS 37.483 [21] Section 9.3.1.2 |
| 26309 | >>>Transport Layer Cause | ELEMENT | FALSE | Transport Layer Cause IE in TS 37.483 [21] Section 9.3.1.2 |  |
| 26310 | >>Protocol | STRUCTURE |  |  | Protocol IE in TS 37.483 [21] Section 9.3.1.2 |
| 26311 | >>>Protocol Cause | ELEMENT | FALSE | Protocol Cause IE in TS 37.483 [21] Section 9.3.1.2 |  |
| 26312 | >>Miscellaneous | STRUCTURE |  |  | Misc IE in TS 37.483 [21] Section 9.3.1.2 |
| 26313 | >>>Miscellaneous Cause | ELEMENT | FALSE | Miscellaneous Cause IE in TS 37.483 [21] Section 9.3.1.2 |  |

#### 8.1.2.3	Mobility Management

The RAN Parameters for the call process type of “Mobility Management” are defined as follows.

##### 8.1.2.3.1	Handover Preparation

| RAN Parameter ID | RAN Parameter | RAN Parameter Value Type | Key Flag | RAN Parameter Definition | Semantics Description |
| --- | --- | --- | --- | --- | --- |
| 27101 | Event AN and inter-RAT B1 mobility thresholds | STRUCTURE |  | Connectivity and Mobility Event Thresholds IE in clause 8.1.1.10 |  |
| 27102 | List of Serving cells | LIST |  |  | MeasResultServMOList IE in TS 38.331 [22] |
| 27103 | >Serving Cell Item | STRUCTURE |  |  | MeasResultServMO IE in TS 38.331 [22] |
| 27104 | >>Serving Cell Index | ELEMENT | TRUE | servCellID IE in TS 38.331 [22] |  |
| 27105 | >>CHOICE Serving Cell | STRUCTURE |  |  | MeasResultServMO IE in TS 38.331 [22] |
| 27106 | >>>NR Cell | STRUCTURE |  | 8.1.1.1 |  |
| 27107 | >>>E-UTRA Cell | STRUCTURE |  | 8.1.1.2 |  |
| 27108 | >>CHOICE Best Neighboring Cell | STRUCTURE |  |  | measResultBestNeighCell IE in TS 38.331 [22] |
| 27109 | >>>NR Cell | STRUCTURE |  | 8.1.1.1 |  |
| 27110 | >>>E-UTRA Cell | STRUCTURE |  | 8.1.1.2 |  |
| 27113 | Primary Cell ID | STRUCTURE |  |  | This is for the primary serving cell of the UE. The structuring of this parameter is based on Target Cell Global ID IE in TS 38.423 [15] clause 9.2.3.25 |
| 27114 | >CHOICE Primary Serving Cell | STRUCTURE |  |  | The primary cell could either be an NR primary cell or an LTE primary cell. The structuring is based onTarget Cell IE in TS 38.423 [15] clause 9.2.3.25 |
| 27115 | >>NR SpCell | STRUCTURE |  | 8.1.1.1 | NR IE in TS 38.423 [15] clause 9.2.3.25 or SpCell ID IE in TS 38.473 [19] clause 9.2.2.1 |
| 27116 | >>LTE E-UTRA PCell | STRUCTURE |  | 8.1.1.2 | E-UTRA IE in TS 38.423 [15] clause 9.2.3.25 |
| 27117 | List of Secondary cells | LIST |  |  | SCell To Be Setup List IE in TS 38.473 [19] Section 9.2.2.1 |
| 27118 | >Secondary Cell Item | STRUCTURE |  |  | SCell To Be Setup Item IEs IE in TS 38.473 [19] Section 9.2.2.1 |
| 27119 | >>Secondary Cell Index | ELEMENT | FALSE |  | SCellIndex IE in TS 38.473 [19] Section 9.2.2.1 |
| 27120 | >>CHOICE Secondary Cell | STRUCTURE |  |  | SCell To Be Setup Item IEs IE in TS 38.473 [19] Section 9.2.2.1 |
| 27121 | >>>NR Cell | STRUCTURE |  | 8.1.1.1 |  |
| 27122 | >>>E-UTRA Cell | STRUCTURE |  | 8.1.1.2 |  |
| 27123 | List of Neighbor cells | LIST |  |  | measResultNeighCells IE in TS 38.331 [22] |
| 27124 | >Neighbor Cell Item | STRUCTURE |  |  | measResultNeighCells IE in TS 38.331 [22] |
| 27125 | >>CHOICE Neighbor Cell | STRUCTURE |  |  | measResultNeighCells IE in TS 38.331 [22] |
| 27126 | >>>NR Cell | STRUCTURE |  | 8.1.1.1 | MeasResultNR IE in TS 38.331 [22] |
| 27127 | >>>E-UTRA Cell | STRUCTURE |  | 8.1.1.2 | MeasResultEUTRA IE in TS 38.331 [22] |
| 27128 | Target Primary Cell ID | STRUCTURE |  |  | Target Cell Global ID IE in TS 38.423 [15] Section 9.2.3.25 |
| 27129 | >CHOICE Target Cell | STRUCTURE |  |  | Target Cell IE in TS 38.423 [15] Section 9.2.3.25 |
| 27130 | >>NR SpCell | STRUCTURE |  | 8.1.1.1 | NR IE in TS 38.423 [15] Section 9.2.3.25 or SpCell ID IE in TS 38.473 [19] clause 9.2.2.1 |
| 27131 | >>LTE E-UTRA PCell | STRUCTURE |  | 8.1.1.2 | E-UTRA IE in TS 38.423 [15] Section 9.2.3.25 |
| 27132 | List of Candidate Target cells for conditional HO | LIST |  |  | Candidate SpCell List IE in TS 38.473 [19] Section 9.2.2.1 |
| 27133 | >Candidate Target cell Item | STRUCTURE |  |  | Candidate SpCell Item IEs IE in TS 38.473 [19] Section 9.2.2.1 |
| 27134 | >>CHOICE Candidate Target Cell | STRUCTURE |  |  | Target Cell IE in TS 38.423 [15] Section 9.2.3.25 |
| 27135 | >>>NR SpCell candidate | STRUCTURE |  | 8.1.1.1 | NR IE in TS 38.423 [15] clause 9.2.3.25 or SpCell ID IE in TS 38.473 [19] clause 9.2.2.1 |
| 27136 | >>>LTE E-UTRA PCell candidate | STRUCTURE |  | 8.1.1.2 | E-UTRA IE in TS 38.423 [15] clause 9.2.3.25 |
| 27137 | >>Estimated Arrival Probability | ELEMENT | FALSE | Estimated Arrival Probability IE in TS 38.423 [15] Section 9.1.1.1 |  |
| 27138 | List of PDU Sessions for Handover | LIST |  |  | This is the list of PDU sessions of the UE that are subject to handover. The structuring is based on PDU Session Resources To Be Setup List IE in TS 38.423 [15] Section 9.2.1.1 |
| 27139 | >PDU Session Item | STRUCTURE |  |  | PDU Session Resources To Be Setup Item IE in TS 38.423 [15] Section 9.2.1.1 |
| 27140 | >>PDU Session ID | ELEMENT | TRUE | PDU Session ID IE in TS 38.423 [15] Section 9.2.3.18 |  |
| 27141 | >>PDU Session | STRUCTURE |  | 8.1.1.16 |  |
| 27142 | >>List of QoS flows in the PDU Session | LIST |  |  | QoS Flows To Be Setup List IE in TS 38.423 [15] Section 9.2.1.1 |
| 27143 | >>>QoS Flow Item | LIST |  |  | QoS Flow To Be Setup Item IE in TS 38.423 [15] Section 9.2.1.1 |
| 27144 | >>>>QoS Flow Identifier | ELEMENT | TRUE | QoS Flow Identifier IE in TS 38.423 [15] Section 9.2.3.10 |  |
| 27145 | >>>>QoS Flow | STRUCTURE |  | 8.1.1.6 |  |
| 27146 | >>List of DRBs for Handover | LIST |  |  | This is the list of DRBs corresponding to the PDU session to be handed over. The structuring is based on DRB to Be Setup List IE in TS 38.473 [19] Section 9.2.2.7 |
| 27147 | >>>DRB Item for Handover | STRUCTURE |  |  | DRB to Be Setup Item IE in TS 38.473 [19] Section 9.2.2.7 |
| 27148 | >>>>DRB ID | ELEMENT | TRUE | DRB ID IE in TS 38.473 [19] Section 9.3.1.8 |  |
| 27149 | >>>>CHOICE DRB Type | STRUCTURE |  |  |  |
| 27150 | >>>>>NG-RAN DRB | STRUCTURE |  | 8.1.1.5 |  |
| 27151 | >>>>>E-UTRA DRB | STRUCTURE |  | 8.1.1.9 |  |
| 27152 | >>>>List of QoS flows in the DRB | LIST |  |  | QoS Flows Information To Be Setup IE in TS 37.483 [21] Section 9.3.3.2 |
| 27153 | >>>>>QoS Flow Item | STRUCTURE |  |  | QoS Flow Item IE in TS 37.483 [21] Section 9.3.1.25 |
| 27154 | >>>>>>QoS Flow Identifier | ELEMENT | TRUE | QoS Flow Identifier IE in TS 37.483 [21] Section 9.3.1.25 |  |
| 27155 | >>>>>>QoS Flow | STRUCTURE |  | 8.1.1.6 |  |
| 27175 | List of DRBs for Handover | LIST |  |  | DRB to Be Setup List IE in TS 38.473 [19] Section 9.2.2.1 |
| 27176 | >DRB Item for Handover | STRUCTURE |  |  | DRB to Be Setup Item IE in TS 38.473 [19] Section 9.2.2.1 |
| 27177 | >>DRB ID | ELEMENT | TRUE | DRB ID IE in TS 38.473 [19] Section 9.3.1.8 |  |
| 27178 | >>CHOICE DRB Type | STRUCTURE |  |  |  |
| 27179 | >>>NG-RAN DRB | STRUCTURE |  | 8.1.1.5 |  |
| 27180 | >>>E-UTRA DRB | STRUCTURE |  | 8.1.1.9 |  |
| 27181 | >>List of QoS flows in the DRB | LIST |  |  | QoS Flows Information To Be Setup IE in TS 37.483 [21] Section 9.3.3.2 |
| 27182 | >>>QoS Flow Item | STRUCTURE |  |  | QoS Flow Item IE in TS 37.483 [21] Section 9.3.1.25 |
| 27183 | >>>>QoS Flow Identifier | ELEMENT | TRUE | QoS Flow Identifier IE in TS 37.483 [21] Section 9.3.1.25 |  |
| 27184 | >>>>QoS Flow | STRUCTURE |  | 8.1.1.6 |  |

##### 8.1.2.3.2	Handover Cancel

| RAN Parameter ID | RAN Parameter | RAN Parameter Value Type | Key Flag | RAN Parameter Definition | Semantics Description |
| --- | --- | --- | --- | --- | --- |
| 27201 | Event AN and inter-RAT B1 mobility thresholds | STRUCTURE |  | Connectivity and Mobility Event Thresholds IE in clause 8.1.1.10 |  |  |
| 27209 | List of candidate cells to be cancelled list | LIST |  |  | Candidate Cells To Be Cancelled List IE in TS 38.473 [19] Section 9.2.2.4 |  |
| 27210 | >Candidate cell to be cancelled item | STRUCTURE |  |  | Candidate Cells To Be Cancelled List IE in TS 38.473 [19] Section 9.2.2.4 |  |
| 27211 | >>Target cell ID | STRUCTURE |  |  | Target Cell ID IE in TS 38.473 [19] Section 9.2.2.4 |  |
| 27212 | >>>CHOICE Target cell ID | STRUCTURE |  |  | Target Cell ID IE in TS 38.473 [19] Section 9.2.2.4 |  |
| 27213 | >>>>NR Cell | STRUCTURE |  | 8.1.1.1 |  |  |
| 27214 | >>>>LTE E-UTRA Cell | STRUCTURE |  | 8.1.1.2 |  |  |
| 27215 | Cause for canceling HO or CHO | STRUCTURE |  |  | Cause IE in TS 38.423 [15] Section 9.1.1.6 |  |
| 27216 | >CHOICE Cause Group | STRUCTURE |  |  | Cause Group IE in TS 38.423 [15] Section 9.2.3.2 |  |
| 27217 | >>Radio Network Layer | STRUCTURE |  |  | Radio Network Layer IE in TS 38.423 [15] Section 9.2.3.2 |  |
| 27218 | >>>Radio Network Layer Cause | ELEMENT | FALSE | Radio Network Layer Cause IE in TS 38.423 [15] Section 9.2.3.2 |  |  |
| 27219 | >>Transport Layer | STRUCTURE |  |  | Transport Layer IE in TS 38.423 [15] Section 9.2.3.2 |  |
| 27220 | >>>Transport Layer Cause | ELEMENT | FALSE | Transport Layer Cause IE in TS 38.423 [15] Section 9.2.3.2 |  |  |
| 27221 | >>Protocol | STRUCTURE |  |  | Protocol IE in TS 38.423 [15] Section 9.2.3.2 |  |
| 27222 | >>>Protocol Cause | ELEMENT | FALSE | Protocol Cause IE in TS 38.423 [15] Section 9.2.3.2 |  |  |
| 27223 | >>Misc | STRUCTURE |  |  | Misc IE in TS 38.423 [15] Section 9.2.3.2 |  |
| 27224 | >>>Miscellaneous Cause | ELEMENT | FALSE | Miscellaneous Cause IE in TS 38.423 [15] Section 9.2.3.2 |  |  |

##### 8.1.2.3.3	Handover Resource Allocation

| RAN Parameter ID | RAN Parameter | RAN Parameter Value Type | Key Flag | RAN Parameter Definition | Semantics Description |
| --- | --- | --- | --- | --- | --- |
| 27301 | Event AN and inter-RAT B1 mobility thresholds | STRUCTURE |  | Connectivity and Mobility Event Thresholds IE in clause 8.1.1.10 |  |
| 27302 | Handover Type | ELEMENT | FALSE | Handover Type IE in TS 38.413 [11] Section 9.3.1.22 |  |
| 27303 | List of PDU Sessions for Handover | LIST |  |  | PDU Session Resources To Be Setup List IE in TS 38.423 [15] Section 9.2.1.1 |
| 27304 | >PDU Session Item | STRUCTURE |  |  | PDU Session Resources To Be Setup Item IE in TS 38.423 [15] Section 9.2.1.1 |
| 27305 | >>PDU Session ID | ELEMENT | TRUE | PDU Session ID IE in TS 38.423 [15] Section 9.2.3.18 |  |
| 27306 | >>PDU Session | STRUCTURE |  | 8.1.1.16 |  |
| 27307 | >>List of QoS flows in the PDU Session | LIST |  |  | QoS Flows To Be Setup List IE in TS 38.423 [15] Section 9.2.1.1 |
| 27308 | >>>QoS Flow Item | LIST |  |  | QoS Flow To Be Setup Item IE in TS 38.423 [15] Section 9.2.1.1 |
| 27309 | >>>>QoS Flow Identifier | ELEMENT | TRUE | QoS Flow Identifier IE in TS 38.423 [15] Section 9.2.3.10 |  |
| 27310 | >>>>QoS Flow | STRUCTURE |  | 8.1.1.6 |  |
| 27311 | >>List of DRBs for Handover | LIST |  |  | DRB to Be Setup List IE in TS 38.473 [19] Section 9.2.2.1 |
| 27312 | >>>DRB Item for Handover | STRUCTURE |  |  | DRB to Be Setup Item IE in TS 38.473 [19] Section 9.2.2.1 |
| 27313 | >>>>DRB ID | ELEMENT | TRUE | DRB ID IE in TS 38.473 [19] Section 9.3.1.8 |  |
| 27314 | >>>>CHOICE DRB Type | STRUCTURE |  |  | DRB to Be Setup Item IE in TS 38.473 [19] Section 9.2.2.1 |
| 27315 | >>>>>NG-RAN DRB | STRUCTURE |  | 8.1.1.5 |  |
| 27316 | >>>>>E-UTRA DRB | STRUCTURE |  | 8.1.1.9 |  |
| 27317 | >>>>List of QoS flows in the DRB | LIST |  |  | QoS Flows Information To Be Setup IE in TS 37.483 [21] Section 9.3.3.2 |
| 27318 | >>>>>QoS Flow Item | STRUCTURE |  |  | QoS Flow Item IE in TS 37.483 [21] Section 9.3.1.25 |
| 27319 | >>>>>>QoS Flow Identifier | ELEMENT | TRUE | QoS Flow Identifier IE in TS 37.483 [21] Section 9.3.1.25 |  |
| 27320 | >>>>>>QoS Flow | STRUCTURE |  | 8.1.1.6 |  |
| 27321 | List of DRBs for Handover | LIST |  |  | DRB to Be Setup List IE in TS 38.473 [19] Section 9.2.2.1 |
| 27322 | >DRB Item for Handover | STRUCTURE |  |  | DRB to Be Setup Item IE in TS 38.473 [19] Section 9.2.2.1 |
| 27323 | >>DRB ID | ELEMENT | TRUE | DRB ID IE in TS 38.473 [19] Section 9.3.1.8 |  |
| 27324 | >>CHOICE DRB Type | STRUCTURE |  |  | DRB to Be Setup Item IE in TS 38.473 [19] Section 9.2.2.1 |
| 27325 | >>>NG-RAN DRB | STRUCTURE |  | 8.1.1.5 |  |
| 27326 | >>>E-UTRA DRB | STRUCTURE |  | 8.1.1.9 |  |
| 27327 | >>List of QoS flows in the DRB | LIST |  |  | QoS Flows Information To Be Setup IE in TS 37.483 [21] Section 9.3.3.2 |
| 27328 | >>>QoS Flow Item | STRUCTURE |  |  | QoS Flow Item IE in TS 37.483 [21] Section 9.3.1.25 |
| 27329 | >>>>QoS Flow Identifier | ELEMENT | TRUE | QoS Flow Identifier IE in TS 37.483 [21] Section 9.3.1.25 |  |
| 27330 | >>>>QoS Flow | STRUCTURE |  | 8.1.1.6 |  |
| 27331 | Allowed S-NSSAI | LIST |  |  | Allowed S-NSSAI IE in TS 38.413 [11] Section 9.3.1.31 |
| 27332 | >Allowed S-NSSAI Item | STRUCTURE |  |  | Allowed S-NSSAI Item IE in TS 38.413 [11] Section 9.3.1.31 |
| 27333 | >>S-NSSAI | STRUCTURE |  |  | S-NSSAI IE in TS 38.413 [11] Section 9.3.1.24 |
| 27334 | >>>SST | ELEMENT | TRUE | SST IE in TS 38.413 [11] Section 9.3.1.24 |  |
| 27335 | >>>SD | ELEMENT | FALSE | SD IE in TS 38.413 [11] Section 9.3.1.24 |  |
| 27336 | Mobility Restriction List | STRUCTURE |  |  | Mobility Restriction List IE in TS 38.413 [11] Section 9.3.1.85 |
| 27337 | >Serving PLMN | ELEMENT | FALSE | PLMN Identity IE in TS 38.413 [11] Section 9.3.3.5 |  |
| 27338 | >List of Equivalent PLMNs | LIST |  |  | Equivalent PLMNs IE in TS 38.413 [11] Section 9.3.1.85 |
| 27339 | >>Equivalent PLMN Item | STRUCTURE |  |  |  |
| 27340 | >>>PLMN Identity | ELEMENT | FALSE | PLMN Identity IE in TS 38.413 [11] Section 9.3.3.5 |  |
| 27341 | >RAT Restrictions | LIST |  |  | RAT Restrictions IE in TS 38.413 [11] Section 9.3.1.85 |
| 27342 | >>RAT Restriction Item | STRUCTURE |  |  |  |
| 27343 | >>>PLMN Identity | ELEMENT | FALSE | PLMN Identity IE in TS 38.413 [11] Section 9.3.3.5 |  |
| 27344 | >>>RAT Restriction Information | ELEMENT | FALSE | RAT Restriction Information IE in TS 38.413 [11] Section 9.3.1.85 |  |
| 27345 | >Forbidden Area Information | LIST |  |  | Forbidden Area Information IE in TS 38.413 [11] Section 9.3.1.85 |
| 27346 | >>Forbidden Area | STRUCTURE |  |  | Forbidden Area Information IE in TS 38.413 [11] Section 9.3.1.85 |
| 27347 | >>>PLMN Identity | ELEMENT | FALSE | PLMN Identity IE in TS 38.413 [11] Section 9.3.3.5 |  |
| 27348 | >>>List of Forbidden TACs | LIST |  |  | Forbidden TACs IE in TS 38.413 [11] Section 9.3.1.85 |
| 27349 | >>>>Forbidden TAC Item | STRUCTURE |  |  | Forbidden TACs IE in TS 38.413 [11] Section 9.3.1.85 |
| 27350 | >>>>>TAC | ELEMENT | FALSE | TAC IE in TS 38.413 [11] Section 9.3.3.10 |  |
| 27351 | >Service Area Information | LIST |  |  | Service Area Information IE in TS 38.413 [11] Section 9.3.1.85 |
| 27352 | >>Service Area | STRUCTURE |  |  | Service Area Information IE in TS 38.413 [11] Section 9.3.1.85 |
| 27353 | >>>PLMN Identity | ELEMENT | FALSE | PLMN Identity IE in TS 38.413 [11] Section 9.3.3.5 |  |
| 27364 | >>>List of allowed TACs | LIST |  |  | Allowed TACs IE in TS 38.413 [11] Section 9.3.1.85 |
| 27365 | >>>>Allowed TAC Item | STRUCTURE |  |  | Allowed TACs IE in TS 38.413 [11] Section 9.3.1.85 |
| 27366 | >>>>>TAC | ELEMENT | FALSE | TAC IE in TS 38.413 [11] Section 9.3.3.10 |  |
| 27367 | >>>List of Not allowed TACs | LIST |  |  | Not Allowed TACs IE in TS 38.413 [11] Section 9.3.1.85 |
| 27368 | >>>>Not Allowed TAC Item | STRUCTURE |  |  | Not Allowed TACs IE in TS 38.413 [11] Section 9.3.1.85 |
| 27369 | >>>>>TAC | ELEMENT | FALSE | TAC IE in TS 38.413 [11] Section 9.3.3.10 |  |

#### 8.1.2.4	Multi-RAT Dual Connectivity Management

The RAN Parameters for the call process type of “Multi-RAT Dual Connectivity Management” are defined as follows.

##### 8.1.2.4.1	SN Addition

| RAN Parameter ID | RAN Parameter | RAN Parameter Value Type | Key Flag | RAN Parameter Definition | Semantics Description |
| --- | --- | --- | --- | --- | --- |
| 28101 | Event AN and inter-RAT B1 mobility thresholds | STRUCTURE |  | Connectivity and Mobility Event Thresholds IE in clause 8.1.1.10 |  |
| 28102 | Secondary Node | STRUCTURE |  | 8.1.1.11 |  |  |
| 28103 | CHOICE PDCP Change Indication | STRUCTURE |  |  | PDCP Change Indication IE in TS 38.423 [15] Section 9.2.3.74 |  |
| 28104 | >From S-NG-RAN Node | STRUCTURE |  |  | From S-NG-RAN Node IE in TS 38.423 [15] Section 9.2.3.74 |  |
| 28105 | >>Indication from S-NG-RAN node to M-NG-RAN node | ELEMENT | FALSE | Indication from S-NG-RAN node to M-NG-RAN node IE in TS 38.423 [15] Section 9.2.3.74 |  |  |
| 28106 | >From M-NG-RAN Node | STRUCTURE |  |  | From M-NG-RAN Node IE in TS 38.423 [15] Section 9.2.3.74 |  |
| 28107 | >>Indication from M-NG-RAN node to S-NG-RAN node | ELEMENT | FALSE | Indication from M-NG-RAN node to S-NG-RAN node IE in TS 38.423 [15] Section 9.2.3.74 |  |  |
| 28108 | Secondary Node UE Aggregate Maximum Bit Rate | STRUCTURE |  |  | UE Aggregate Maximum Bit Rate IE in TS 36.423 [17] Section 9.2.12 |  |
| 28109 | >UE Aggregate Maximum Bit Rate Downlink | ELEMENT | FALSE | UE Aggregate Maximum Bit Rate Downlink IE in TS 36.423 [17] clause 9.2.12 |  |  |
| 28110 | >UE Aggregate Maximum Bit Rate Uplink | ELEMENT | FALSE | UE Aggregate Maximum Bit Rate Uplink IE in TS 36.423 [17] clause 9.2.12 |  |  |
| 28111 | List of PDU sessions to be added | LIST |  |  | PDU Session Resources To Be Setup List IE in TS 38.423 [15] Section 9.2.1.1 |  |
| 28112 | >PDU Session Item | STRUCTURE |  |  | PDU Session Resources To Be Setup Item IE in TS 38.423 [15] Section 9.2.1.1 |  |
| 28113 | >>PDU Session ID | ELEMENT | TRUE | PDU Session ID IE in TS 38.423 [15] Section 9.2.3.18 |  |  |
| 28114 | >>PDU Session | STRUCTURE |  | 8.1.1.16 |  |  |
| 28115 | >>CHOICE PDU Session Setup Info | STRUCTURE |  |  | PDU Session Resource Setup Info – SN terminated IE in TS 38.423 [15] Section 9.1.2.5 |  |
| 28116 | >>>SN terminated PDU session | STRUCTURE |  |  | PDU Session Resource Setup Info – SN terminated IE in TS 38.423 [15] Section 9.1.2.5 |  |
| 28118 | >>>>List of QoS flows to be setup | LIST |  |  | QoS Flows To Be Setup List IE in TS 38.423 [15] Section 9.2.1.1 |  |
| 28119 | >>>>>QoS flow Item to be setup | STRUCTURE |  |  | QoS Flow To Be Setup Item IE in TS 38.423 [15] Section 9.2.1.1 |  |
| 28120 | >>>>>>QoS Flow Indicator | ELEMENT | TRUE | QoS Flow Identifier IE in TS 38.423 [15] Section 9.2.3.10 |  |  |
| 28121 | >>>>>>QoS flow | STRUCTURE |  | 8.1.1.6 |  |  |
| 28122 | >>>>Count of QoS flows to be setup | ELEMENT | FALSE | INTEGER (0..64) |  |  |
| 28123 | >>>MN terminated PDU session | STRUCTURE |  |  | PDU Session Resource Setup Info – MN terminated IE in TS 38.423 [15] Section 9.1.2.5 |  |
| 28125 | >>>>List of DRBs to be setup | LIST |  |  | DRB to QoS Flow Mapping List IE in TS 38.423 [15] Section 9.2.1.15 |  |
| 28126 | >>>>>DRB Item to be setup | STRUCTURE |  |  | DRB to QoS Flow Mapping Item IE in TS 38.423 [15] Section 9.2.1.15 |  |
| 28127 | >>>>>>DRB ID | ELEMENT | TRUE | DRB ID IE in TS 38.423 [15] Section 9.2.3.33 |  |  |
| 28128 | >>>>>>NG-RAN DRB | STRUCTURE |  | 8.1.1.5 |  |  |
| 28129 | >>>>>>List of QoS flows to be setup | LIST |  |  | QoS Flows List IE in TS 38.423 [15] Section 9.2.1.15 |  |
| 28130 | >>>>>>>QoS flow Item to be setup | STRUCTURE |  |  | QoS Flow Item IE in TS 38.423 [15] Section 9.2.1.15 |  |
| 28131 | >>>>>>>>QoS Flow Indicator | ELEMENT | TRUE | QoS Flow Indicator IE in TS 38.423 [15] Section 9.2.3.10 |  |  |
| 28132 | >>>>>>>>QoS Flow | STRUCTURE |  | 8.1.1.6 |  |  |
| 28133 | >>>>>>Count of list of QoS flows to be setup | ELEMENT | FALSE | INTEGER (0..64) |  |  |
| 28134 | >>>>Count of list of DRBs to be setup | ELEMENT | FALSE | INTEGER (0..64) |  |  |
| 28135 | Count of list of PDU sessions to be added | ELEMENT | FALSE | INTEGER (0..256) |  |  |
| 28136 | List of E-RABs to be added | LIST |  |  | E-RABs To Be Added List IE in TS 36.423 [17] Section 9.1.4.1 |  |
| 28137 | >E-RAB Item | STRUCTURE |  |  | E-RABs To Be Added Item IE in TS 36.423 [17] Section 9.1.4.1 |  |
| 28138 | >>E-RAB ID | ELEMENT | TRUE | E-RAB ID IE in TS 36.423 [17] Section 9.2.23 |  |  |
| 28139 | >>E-RAB | STRUCTURE |  | 8.1.1.9 |  |  |
| 28140 | >>EN-DC Resource Configuration | STRUCTURE |  |  | EN-DC Resource Configuration IE in TS 36.423 [17] Section 9.2.108 |  |
| 28141 | >>>PDCP at SgNB | ELEMENT | FALSE | PDCP at SgNB IE in TS 36.423 [17] Section 9.2.108 |  |  |
| 28142 | >>>MCG resources | ELEMENT | FALSE | MCG resources IE in TS 36.423 [17] Section 9.2.108 |  |  |
| 28143 | >>>SCG resources | ELEMENT | FALSE | SCG resources IE in TS 36.423 [17] Section 9.2.108 |  |  |
| 28144 | >>CHOICE Resource Configuration | STRUCTURE |  |  | Resource Configuration IE in TS 36.423 [17] Section 9.1.4.1 |  |
| 28145 | >>>PDCP present in SN | STRUCTURE |  |  | PDCP Present in SN IE in TS 36.423 [17] Section 9.1.4.1 |  |
| 28146 | >>>>Maximum MCG admittable E-RAB Level QoS parameters | STRUCTURE |  | 8.1.1.9 |  |  |
| 28147 | >>>>RLC Mode | ELEMENT | FALSE | RLC Mode IE in TS 36.423 [17] Section 9.1.4.1 |  |  |
| 28148 | >>>>UL Configuration | ELEMENT | FALSE | UL Configuration IE in TS 36.423 [17] Section 9.1.4.1 |  |  |
| 28149 | >>>PDCP not present in SN | STRUCTURE |  |  | PDCP Not Present in SN IE in TS 36.423 [17] Section 9.1.4.1 |  |
| 28150 | >>>>Requested SCG E-RAB Level QoS Parameters | STRUCTURE |  | 8.1.1.9 |  |  |
| 28151 | >>>>RLC Mode | ELEMENT | FALSE | RLC Mode IE in TS 36.423 [17] Section 9.1.4.1 |  |  |
| 28152 | >>>>UL Configuration | ELEMENT | FALSE | UL Configuration IE in TS 36.423 [17] Section 9.1.4.1 |  |  |
| 28153 | Count of list of E-RABs to be added | ELEMENT | FALSE | INTEGER (0..256) |  |  |
| 28154 | List of SN-terminated DRBs | LIST |  |  | Available DRB IDs IE in TS 38.423 [15] Section 9.1.2.1 |  |
| 28155 | >SN-terminated DRB item | STRUCTURE |  |  |  |  |
| 28156 | >>DRB ID | ELEMENT | TRUE | DRB ID IE in TS 38.423 [15] Section 9.2.3.33 |  |  |
| 28157 | >>CHOICE DRB Type | STRUCTURE |  |  |  |  |
| 28158 | >>>NG-RAN DRB | STRUCTURE |  | 8.1.1.5 |  |  |
| 28159 | >>>E-UTRA DRB | STRUCTURE |  | 8.1.1.9 |  |  |
| 28160 | Count of list of SN-terminated DRBs | ELEMENT | FALSE | INTEGER (0..64) |  |  |
| 28161 | Primary Cell ID | STRUCTURE |  |  | This is for the primary serving cell of the UE. The structuring of this parameter is based on Target Cell Global ID IE in TS 38.423 [15] clause 9.2.3.25 |  |
| 28162 | >CHOICE Primary Cell | STRUCTURE |  |  | The primary cell could either be an NR primary cell or an LTE primary cell. The structuring is based onTarget Cell IE in TS 38.423 [15] clause 9.2.3.25 |  |
| 28163 | >>NR SpCell | STRUCTURE |  | 8.1.1.1 | NR Cell IE in TS 38.423 [15] clause 9.2.3.25 or SpCell ID IE in TS 38.473 [19] clause 9.2.2.1 |  |
| 28164 | >>LTE PCell | STRUCTURE |  | 8.1.1.2 | E-UTRA Cell IE in TS 38.423 [15] clause 9.2.3.25 |  |

##### 8.1.2.4.2	SN Modification and deletion

| RAN Parameter ID | RAN Parameter | RAN Parameter Value Type | Key Flag | RAN Parameter Definition | Semantics Description |
| --- | --- | --- | --- | --- | --- |
| 28201 | Event AN and inter-RAT B1 mobility thresholds | STRUCTURE |  | Connectivity and Mobility Event Thresholds IE in clause 8.1.1.10 | Event AN and inter-RAT B1 mobility thresholds |  |
| 28202 | Secondary Node | STRUCTURE |  | 8.1.1.11 |  |  |
| 28205 | CHOICE PDCP Change Indication | STRUCTURE |  |  | PDCP Change Indication IE in TS 38.423 [15] Section 9.2.3.74 |  |
| 28206 | >From S-NG-RAN Node | STRUCTURE |  |  | From S-NG-RAN Node IE in TS 38.423 [15] Section 9.2.3.74 |  |
| 28207 | >>Indication from S-NG-RAN node to M-NG-RAN node | STRUCTURE |  | Indication from S-NG-RAN node to M-NG-RAN node IE in TS 38.423 [15] Section 9.2.3.74 |  |  |
| 28208 | >From M-NG-RAN Node | STRUCTURE |  |  | From M-NG-RAN Node IE in TS 38.423 [15] Section 9.2.3.74 |  |
| 28209 | >>Indication from M-NG-RAN node to S-NG-RAN node | STRUCTURE |  | Indication from M-NG-RAN node to S-NG-RAN node IE in TS 38.423 [15] Section 9.2.3.74 |  |  |
| 28210 | Secondary Node UE Aggregate Maximum Bit Rate | STRUCTURE |  |  | UE Aggregate Maximum Bit Rate IE in TS 36.423 [17] Section 9.2.12 |  |
| 28211 | >UE Aggregate Maximum Bit Rate Downlink | ELEMENT | FALSE | UE Aggregate Maximum Bit Rate Downlink IE in TS 36.423 [17] clause 9.2.12 |  |  |
| 28212 | >UE Aggregate Maximum Bit Rate Uplink | ELEMENT | FALSE | UE Aggregate Maximum Bit Rate Uplink IE in TS 36.423 [17] clause 9.2.12 |  |  |
| 28213 | List of PDU sessions to be added | LIST |  |  | PDU Session Resources To Be Setup List IE in TS 38.423 [15] Section 9.2.1.1 |  |
| 28214 | >PDU Session Item | STRUCTURE |  |  | PDU Session Resources To Be Setup Item IE in TS 38.423 [15] Section 9.2.1.1 |  |
| 28215 | >>PDU Session ID | ELEMENT | TRUE | PDU Session ID IE in TS 38.423 [15] Section 9.2.3.18 |  |  |
| 28216 | >>PDU Session | STRUCTURE |  | 8.1.1.16 |  |  |
| 28217 | >>CHOICE PDU Session Setup Info | STRUCTURE |  |  | PDU Session Resource Setup Info – SN terminated IE in TS 38.423 [15] Section 9.1.2.5 |  |
| 28218 | >>>SN terminated PDU session | STRUCTURE |  |  | PDU Session Resource Setup Info – SN terminated IE in TS 38.423 [15] Section 9.1.2.5 |  |
| 28219 | >>>>List of QoS flows to be setup | LIST |  |  | QoS Flows To Be Setup List IE in TS 38.423 [15] Section 9.2.1.1 |  |
| 28220 | >>>>>QoS flow Item to be setup | STRUCTURE |  |  | QoS Flow To Be Setup Item IE in TS 38.423 [15] Section 9.2.1.1 |  |
| 28221 | >>>>>>QoS Flow Indicator | ELEMENT | TRUE | QoS Flow Identifier IE in TS 38.423 [15] Section 9.2.3.10 |  |  |
| 28222 | >>>>>>QoS flow | STRUCTURE |  | 8.1.1.6 |  |  |
| 28223 | >>>>Count of QoS flows to be setup | ELEMENT | FALSE | INTEGER (0..64) |  |  |
| 28224 | >>>MN terminated PDU session | STRUCTURE |  |  | PDU Session Resource Setup Info – MN terminated IE in TS 38.423 [15] Section 9.1.2.5 |  |
| 28225 | >>>>List of DRBs to be setup | LIST |  |  | DRB to QoS Flow Mapping List IE in TS 38.423 [15] Section 9.2.1.15 |  |
| 28226 | >>>>>DRB Item to be setup | STRUCTURE |  |  | DRB to QoS Flow Mapping Item IE in TS 38.423 [15] Section 9.2.1.15 |  |
| 28227 | >>>>>>DRB ID | ELEMENT | TRUE | DRB ID IE in TS 38.423 [15] Section 9.2.3.33 |  |  |
| 28228 | >>>>>>NG-RAN DRB | STRUCTURE |  | 8.1.1.5 |  |  |
| 28229 | >>>>>>List of QoS flows to be setup | LIST |  |  | QoS Flows List IE in TS 38.423 [15] Section 9.2.1.15 |  |
| 28230 | >>>>>>>QoS flow Item to be setup | STRUCTURE |  |  | QoS Flow Item IE in TS 38.423 [15] Section 9.2.1.15 |  |
| 28231 | >>>>>>>>QoS Flow Indicator | ELEMENT | TRUE | QoS Flow Indicator IE in TS 38.423 [15] Section 9.2.3.10 |  |  |
| 28232 | >>>>>>>>QoS Flow | STRUCTURE |  | 8.1.1.6 |  |  |
| 28233 | >>>>>>Count of list of QoS flows to be setup | ELEMENT | FALSE | INTEGER (0..64) |  |  |
| 28234 | >>>>Count of list of DRBs to be setup | ELEMENT | FALSE | INTEGER (0..64) |  |  |
| 28235 | Count of list of PDU sessions to be added | ELEMENT | FALSE | INTEGER (0..256) |  |  |
| 28236 | List of PDU sessions to be modified | LIST |  |  | PDU Session Resources To Be Setup List IE in TS 38.423 [15] Section 9.2.1.1 |  |
| 28237 | >PDU Session Item | STRUCTURE |  |  | PDU Session Resources To Be Setup Item IE in TS 38.423 [15] Section 9.2.1.1 |  |
| 28238 | >>PDU Session ID | ELEMENT | TRUE | PDU Session ID IE in TS 38.423 [15] Section 9.2.3.18 |  |  |
| 28239 | >>PDU Session | STRUCTURE |  | 8.1.1.16 |  |  |
| 28240 | >>CHOICE PDU Session Modification Info | STRUCTURE |  |  |  |  |
| 28241 | >>>SN terminated PDU session | STRUCTURE |  |  | PDU Session Resource Setup Info – SN terminated IE in TS 38.423 [15] Section 9.1.2.5 |  |
| 28242 | >>>>List of QoS flows to be modified | LIST |  |  | QoS Flows To Be Setup List IE in TS 38.423 [15] Section 9.2.1.1 |  |
| 28243 | >>>>>QoS flow Item to be modified | STRUCTURE |  |  | QoS Flow To Be Setup Item IE in TS 38.423 [15] Section 9.2.1.1 |  |
| 28244 | >>>>>>QoS Flow Indicator | ELEMENT | TRUE | QoS Flow Identifier IE in TS 38.423 [15] Section 9.2.3.10 |  |  |
| 28245 | >>>>>>QoS flow | STRUCTURE |  | 8.1.1.6 |  |  |
| 28246 | >>>>Count of QoS flows to be modified | ELEMENT | FALSE | INTEGER (0..64) |  |  |
| 28247 | >>>MN terminated PDU session | STRUCTURE |  |  | PDU Session Resource Setup Info – MN terminated IE in TS 38.423 [15] Section 9.1.2.5 |  |
| 28248 | >>>>List of DRBs to be modified | LIST |  |  | DRB to QoS Flow Mapping List IE in TS 38.423 [15] Section 9.2.1.15 |  |
| 28249 | >>>>>DRB Item to be modified | STRUCTURE |  |  | DRB to QoS Flow Mapping Item IE in TS 38.423 [15] Section 9.2.1.15 |  |
| 28250 | >>>>>>DRB ID | ELEMENT | TRUE | DRB ID IE in TS 38.423 [15] Section 9.2.3.33 |  |  |
| 28251 | >>>>>>NG-RAN DRB | STRUCTURE |  | 8.1.1.5 |  |  |
| 28252 | >>>>>>List of QoS flows to be modified | LIST |  |  | QoS Flows List IE in TS 38.423 [15] Section 9.2.1.15 |  |
| 28253 | >>>>>>>QoS flow Item to be setup | STRUCTURE |  |  | QoS Flow Item IE in TS 38.423 [15] Section 9.2.1.15 |  |
| 28254 | >>>>>>>>QoS Flow Indicator | ELEMENT | TRUE | QoS Flow Indicator IE in TS 38.423 [15] Section 9.2.3.10 |  |  |
| 28255 | >>>>>>>>QoS Flow | STRUCTURE |  | 8.1.1.6 |  |  |
| 28256 | >>>>>>Count of list of QoS flows to be setup | ELEMENT | FALSE | INTEGER (0..64) |  |  |
| 28257 | >>>>Count of list of DRBs to be modified | ELEMENT | FALSE | INTEGER (0..64) |  |  |
| 28258 | >>MR-DC Usage Information | STRUCTURE |  |  | MR-DC Usage Information IE in TS 37.483 [21] Section 9.3.1.63 |  |
| 28259 | >>>Secondary RAT Type | ELEMENT | FALSE | Secondary RAT Type IE in TS 37.483 [21] Section 9.3.1.63 |  |  |
| 28260 | >>>PDU Session Timed Report List | LIST |  |  | Data Usage Report List IE in TS 37.483 [21] Section 9.3.1.64 |  |
| 28261 | >>>>MR-DC Data Usage Report Item | STRUCTURE |  |  | Data Report Usage Item IE in TS 37.483 [21] Section 9.3.1.64 |  |
| 28262 | >>>>>Start Timestamp | ELEMENT | FALSE | Start timestamp IE in TS 37.483 [21] Section 9.3.1.64 |  |  |
| 28263 | >>>>>End Timestamp | ELEMENT | FALSE | End timestamp IE in TS 37.483 [21] Section 9.3.1.64 |  |  |
| 28264 | >>>>>Usage Count UL | ELEMENT | FALSE | Usage count DL IE in TS 37.483 [21] Section 9.3.1.64 |  |  |
| 28265 | >>>>>Usage Count DL | ELEMENT | FALSE | Usage count DL IE in TS 37.483 [21] Section 9.3.1.64 |  |  |
| 28266 | >>MR-DC Usage for QoS flows | LIST |  |  | Data Usage Per QoS Flow List IE in TS 37.483 [21] Section 9.3.1.63 |  |
| 28267 | >>>QoS Flow Item | STRUCTURE |  |  | Data Usage Per QoS Flow Item IE in TS 37.483 [21] Section 9.3.1.63 |  |
| 28268 | >>>>QoS flow Indicator | ELEMENT | TRUE | QoS Flow Indicator IE in TS 37.483 [21] Section 9.3.1.24 |  |  |
| 28269 | >>>>Secondary RAT Type | ELEMENT | FALSE | Secondary RAT Type IE in TS 37.483 [21] Section 9.3.1.63 |  |  |
| 28270 | >>>>QoS Flow Timed Report List | LIST |  |  | Data Usage Report List IE in TS 37.483 [21] Section 9.3.1.64 |  |
| 28271 | >>>>>MR-DC Data Usage Report Item | STRUCTURE |  |  | Data Report Usage Item IE in TS 37.483 [21] Section 9.3.1.64 |  |
| 28272 | >>>>>>Start Timestamp | ELEMENT | FALSE | Start timestamp IE in TS 37.483 [21] Section 9.3.1.64 |  |  |
| 28273 | >>>>>>End timestamp | ELEMENT | FALSE | End timestamp IE in TS 37.483 [21] Section 9.3.1.64 |  |  |
| 28274 | >>>>>>Usage Count UL | ELEMENT | FALSE | Usage count DL IE in TS 37.483 [21] Section 9.3.1.64 |  |  |
| 28275 | >>>>>>Usage Count DL | ELEMENT | FALSE | Usage count DL IE in TS 37.483 [21] Section 9.3.1.64 |  |  |
| 28276 | Count of list of PDU sessions to be modified | ELEMENT | FALSE | INTEGER (0..256) |  |  |
| 28277 | List of PDU sessions to be released | LIST |  |  | PDU Session Resources To Be Released List IE in TS 38.423 [15] Section 9.1.2.14 |  |
| 28278 | >PDU Session Item | STRUCTURE |  |  | PDU Session List With Cause IE in TS 38.423 [15] Section 9.2.1.26 |  |
| 28279 | >>PDU Session ID | ELEMENT | TRUE | PDU Session ID IE in TS 38.423 [15] Section 9.2.3.18 |  |  |
| 28280 | >>PDU Session | STRUCTURE |  | 8.1.1.16 |  |  |
| 26281 | >>Release Cause | STRUCTURE |  |  | Cause IE in TS 37.483 [21] Section 9.3.1.2 |
| 26282 | >>>CHOICE Cause Group | STRUCTURE |  |  | Cause Group IE in TS 37.483 [21] Section 9.3.1.2 |
| 26283 | >>>>Radio Network Layer | STRUCTURE |  |  | Radio Network Layer IE in TS 37.483 [21] Section 9.3.1.2 |
| 26284 | >>>>>Radio Network Layer Cause | ELEMENT | FALSE | Radio Network Layer Cause IE in TS 37.483 [21] Section 9.3.1.2 |  |
| 26285 | >>>>Transport Layer | STRUCTURE |  |  | Transport Layer IE in TS 37.483 [21] Section 9.3.1.2 |
| 26286 | >>>>>Transport Layer Cause | ELEMENT | FALSE | Transport Layer Cause IE in TS 37.483 [21] Section 9.3.1.2 |  |
| 26287 | >>>>Protocol | STRUCTURE |  |  | Protocol IE in TS 37.483 [21] Section 9.3.1.2 |
| 26288 | >>>>>Protocol Cause | ELEMENT | FALSE | Protocol Cause IE in TS 37.483 [21] Section 9.3.1.2 |  |
| 26289 | >>>>Miscellaneous | STRUCTURE |  |  | Misc IE in TS 37.483 [21] Section 9.3.1.2 |
| 26290 | >>>>>Miscellaneous Cause | ELEMENT | FALSE | Miscellaneous Cause IE in TS 37.483 [21] Section 9.3.1.2 |  |
| 28291 | Count of list of PDU sessions to be released | ELEMENT | FALSE | INTEGER (0..256) |  |  |
| 28292 | List of E-RABs to be added | LIST |  |  | E-RABs To Be Added List IE in TS 36.423 [17] Section 9.1.4.1 |  |
| 28293 | >E-RAB Item | STRUCTURE |  |  | E-RABs To Be Added Item IE in TS 36.423 [17] Section 9.1.4.1 |  |
| 28294 | >>E-RAB ID | ELEMENT | TRUE | E-RAB ID IE in TS 36.423 [17] Section 9.2.23 |  |  |
| 28295 | >>E-RAB | STRUCTURE |  | 8.1.1.9 |  |  |
| 28296 | >>EN-DC Resource Configuration | STRUCTURE |  |  | EN-DC Resource Configuration IE in TS 36.423 [17] Section 9.2.108 |  |
| 28297 | >>>PDCP at SgNB | STRUCTURE |  | PDCP at SgNB IE in TS 36.423 [17] Section 9.2.108 |  |  |
| 28298 | >>>MCG resources | STRUCTURE |  | MCG resources IE in TS 36.423 [17] Section 9.2.108 |  |  |
| 28299 | >>>SCG resources | STRUCTURE |  | SCG resources IE in TS 36.423 [17] Section 9.2.108 |  |  |
| 28300 | >>CHOICE Resource Configuration | STRUCTURE |  |  | Resource Configuration IE in TS 36.423 [17] Section 9.1.4.1 |  |
| 28301 | >>>PDCP present in SN | STRUCTURE |  |  | PDCP Present in SN IE in TS 36.423 [17] Section 9.1.4.1 |  |
| 28302 | >>>>Maximum MCG admittable E-RAB Level QoS parameters | STRUCTURE |  | 8.1.1.9 |  |  |
| 28303 | >>>>RLC Mode | ELEMENT | FALSE | RLC Mode IE in TS 36.423 [17] Section 9.1.4.1 |  |  |
| 28304 | >>>>UL Configuration | ELEMENT | FALSE | UL Configuration IE in TS 36.423 [17] Section 9.1.4.1 |  |  |
| 28305 | >>>PDCP not present in SN | STRUCTURE |  |  | PDCP Not Present in SN IE in TS 36.423 [17] Section 9.1.4.1 |  |
| 28306 | >>>>Requested SCG E-RAB Level QoS Parameters | STRUCTURE |  | 8.1.1.9 |  |  |
| 28307 | >>>>RLC Mode | ELEMENT | FALSE | RLC Mode IE in TS 36.423 [17] Section 9.1.4.1 |  |  |
| 28308 | >>>>UL Configuration | ELEMENT | FALSE | UL Configuration IE in TS 36.423 [17] Section 9.1.4.1 |  |  |
| 28309 | Count of list of E-RABs to be added | ELEMENT | FALSE | INTEGER (0..256) |  |  |
| 28310 | List of E-RABs to be modified | LIST |  |  | E-RABs To Be Modified List IE in TS 36.423 [17] Section 9.1.4.5 |  |
| 28311 | >E-RAB Item | STRUCTURE |  |  | E-RABs To Be Modified Item IE in TS 36.423 [17] Section 9.1.4.5 |  |
| 28312 | >>E-RAB ID | ELEMENT | TRUE | E-RAB ID IE in TS 36.423 [17] Section 9.2.23 |  |  |
| 28313 | >>E-RAB | STRUCTURE |  | 8.1.1.9 |  |  |
| 28314 | >>EN-DC Resource Configuration | STRUCTURE |  |  | EN-DC Resource Configuration IE in TS 36.423 [17] Section 9.2.108 |  |
| 28315 | >>>PDCP at SgNB | STRUCTURE |  | PDCP at SgNB IE in TS 36.423 [17] Section 9.2.108 |  |  |
| 28316 | >>>MCG resources | STRUCTURE |  | MCG resources IE in TS 36.423 [17] Section 9.2.108 |  |  |
| 28317 | >>>SCG resources | STRUCTURE |  | SCG resources IE in TS 36.423 [17] Section 9.2.108 |  |  |
| 28318 | >>CHOICE Resource Configuration | STRUCTURE |  |  | Resource Configuration IE in TS 36.423 [17] Section 9.1.4.1 |  |
| 28319 | >>>PDCP present in SN | STRUCTURE |  |  | PDCP Present in SN IE in TS 36.423 [17] Section 9.1.4.1 |  |
| 28320 | >>>>Maximum MCG admittable E-RAB Level QoS parameters | STRUCTURE |  | 8.1.1.9 |  |  |
| 28321 | >>>>RLC Mode | ELEMENT | FALSE | RLC Mode IE in TS 36.423 [17] Section 9.1.4.1 |  |  |
| 28322 | >>>>UL Configuration | ELEMENT | FALSE | UL Configuration IE in TS 36.423 [17] Section 9.1.4.1 |  |  |
| 28323 | >>>PDCP not present in SN | STRUCTURE |  |  | PDCP Not Present in SN IE in TS 36.423 [17] Section 9.1.4.1 |  |
| 28324 | >>>>Requested SCG E-RAB Level QoS Parameters | STRUCTURE |  | 8.1.1.9 |  |  |
| 28325 | >>>>RLC Mode | ELEMENT | FALSE | RLC Mode IE in TS 36.423 [17] Section 9.1.4.1 |  |  |
| 28326 | >>>>UL Configuration | ELEMENT | FALSE | UL Configuration IE in TS 36.423 [17] Section 9.1.4.1 |  |  |
| 28327 | Count of list of E-RABs to be modified | ELEMENT | FALSE | INTEGER (0..256) |  |  |
| 28328 | List of E-RABs to be released | LIST |  |  | E-RABs To Be Released List IE in TS 36.423 [17] Section 9.1.4.5 |  |
| 28329 | >E-RAB Item | STRUCTURE |  |  | E-RABs To Be Released Item IE in TS 36.423 [17] Section 9.1.4.5 |  |
| 28330 | >>E-RAB ID | ELEMENT | TRUE | E-RAB ID IE in TS 36.423 [17] Section 9.2.23 |  |  |
| 28331 | >>E-RAB | STRUCTURE |  | 8.1.1.9 |  |  |
| 28332 | >>EN-DC Resource Configuration | STRUCTURE |  |  | EN-DC Resource Configuration IE in TS 36.423 [17] Section 9.2.108 |  |
| 28333 | >>>PDCP at SgNB | ELEMENT | FALSE | PDCP at SgNB IE in TS 36.423 [17] Section 9.2.108 |  |  |
| 28334 | >>>MCG resources | ELEMENT | FALSE | MCG resources IE in TS 36.423 [17] Section 9.2.108 |  |  |
| 28335 | >>>SCG resources | ELEMENT | FALSE | SCG resources IE in TS 36.423 [17] Section 9.2.108 |  |  |
| 28336 | >>CHOICE Resource Configuration | STRUCTURE |  |  | Resource Configuration IE in TS 36.423 [17] Section 9.1.4.1 |  |
| 28337 | >>>PDCP present in SN | STRUCTURE |  |  | PDCP Present in SN IE in TS 36.423 [17] Section 9.1.4.1 |  |
| 28338 | >>>>Maximum MCG admittable E-RAB Level QoS parameters | STRUCTURE |  | 8.1.1.9 |  |  |
| 28339 | >>>>RLC Mode | ELEMENT | FALSE | RLC Mode IE in TS 36.423 [17] Section 9.1.4.1 |  |  |
| 28340 | >>>>UL Configuration | ELEMENT | FALSE | UL Configuration IE in TS 36.423 [17] Section 9.1.4.1 |  |  |
| 28341 | >>>PDCP not present in SN | STRUCTURE |  |  | PDCP Not Present in SN IE in TS 36.423 [17] Section 9.1.4.1 |  |
| 28342 | >>>>Requested SCG E-RAB Level QoS Parameters | STRUCTURE |  | 8.1.1.9 |  |  |
| 28343 | >>>>RLC Mode | ELEMENT | FALSE | RLC Mode IE in TS 36.423 [17] Section 9.1.4.1 |  |  |
| 28344 | >>>>UL Configuration | ELEMENT | FALSE | UL Configuration IE in TS 36.423 [17] Section 9.1.4.1 |  |  |
| 28345 | Count of list of E-RABs to be released | ELEMENT | FALSE | INTEGER (0..256) |  |  |
| 28346 | List of SN-terminated DRBs to be modified or released | LIST |  |  | Additional DRB IDs IE in TS 38.423 [15] Section 9.1.2.5 |  |
| 28347 | >SN-terminated DRB item | STRUCTURE |  |  | Additional DRB IDs IE in TS 38.423 [15] Section 9.1.2.5 |  |
| 28358 | >>DRB ID | ELEMENT | TRUE | DRB ID IE in TS 38.423 [15] Section 9.2.3.33 |  |  |
| 28348 | >>CHOICE DRB Type | STRUCTURE |  |  | Additional DRB IDs IE in TS 38.423 [15] Section 9.1.2.5 |  |
| 28349 | >>>NG-RAN DRB | STRUCTURE |  | 8.1.1.5 |  |  |
| 28350 | >>>E-UTRA DRB | STRUCTURE |  | 8.1.1.9 |  |  |
| 28351 | Count of list of SN-terminated DRBs | ELEMENT | FALSE | INTEGER (0..64) |  |  |
| 28352 | Primary Cell ID | STRUCTURE |  |  | SpCell ID IE in TS 38.473 [19] clause 9.2.2.1 |  |
| 28353 | >CHOICE Primary Cell | STRUCTURE |  |  | SpCell ID IE in TS 38.473 [19] clause 9.2.2.1 |  |
| 28354 | >>NR SpCell | STRUCTURE |  | 8.1.1.1 |  |  |
| 28355 | >>LTE PCell | STRUCTURE |  | 8.1.1.2 |  |  |

#### 8.1.2.6	PDU Session Management

The RAN Parameters for the call process type of “PDU Session Management” are defined as follows.

##### 8.1.2.6.1	PDU Session Resource Setup

| RAN Parameter ID | RAN Parameter | RAN Parameter Value Type | Key Flag | RAN Parameter Definition | Semantics Description |
| --- | --- | --- | --- | --- | --- |
| 30001 | PDU Session ID | ELEMENT | TRUE | PDU Session ID IE in TS 38.413 [11] Section 9.3.1.50 |  |
| 30002 | PDU Session | STRUCTURE |  | 8.1.1.16 |  |
| 30003 | List of QoS Flows to be setup | LIST |  |  | QoS Flow Setup Request List IE in TS 38.413 [11] Section 9.3.4.1 |
| 30004 | >QoS flow setup request item | STRUCTURE |  |  | QoS Flow Setup Request Item IE in TS 38.413 [11] Section 9.3.4.1 |
| 30005 | >>QoS Flow Identifier | ELEMENT | TRUE | QoS Flow Identifier IE in TS 38.413 [11] Section 9.3.1.51 |  |
| 30006 | >>QoS flow | STRUCTURE |  | 8.1.1.6 |  |
| 30010 | List of QoS Flows failed to be setup | LIST |  |  | QoS Flow Failed To Setup List IE in TS 38.413 [11] Section 9.3.4.2 |
| 30011 | >QoS flow item failed for setup | STRUCTURE |  |  | QoS Flow Item IE in TS 38.413 [11] Section 9.3.1.13 |
| 30012 | >>QoS Flow Indicator | ELEMENT | TRUE | QoS Flow Identifier IE in TS 38.413 [11] Section 9.3.1.51 |  |
| 30013 | >>QoS flow | STRUCTURE |  | 8.1.1.6 |  |
| 30014 | >>Cause | STRUCTURE |  |  | Cause IE in TS 38.413 [11] Section 9.3.1.2 |
| 30015 | >>>CHOICE Cause group | STRUCTURE |  |  | Cause Group IE in TS 38.413 [11] Section 9.3.1.2 |
| 30016 | >>>>Radio Network Layer | STRUCTURE |  |  | Radio Network Layer IE in TS 38.413 [11] Section 9.3.1.2 |
| 30017 | >>>>>Radio Network Layer Cause | ELEMENT | FALSE | Radio Network Layer Cause IE in TS 38.413 [11] Section 9.3.1.2 |  |
| 30018 | >>>>Transport Layer | STRUCTURE |  |  | Transport Layer IE in TS 38.413 [11] Section 9.3.1.2 |
| 30019 | >>>>>Transport Layer Cause | ELEMENT | FALSE | Transport Layer Cause IE in TS 37.483 [21] Section 9.3.1.2 |  |
| 30020 | >>>>NAS | STRUCTURE |  |  | NAS IE in TS 38.413 [11] Section 9.3.1.2 |
| 30021 | >>>>>NAS Cause | ELEMENT | FALSE | NAS Cause IE in TS 38.413 [11] Section 9.3.1.2 |  |
| 30022 | >>>>Protocol | STRUCTURE |  |  | Protocol IE in TS 38.413 [11] Section 9.3.1.2 |
| 30023 | >>>>>Protocol Cause | ELEMENT | FALSE | Protocol Cause IE in TS 38.413 [11] Section 9.3.1.2 |  |
| 30024 | >>>>Miscellaneous | STRUCTURE |  |  | Misc IE in TS 38.413 [11] Section 9.3.1.2 |
| 30025 | >>>>>Miscellaneous Cause | ELEMENT | FALSE | Miscellaneous Cause IE in TS 38.413 [11] Section 9.3.1.2 |  |
| 30031 | Primary Cell ID | STRUCTURE |  |  | This is for the primary serving cell of the UE. The structuring of this parameter is based on Target Cell Global ID IE in TS 38.423 [15] clause 9.2.3.25 |
| 30032 | >CHOICE Primary Cell | STRUCTURE |  |  | The primary cell could either be an NR primary cell or an LTE primary cell. The structuring is based onTarget Cell IE in TS 38.423 [15] clause 9.2.3.25 |
| 30033 | >>NR SpCell | STRUCTURE |  | 8.1.1.1 | NR Cell IE in TS 38.423 [15] clause 9.2.3.25 |
| 30034 | >>E-UTRA PCell | STRUCTURE |  | 8.1.1.2 | E-UTRA Cell IE in TS 38.423 [15] clause 9.2.3.25 |

##### 8.1.2.6.2	PDU Session Resource Modification

| RAN Parameter ID | RAN Parameter | RAN Parameter Value Type | Key Flag | RAN Parameter Definition | Semantics Description |
| --- | --- | --- | --- | --- | --- |
| 31001 | PDU Session ID | ELEMENT | TRUE | PDU Session ID IE in TS 38.413 [11] Section 9.3.1.50 |  |
| 31002 | PDU Session | STRUCTURE |  | 8.1.1.16 |  |
| 31003 | List of QoS Flows to add or modify | LIST |  |  | QoS Flow Add or Modify Request List IE in TS 38.413 [11] Section 9.3.4.3 |
| 31004 | >QoS flow add or modify request item | STRUCTURE |  |  | QoS Flow Add or Modify Request Item IE in TS 38.413 [11] Section 9.3.4.3 |
| 31005 | >QoS Flow Indicator | ELEMENT | TRUE | QoS Flow Identifier IE in TS 38.413 [11] Section 9.3.1.51 |  |
| 31006 | >>QoS flow | STRUCTURE |  | 8.1.1.6 |  |
| 31012 | List of QoS Flows failed to be added or modified | LIST |  |  | QoS Flow Failed To Add or Modify List IE in TS 38.413 [11] Section 9.3.4.2 |
| 31013 | >QoS flow item failed to be added or modified | STRUCTURE |  |  | QoS Flow Item IE in TS 38.413 [11] Section 9.3.1.13 |
| 31014 | >>QoS Flow Identifier | ELEMENT | TRUE | QoS Flow Identifier IE in TS 38.413 [11] Section 9.3.1.51 |  |
| 31015 | >>QoS flow | STRUCTURE |  | 8.1.1.6 |  |
| 31016 | >>Cause | STRUCTURE |  |  | Cause IE in TS 38.413 [11] Section 9.3.1.2 |
| 31017 | >>>CHOICE Cause group | STRUCTURE |  |  | Cause Group IE in TS 38.413 [11] Section 9.3.1.2 |
| 31018 | >>>>Radio Network Layer | STRUCTURE |  |  | Radio Network Layer IE in TS 38.413 [11] Section 9.3.1.2 |
| 31019 | >>>>>Radio Network Layer Cause | ELEMENT | FALSE | Radio Network Layer Cause IE in TS 38.413 [11] Section 9.3.1.2 |  |
| 31020 | >>>>Transport Layer | STRUCTURE |  |  | Transport Layer IE in TS 38.413 [11] Section 9.3.1.2 |
| 31021 | >>>>>Transport Layer Cause | ELEMENT | FALSE | Transport Layer Cause IE in TS 37.483 [21] Section 9.3.1.2 |  |
| 31022 | >>>>NAS | STRUCTURE |  |  | NAS IE in TS 38.413 [11] Section 9.3.1.2 |
| 31023 | >>>>>NAS Cause | ELEMENT | FALSE | NAS Cause IE in TS 38.413 [11] Section 9.3.1.2 |  |
| 31024 | >>>>Protocol | STRUCTURE |  |  | Protocol IE in TS 38.413 [11] Section 9.3.1.2 |
| 31025 | >>>>>Protocol Cause | ELEMENT | FALSE | Protocol Cause IE in TS 38.413 [11] Section 9.3.1.2 |  |
| 31026 | >>>>Miscellaneous | STRUCTURE |  |  | Misc IE in TS 38.413 [11] Section 9.3.1.2 |
| 31027 | >>>>>Miscellaneous Cause | ELEMENT | FALSE | Miscellaneous Cause IE in TS 38.413 [11] Section 9.3.1.2 |  |
| 31031 | List of QoS Flows to be released | LIST |  |  | QoS Flow To Release List IE in TS 38.413 [11] Section 9.3.4.2 |
| 31032 | >QoS flow item to be released | STRUCTURE |  |  | QoS Flow Item IE in TS 38.413 [11] Section 9.3.1.13 |
| 31033 | >>QoS Flow Identifier | ELEMENT | TRUE | QoS Flow Identifier IE in TS 38.413 [11] Section 9.3.1.51 |  |
| 31034 | >>QoS flow | STRUCTURE |  | 8.1.1.6 |  |
| 31035 | >>Cause | STRUCTURE |  |  | Cause IE in TS 38.413 [11] Section 9.3.1.2 |
| 31036 | >>>CHOICE Cause group | STRUCTURE |  |  | Cause Group IE in TS 38.413 [11] Section 9.3.1.2 |
| 31037 | >>>>Radio Network Layer | STRUCTURE |  |  | Radio Network Layer IE in TS 38.413 [11] Section 9.3.1.2 |
| 31038 | >>>>>Radio Network Layer Cause | ELEMENT |  | Radio Network Layer Cause IE in TS 38.413 [11] Section 9.3.1.2 |  |
| 31039 | >>>>Transport Layer | STRUCTURE |  |  | Transport Layer IE in TS 38.413 [11] Section 9.3.1.2 |
| 31040 | >>>>>Transport Layer Cause | ELEMENT | FALSE | Transport Layer Cause IE in TS 37.483 [21] Section 9.3.1.2 |  |
| 31041 | >>>>NAS | STRUCTURE |  |  | NAS IE in TS 38.413 [11] Section 9.3.1.2 |
| 31042 | >>>>>NAS Cause | ELEMENT | FALSE | NAS Cause IE in TS 38.413 [11] Section 9.3.1.2 |  |
| 31043 | >>>>Protocol | STRUCTURE |  |  | Protocol IE in TS 38.413 [11] Section 9.3.1.2 |
| 31044 | >>>>>Protocol Cause | ELEMENT | FALSE | Protocol Cause IE in TS 38.413 [11] Section 9.3.1.2 |  |
| 31045 | >>>>Miscellaneous | STRUCTURE |  |  | Misc IE in TS 38.413 [11] Section 9.3.1.2 |
| 31046 | >>>>>Miscellaneous Cause | ELEMENT | FALSE | Miscellaneous Cause IE in TS 38.413 [11] Section 9.3.1.2 |  |
| 31051 | MR-DC Usage Information | STRUCTURE |  |  | MR-DC Usage Information IE in TS 37.483 [21] Section 9.3.1.63 |
| 31052 | >Secondary RAT Type | ELEMENT | FALSE | Secondary RAT Type IE in TS 37.483 [21] Section 9.3.1.63 |  |
| 31053 | >PDU Session Timed Report List | LIST |  |  | Data Usage Report List IE in TS 37.483 [21] Section 9.3.1.64 |
| 31054 | >>MR-DC Data Usage Report Item | STRUCTURE |  |  | Data Report Usage Item IE in TS 37.483 [21] Section 9.3.1.64 |
| 31055 | >>>Start Timestamp | ELEMENT | FALSE | Start timestamp IE in TS 37.483 [21] Section 9.3.1.64 |  |
| 31075 | >>>End Timestamp | ELEMENT | FALSE | End timestamp IE in TS 37.483 [21] Section 9.3.1.64 |  |
| 31056 | >>>Usage Count UL | ELEMENT | FALSE | Usage count DL IE in TS 37.483 [21] Section 9.3.1.64 |  |
| 31057 | >>>Usage Count DL | ELEMENT | FALSE | Usage count DL IE in TS 37.483 [21] Section 9.3.1.64 |  |
| 31061 | MR-DC Usage for QoS flows | LIST |  |  | Data Usage Per QoS Flow List IE in TS 37.483 [21] Section 9.3.1.63 |
| 31062 | >QoS Flow Item | STRUCTURE |  |  | Data Usage Per QoS Flow Item IE in TS 37.483 [21] Section 9.3.1.63 |
| 31063 | >>QoS flow Indicator | ELEMENT | TRUE | QoS Flow Indicator IE in TS 37.483 [21] Section 9.3.1.24 |  |
| 31064 | >>Secondary RAT Type | ELEMENT | FALSE | Secondary RAT Type IE in TS 37.483 [21] Sec 9.3.1.63 |  |
| 31065 | >>QoS Flow Timed Report List | LIST |  |  | Data Usage Report List IE in TS 37.483 [21] Section 9.3.1.64 |
| 31066 | >>>MR-DC Data Usage Report Item | STRUCTURE |  |  | Data Report Usage Item IE in TS 37.483 [21] Section 9.3.1.64 |
| 31067 | >>>>Start Timestamp | ELEMENT | FALSE | Start timestamp IE in TS 37.483 [21] Section 9.3.1.64 |  |
| 31068 | >>>>End timestamp | ELEMENT | FALSE | End timestamp IE in TS 37.483 [21] Section 9.3.1.64 |  |
| 31069 | >>>>Usage Count UL | ELEMENT | FALSE | Usage count DL IE in TS 37.483 [21] Section 9.3.1.64 |  |
| 31070 | >>>>Usage Count DL | ELEMENT | FALSE | Usage count DL IE in TS 37.483 [21] Section 9.3.1.64 |  |
| 31081 | Primary Cell ID | STRUCTURE |  |  | This is for the primary serving cell of the UE. The structuring of this parameter is based on Target Cell Global ID IE in TS 38.423 [15] clause 9.2.3.25 |
| 31082 | >CHOICE Primary Cell | STRUCTURE |  |  | The primary cell could either be an NR primary cell or an LTE primary cell. The structuring is based on Target Cell IE in TS 38.423 [15] clause 9.2.3.25 |
| 31083 | >>NR SpCell | STRUCTURE |  | 8.1.1.1 | NR Cell IE in TS 38.423 [15] clause 9.2.3.25 |
| 31084 | >>E-UTRA PCell | STRUCTURE |  | 8.1.1.2 | E-UTRA Cell IE in TS 38.423 [15] clause 9.2.3.25 |

##### 8.1.2.6.3	PDU Session Resource Release

| RAN Parameter ID | RAN Parameter | RAN Parameter Value Type | Key Flag | RAN Parameter Definition |  | Semantics Description |
| --- | --- | --- | --- | --- | --- | --- |
| 32001 | PDU Session ID | ELEMENT | TRUE | PDU Session ID IE in TS 38.413 [11] Section 9.3.1.50 |  |  |
| 32002 | PDU Session | STRUCTURE |  | 8.1.1.16 |  |  |
| 32011 | Cause | STRUCTURE |  |  |  | Cause IE in TS 38.413 [11] Section 9.3.1.2 |
| 32012 | >CHOICE Cause group | STRUCTURE |  |  |  | Cause Group IE in TS 38.413 [11] Section 9.3.1.2 |
| 32013 | >>Radio Network Layer | STRUCTURE |  |  |  | Radio Network Layer IE in TS 38.413 [11] Section 9.3.1.2 |
| 32014 | >>>Radio Network Layer Cause | ELEMENT | FALSE | Radio Network Layer Cause IE in TS 38.413 [11] Section 9.3.1.2 |  |  |
| 32015 | >>Transport Layer | STRUCTURE |  |  |  | Transport Layer IE in TS 38.413 [11] Section 9.3.1.2 |
| 32016 | >>>Transport Layer Cause | ELEMENT | FALSE | Transport Layer Cause IE in TS 37.483 [21] Section 9.3.1.2 |  |  |
| 32017 | >>NAS | STRUCTURE |  |  |  | NAS IE in TS 38.413 [11] Section 9.3.1.2 |
| 32018 | >>>NAS Cause | ELEMENT | FALSE | NAS Cause IE in TS 38.413 [11] Section 9.3.1.2 |  |  |
| 32019 | >>Protocol | STRUCTURE |  |  |  | Protocol IE in TS 38.413 [11] Section 9.3.1.2 |
| 32020 | >>>Protocol Cause | ELEMENT | FALSE | Protocol Cause IE in TS 38.413 [11] Section 9.3.1.2 |  |  |
| 32021 | >>Miscellaneous | STRUCTURE |  |  |  | Misc IE in TS 38.413 [11] Section 9.3.1.2 |
| 32022 | >>>Miscellaneous Cause | ELEMENT | FALSE | Miscellaneous Cause IE in TS 38.413 [11] Section 9.3.1.2 |  |  |
| 32031 | MR-DC Usage Information | STRUCTURE |  |  |  | MR-DC Usage Information IE in TS 37.483 [21] Section 9.3.1.63 |
| 32032 | >Secondary RAT Type | ELEMENT | FALSE | Secondary RAT Type IE in TS 37.483 [21] Section 9.3.1.63 |  |  |
| 32033 | >PDU Session Timed Report List | LIST |  |  |  | Data Usage Report List IE in TS 37.483 [21] Section 9.3.1.64 |
| 32034 | >>MR-DC Data Usage Report Item | STRUCTURE |  |  |  | Data Report Usage Item IE in TS 37.483 [21] Section 9.3.1.64 |
| 32035 | >>>Start Timestamp | ELEMENT | FALSE | Start timestamp IE in TS 37.483 [21] Section 9.3.1.64 |  |  |
| 32036 | >>>End Timestamp | ELEMENT | FALSE | End timestamp IE in TS 37.483 [21] Section 9.3.1.64 |  |  |
| 32037 | >>>Usage Count UL | ELEMENT | FALSE | Usage count DL IE in TS 37.483 [21] Section 9.3.1.64 |  |  |
| 32038 | >>>Usage Count DL | ELEMENT | FALSE | Usage count DL IE in TS 37.483 [21] Section 9.3.1.64 |  |  |
| 32041 | MR-DC Usage for QoS flows | LIST |  |  | Data Usage Per QoS Flow List IE in TS 37.483 [21] Section 9.3.1.63 |  |
| 32042 | >QoS Flow Item | STRUCTURE |  |  | Data Usage Per QoS Flow Item IE in TS 37.483 [21] Section 9.3.1.63 |  |
| 32043 | >>QoS flow Indicator | ELEMENT | TRUE | QoS Flow Indicator IE in TS 37.483 [21] Sec 9.3.1.24 |  |  |
| 32054 | >>QoS flow | STRUCTURE |  | 8.1.1.6 |  |  |
| 32044 | >>Secondary RAT Type | ELEMENT | FALSE | Secondary RAT Type IE in TS 37.483 [21] Section 9.3.1.63 |  |  |
| 32045 | >>QoS Flow Timed Report List | LIST |  |  | Data Usage Report List IE in TS 37.483 [21] Section 9.3.1.64 |  |
| 32046 | >>>MR-DC Data Usage Report Item | STRUCTURE |  |  | Data Report Usage Item IE in TS 37.483 [21] Section 9.3.1.64 |  |
| 32047 | >>>>Start Timestamp | ELEMENT | FALSE | Start timestamp IE in TS 37.483 [21] Section 9.3.1.64 |  |  |
| 32048 | >>>>End timestamp | ELEMENT | FALSE | End timestamp IE in TS 37.483 [21] Section 9.3.1.64 |  |  |
| 32049 | >>>>Usage Count UL | ELEMENT | FALSE | Usage count DL IE in TS 37.483 [21] Section 9.3.1.64 |  |  |
| 32050 | >>>>Usage Count DL | ELEMENT | FALSE | Usage count DL IE in TS 37.483 [21] Section 9.3.1.64 |  |  |
| 32061 | Primary Cell ID | STRUCTURE |  |  | This is for the primary serving cell of the UE. The structuring of this parameter is based on Target Cell Global ID IE in TS 38.423 [15] clause 9.2.3.25 |  |
| 32062 | >CHOICE Primary Cell | STRUCTURE |  |  | The primary cell could either be an NR primary cell or an LTE primary cell. The structuring is based on Target Cell IE in TS 38.423 [15] clause 9.2.3.25 |  |
| 32063 | >>NR SpCell | STRUCTURE |  | 8.1.1.1 | NR Cell IE in TS 38.423 [15] clause 9.2.3.25 |  |
| 32064 | >>E-UTRA PCell | STRUCTURE |  | 8.1.1.2 | E-UTRA Cell IE in TS 38.423 [15] clause 9.2.3.25 |  |

## 8.4	RAN Parameters for Control Actions

    - 8.4.1	Approach

The approach for RAN parameters associated with Control service is provided in Section 8.0.

### 8.4.2	Radio Bearer Control

#### 8.4.2.1	DRB QoS Configuration

Upon receiving the RIC Control Request message, the E2 node shall invoke procedures related to DRB QoS Configuration, such as Bearer Context Management, UE Context Management, RRC Message Transfer, etc. and include the IEs corresponding to one or more of parameters described below in the related interface messages. If the DRB ID is missing in the RIC Control Request message, the E2 node will send a RIC Control Failure.

| RAN Parameter ID | RAN Parameter | RAN Parameter Value Type | Key Flag | RAN Parameter Definition | Semantics Description |
| --- | --- | --- | --- | --- | --- |
| 1 | DRB ID | ELEMENT | TRUE | DRB ID IE in TS 37.483 [21] Section 9.3.1.16 |  |
| 2 | 5QI | ELEMENT | FALSE | 5QI IE in TS 37.483 [21] Section 9.3.1.27 or TS 37.483 [21] Section 9.3.1.28 |  |
| 3 | Packet Delay Budget | ELEMENT | FALSE | Packet Delay Budget IE in TS 37.483 [21] Section 9.3.1.47 |  |
| 4 | Packet Error Rate | STRUCTURE |  |  | Packet Error Rate IE in TS 37.483 [21] Section 9.3.1.48 |
| 5 | >Scalar factor | ELEMENT | FALSE | Scalar IE in TS 37.483 [21] Section 9.3.1.48 |  |
| 6 | >Exponent factor | ELEMENT | FALSE | Exponent IE in TS 37.483 [21] Section 9.3.1.48 |  |
| 7 | NG-RAN DRB Allocation and Retention Priority | STRUCTURE |  |  | NG-RAN Allocation and Retention Priority IE in TS 37.483 [21] Section 9.3.1.29 |
| 8 | >Priority Level | ELEMENT | FALSE | Priority Level IE in TS 37.483 [21] Section 9.3.1.29 |  |
| 9 | >Pre-emption Capability | ELEMENT | FALSE | Pre-emption Capability IE in TS 37.483 [21] Section 9.3.1.29 |  |
| 10 | >Pre-emption Vulnerability | ELEMENT | FALSE | Pre-emption Vulnerability IE in TS 37.483 [21] Section 9.3.1.29 |  |
| 11 | Priority Level of the mapped QoS flows | ELEMENT | FALSE | Priority Level IE in TS 37.483 [21] Section 9.3.1.51 |  |
| 12 | QoS parameters for GBR flows in NG-RAN Bearer | STRUCTURE |  |  | GBR QoS Flow Information IE in TS 37.483 [21] Section 9.3.1.30 |
| 13 | >Maximum Flow Bit Rate Downlink | ELEMENT | FALSE | Bit Rate IE in TS 37.483 [21] Section 9.3.1.30 |  |
| 14 | >Guaranteed Flow Bit Rate Downlink | ELEMENT | FALSE | Bit Rate IE in TS 37.483 [21] Section 9.3.1.30 |  |
| 15 | >Maximum Packet Loss Rate Downlink | ELEMENT | FALSE | Packet Loss Rate IE in TS 37.483 [21] Section 9.3.1.30 |  |
| 16 | >Maximum Flow Bit Rate Uplink | ELEMENT | FALSE | Bit Rate IE in TS 37.483 [21] Section 9.3.1.30 |  |
| 17 | >Guaranteed Flow Bit Rate Uplink | ELEMENT | FALSE | Bit Rate IE in TS 37.483 [21] Section 9.3.1.30 |  |
| 18 | >Maximum Packet Loss Rate Uplink | ELEMENT | FALSE | Packet Loss Rate IE in TS 37.483 [21] Section 9.3.1.30 |  |
| 19 | QoS Monitoring Enable Request | ELEMENT | FALSE | QoS Monitoring Request IE in TS 37.483 [21] Section 9.3.1.26 |  |
| 20 | QoS Monitoring Reporting Frequency | ELEMENT | FALSE | QoS Monitoring Reporting Frequency IE in TS 37.483 [21] Section 9.3.1.26 |  |
| 21 | QoS Monitoring Disabled | ELEMENT | FALSE | QoS Monitoring Disabled IE in TS 37.483 [21] Section 9.3.1.26 |  |
| 22 | Reflective QoS Mapping | ELEMENT | FALSE | RDI IE in TS 37.483 [21] Section 9.3.1.26 |  |

#### 8.4.2.2	QoS flow mapping configuration

Upon receiving the RIC Control Request message, the E2 node shall invoke procedures related to QoS flow mapping Configuration, such as Bearer Context Management, UE Context Management, RRC Message Transfer, etc. and include the IEs corresponding to one or more of parameters described below in the related interface messages. If the DRB ID is missing in the RIC Control Request message, the E2 node will send a RIC Control Failure.

| RAN Parameter ID | RAN Parameter | RAN Parameter Value Type | Key Flag | RAN Parameter Definition | Semantics Description |
| --- | --- | --- | --- | --- | --- |
| 1 | DRB ID | ELEMENT | TRUE | DRB ID IE in TS 37.483 [21] Section 9.3.1.16 |  |
| 2 | List of QoS Flows to be modified in DRB | LIST |  |  | Flow Mapping Information IE in TS 37.483 [21] Section 9.3.1.26 |
| 3 | >QoS Flow Item | STRUCTURE |  |  | QoS Flow Item IE in TS 37.483 [21] Section 9.3.1.12 |
| 4 | >>QoS Flow Identifier | ELEMENT | TRUE | QoS Flow Identifier IE in TS 37.483 [21] Section 9.3.1.24 |  |
| 5 | >>QoS Flow Mapping Indication | ELEMENT | FALSE | QoS Flow Mapping Indication IE in TS 37.483 [21] Section 9.3.1.60 |  |

#### 8.4.2.3	Logical channel configuration

Upon receiving the RIC Control Request message, the E2 node shall invoke procedures related to Logical channel Configuration, such as RRC Message Transfer, etc. and include the IEs corresponding to one or more of parameters described below in the related RRC messages. If the DRB ID is missing in the RIC Control Request message, the E2 node will send a RIC Control Failure.

| RAN Parameter ID | RAN Parameter | RAN Parameter Value Type | Key Flag | RAN Parameter Definition | Semantics Description |
| --- | --- | --- | --- | --- | --- |
| 1 | DRB ID | ELEMENT | TRUE | DRB-Identity IE in TS 38.331 [22] Section 6 |  |
| 2 | Logical Channel ID | ELEMENT | FALSE | LogicalChannelIdentity IE in TS 38.331 [22] Section 6 |  |
| 3 | List of cell groups to be added | LIST |  |  | Cell Group To Add IE in TS 37.483 [21] Section 9.3.3.11 |
| 4 | >Cell group item | STRUCTURE |  |  | Cell Group Item IE in TS 37.483 [21] Section 9.3.1.11 |
| 5 | >>Cell Group ID | ELEMENT | TRUE | Cell Group ID IE in TS 37.483 [21] Section 9.3.1.11 |  |
| 6 | >>UL Configuration | ELEMENT | FALSE | UL Configuration IE in TS 37.483 [21] Section 9.3.1.33 |  |
| 7 | List of cell groups to be modified | LIST |  |  | Cell Group To Modify IE in TS 37.483 [21] Section 9.3.1.11 |
| 8 | >Cell group item | STRUCTURE |  |  | Cell Group Item IE in TS 37.483 [21] Section 9.3.1.11 |
| 9 | >>Cell Group ID | ELEMENT | TRUE | Cell Group ID IE in TS 37.483 [21] Section 9.3.1.11 |  |
| 10 | >>UL Configuration | ELEMENT | FALSE | UL Configuration IE in TS 37.483 [21] Section 9.3.1.11 |  |
| 11 | List of cell groups to be removed | LIST |  |  | Cell Group To Remove IE in TS 37.483 [21] Section 9.3.1.11 |
| 12 | >Cell group item | STRUCTURE |  |  | Cell Group Item IE in TS 37.483 [21] Section 9.3.1.11 |
| 13 | >>Cell Group ID | ELEMENT | FALSE | Cell Group ID IE in TS 37.483 [21] Section 9.3.1.11 |  |
| 14 | Uplink Specific Parameters | STRUCTURE |  |  | ul-SpecificParameters IE in TS 38.331 [22] Section 6 |
| 15 | >Priority | ELEMENT | FALSE | priority IE in TS 38.331 [22] Section 6 |  |
| 16 | >Prioritized Bit rate | ELEMENT | FALSE | prioritisedBitRate IE in TS 38.331 [22] Section 6 |  |
| 17 | >Bucket Size Duration | ELEMENT | FALSE | bucketSizeDuration IE in TS 38.331 [22] Section 6 |  |
| 18 | >List of Allowed Serving Cells | LIST |  |  | allowedServingCells IE in TS 38.331 [22] Section 6 |
| 19 | >>Allowed Serving Cell Item | STRUCTURE |  |  |  |
| 20 | >>>Serving Cell Index | ELEMENT | FALSE | ServCellIndex IE in TS 38.331 [22] Section 6 |  |
| 21 | >List of Allowed SCS Configuration | LIST |  |  | allowedSCS-List IE in TS 38.331 [22] Section 6 |
| 22 | >>Allowed SCS Configuration Item | STRUCTURE |  |  |  |
| 23 | >>Subcarrier Spacing Configuration | ELEMENT | FALSE | SubCarrierSpacing IE in TS 38.331 [22] Section 6 |  |
| 24 | >Maximum PUSCH Duration | ELEMENT | FALSE | maxPUSCH-Duration IE in TS 38.331 [22] Sec 6 |  |
| 25 | >Configured Grant Type 1 Allowed | ELEMENT | FALSE | configuredGrantTypeAllowed IE in TS 38.331 [22] Section 6 |  |
| 26 | >Logical Channel SR – Mask | ELEMENT | FALSE | logicalChannelSR-Mask IE in TS 38.331 [22] Section 6 |  |
| 27 | >Logical Channel SR – Delay Timer Applied | ELEMENT | FALSE | logicalChannelSR-DelayTimerApplied IE in TS 38.331 [22] Sec 6 |  |
| 28 | >Bit Rate Query Prohibit Timer | ELEMENT | FALSE | bitRateQueryProhibitTimer IE in TS 38.331 [22] Section 6 |  |
| 29 | >RLC re-establishment | ELEMENT | FALSE | reestablishRLC IE in TS 38.331 [22] |  |
| 30 | >Logical Channel Group | ELEMENT | FALSE | logicalChannelGroup IE in TS 38.331 [22] |  |

#### 8.4.2.4	Radio Bearer Admission Control

Upon receiving the RIC Control Request message, the E2 node shall invoke procedures related to Radio Bearer Admission Control, such as Bearer Context Management, UE Context Management, RRC Message Transfer, etc. and include the IEs corresponding to one or more of parameters described below in the related interface messages. If the DRB ID is missing in the RIC Control Request message, the E2 node will send a RIC Control Failure.

| RAN Parameter ID | RAN Parameter | RAN Parameter Value Type | Key Flag | RAN Parameter Definition | Semantics description |
| --- | --- | --- | --- | --- | --- |
| 1 | DRB Identity | ELEMENT | TRUE | DRB ID IE in TS 37.483 [21] Section 9.3.1.16 |  |
| 2 | List of cell groups to be added | LIST |  |  | Cell Group To Add IE in TS 37.483 [21] Section 9.3.3.11 |
| 3 | >Cell group item | STRUCTURE |  |  | Cell Group Item IE in TS 37.483 [21] Section 9.3.1.11 |
| 4 | >>Cell Group ID | ELEMENT | TRUE | Cell Group ID IE in TS 37.483 [21] Section 9.3.1.11 |  |
| 5 | SDAP Configuration | STRUCTURE |  |  | SDAP Configuration IE in TS 37.483 [21] Section 9.3.1.39 |
| 6 | >PDU Session ID | ELEMENT | FALSE | PDU-SessionID IE in TS 38.331 [22] Sec 6 |  |
| 7 | >Default DRB | ELEMENT | FALSE | Default DRB IE in TS 37.483 [21] Section 9.3.1.39 |  |
| 8 | PDCP Configuration | STRUCTURE |  |  | PDCP Configuration IE in TS 37.483 [21] Section 9.3.1.38 |
| 9 | >RLC mode | ELEMENT | FALSE | RLC mode IE in TS 37.483 [21] Section 9.3.1.38 |  |
| 10 | >PDCP Duplication | ELEMENT | FALSE | PDCP Duplication IE in TS 37.483 [21] Section 9.3.1.38 |  |
| 11 | >UL Data Split Threshold | ELEMENT | FALSE | UL Data Split Threshold IE in TS 37.483 [21] Section 9.3.1.43 |  |
| 12 | >PDCP Re-establishment | ELEMENT | FALSE | PDCP Re-establishment IE in TS 37.483 [21] Section 9.3.1.38 |  |
| 13 | >PDCP Data Recovery | ELEMENT | FALSE | PDCP Data Recovery IE in TS 37.483 [21] Section 9.3.1.38 |  |
| 14 | >Out-of-Order Delivery | ELEMENT | FALSE | Out of Order Delivery IE in TS 37.483 [21] Section 9.3.1.38 |  |
| 15 | >PDCP Status Report Indication | ELEMENT | FALSE | PDCP Status Report Indication IE in TS 37.483 [21] Section 9.3.1.38 |  |
| 16 | >Number of PDCP duplication | ELEMENT | FALSE | Additional PDCP duplication Information IE in TS 37.483 [21] Section 9.3.1.38 |  |
| 17 | >UL More than one RLC | STRUCTURE |  |  | moreThanOneRLC IE in TS 38.331 [22] Section 6 |
| 18 | >>Primary Path | STRUCTURE |  |  | primaryPath IE in TS 38.331 [22] Section 6 |
| 19 | >>>Cell Group ID | ELEMENT | TRUE | cellGroup IE in TS 38.331 [22] Section 6 |  |
| 20 | >>>Logical Channel ID | ELEMENT | FALSE | logicalChannel IE in TS 38.331 [22] Sec 6 |  |
| 21 | >UL More than two RLC | STRUCTURE |  |  | moreThanTwoRLC-DRB-r16 IE in TS 38.331 [22] Section 6 |
| 22 | >>Split Secondary Path | ELEMENT | FALSE | splitSecondaryPath IE in TS 38.331 [22] Sec 6 |  |
| 23 | >>Duplication State | ELEMENT | FALSE | duplicationState IE in TS 38.331 [22] Sec 6 |  |
| 17 | >DL More than one RLC | STRUCTURE |  |  | moreThanOneRLC IE in TS 38.331 [22] Section 6 |
| 18 | >>Primary Path | STRUCTURE |  |  | primaryPath IE in TS 38.331 [22] Section 6 |
| 19 | >>>Cell Group ID | ELEMENT | TRUE | cellGroup IE in TS 38.331 [22] Section 6 |  |
| 20 | >>>Logical Channel ID | ELEMENT | FALSE | logicalChannel IE in TS 38.331 [22] Sec 6 |  |
| 21 | >DL More than two RLC | STRUCTURE |  |  | moreThanTwoRLC-DRB-r16 IE in TS 38.331 [22] Section 6 |
| 22 | >>Split Secondary Path | ELEMENT | FALSE | splitSecondaryPath IE in TS 38.331 [22] Sec 6 |  |
| 23 | >>Duplication State | ELEMENT | FALSE | duplicationState IE in TS 38.331 [22] Sec 6 |  |

#### 8.4.2.5	DRB Termination Control

Upon receiving the RIC Control Request message, the E2 node shall invoke procedures related to DRB Termination change, such as Dual Connectivity Secondary Node Modification (MN/SN initiated), UE Context Management, RRC Message Transfer, etc. and include the IEs corresponding to one or more of parameters described below in the related interface messages. If the DRB ID is missing in the RIC Control Request message, the E2 node will send a RIC Control Failure.

| RAN Parameter ID | RAN Parameter | RAN Parameter Value Type | Key Param | RAN Parameter definition | Semantics Description |
| --- | --- | --- | --- | --- | --- |
| 1 | List of DRBs to be modified to SN-Termination | LIST |  |  | DRBs To Be Modified IE in TS 38.423 [15] Section 9.2.1.11 |
| 2 | >DRB Item to be modified to SN-Termination | STRUCTURE |  |  | DRBs To Be Modified Item IE in TS 38.423 [15] Section 9.2.1.11 |
| 3 | >>DRB ID | ELEMENT | TRUE | DRB ID IE in TS 38.423 [15] Section 9.2.3.33 |  |
| 4 | >>Logical Channel ID | ELEMENT | FALSE | LCID IE in TS 38.423 [15] Section 9.2.3.70 |  |
| 5 | >>RLC Status | ELEMENT | FALSE | RLC Status IE in TS 38.423 [15] Section 9.2.3.80 |  |
| 6 | >>List of QoS flows to be modified to SN-Termination | LIST |  |  | QoS Flows List IE in TS 38.423 [15] Section 9.2.1.15 |
| 7 | >>>QoS flow item | STRUCTURE |  |  | QoS Flow Item IE in TS 38.423 [15] Section 9.2.1.15 |
| 8 | >>>>QoS flow ID | ELEMENT | TRUE | QoS Flow Identifier IE in TS 38.423 [15] Section 9.2.1.15 |  |
| 9 | >>>>QoS Flow Mapping Indication | ELEMENT | FALSE | QoS Flow Mapping Indication IE in TS 38.423 [15] Section 9.2.1.15 |  |
| 10 | List of DRBs to be modified to MN-Termination | LIST |  |  | DRBs To Be Modified IE in TS 38.423 [15] Section 9.2.1.9 |
| 11 | >DRB Item to be modified to MN-Termination | STRUCTURE |  |  | DRBs To Be Modified Item IE in TS 38.423 [15] Section 9.2.1.9 |
| 12 | >>DRB ID | ELEMENT | TRUE | DRB ID IE in TS 38.423 [15] Section 9.2.3.33 |  |
| 13 | >>Logical Channel ID | ELEMENT | FALSE | LCID IE in TS 38.423 [15] Section 9.2.3.70 |  |
| 14 | >>RLC Status | ELEMENT | FALSE | RLC Status IE in TS 38.423 [15] Section 9.2.3.80 |  |
| 15 | >>List of QoS flows to be modified to SN-Termination | LIST |  |  | QoS Flows List IE in TS 38.423 [15] Section 9.2.1.15 |
| 16 | >>>QoS flow item | STRUCTURE |  |  | QoS Flow Item IE in TS 38.423 [15] Section 9.2.1.15 |
| 17 | >>>>QoS flow ID | ELEMENT | TRUE | QoS Flow Identifier IE in TS 38.423 [15] Section 9.2.1.15 |  |
| 18 | >>>>QoS Flow Mapping Indication | ELEMENT | FALSE | QoS Flow Mapping Indication IE in TS 38.423 [15] Section 9.2.1.15 |  |

#### 8.4.2.6	DRB Split Ratio Control

Upon receiving the RIC Control Request message, in the presence of Downlink PDCP Data Split IE, the E2 node shall split the downlink PDCP traffic between the Master Node and Secondary Node over the X2/Xn interface based on the recommended ratio. In the presence of Uplink PDCP Data Split Threshold IE, the E2 node shall invoke procedures related to DRB Split Ratio Control, such as Bearer Context Management, UE Context Management, RRC Message Transfer, etc. and include the IEs corresponding to one or more of parameters described below in the related interface messages. If the DRB ID is missing in the RIC Control Request message, the E2 node will send a RIC Control Failure.

| RAN Parameter ID | RAN Parameter | RAN Parameter Value Type | Key Flag | RAN Parameter Definition | Semantics Description |
| --- | --- | --- | --- | --- | --- |
| 1 | DRB ID | ELEMENT | TRUE | DRB ID IE in TS 37.483 [21] Section 9.3.1.16 |  |
| 2 | Uplink PDCP Data Split Threshold | ELEMENT | FALSE | UL Data Split Threshold IE in TS 37.483 [21] Section 9.3.1.43 |  |
| 3 | Downlink PDCP Data Split | ELEMENT | FALSE | INTEGER (0..100) Defined in Section 9.4.2 | Indicates the percentage of PDCP traffic that the MN has to split with the SN |

#### 8.4.2.7	PDCP Duplication Control

Upon receiving the RIC Control Request message, the E2 node shall invoke procedures related to PDCP Duplication Control, such as Bearer Context Management, UE Context Management, RRC Message Transfer, etc. and include the IEs corresponding to one or more of parameters described below in the related interface messages. If the DRB ID is missing in the RIC Control Request message, the E2 node will send a RIC Control Failure.

| RAN Parameter ID | RAN Parameter | RAN Parameter Value Type | Key Flag | RAN Parameter Definition | Semantics Description |
| --- | --- | --- | --- | --- | --- |
| 1 | DRB ID | ELEMENT | TRUE | DRB ID IE in TS 37.483 [21] Section 9.3.1.16 |  |
| 2 | PDCP Duplication | ELEMENT | FALSE | PDCP Duplication IE in TS 37.483 [21] Section 9.3.1.38 |  |
| 3 | UL More Than One RLC | STRUCTURE |  |  | moreThanOneRLC IE in TS 38.331 [22] |
| 4 | >Primary Path | STRUCTURE |  |  | primaryPath IE in TS 38.331 [22] Section 6 |
| 5 | >>Cell Group ID | ELEMENT | TRUE | cellGroup IE in TS 38.331 [22] Section 6 |  |
| 6 | >>Logical Channel ID | ELEMENT | FALSE | logicalChannel IE in TS 38.331 [22] Sec 6 |  |
| 7 | UL More Than Two RLC | STRUCTURE |  |  | moreThanTwoRLC-DRB-r16 IE in TS 38.331 [22] Section 6 |
| 8 | >Split Secondary Path | ELEMENT | FALSE | splitSecondaryPath IE in TS 38.331 [22] Sec 6 |  |
| 9 | >Duplication State | ELEMENT | FALSE | duplicationState IE in TS 38.331 [22] Sec 6 |  |
| 10 | PDCP Duplication Activation | ELEMENT | FALSE | Duplication Activation IE in TS 37.483 [21] Section 9.3.1.38 |  |
| 11 | Number of PDCP duplication | ELEMENT | FALSE | Additional PDCP duplication Information IE in TS 37.483 [21] Section 9.3.1.38 |  |

### 8.4.4	Connected Mode Mobility Control

#### 8.4.4.1	Handover Control

Upon receiving the RIC Control Request message, in the case of Xn/X2 or NG or inter-RAT handovers, the E2 node shall invoke procedures, such as Handover Preparation related to UE Mobility Management, Bearer Context Modification, UE Context Modification, RRC Message Transfer, etc. In the case of intra-gNB or F1 handover, the E2 node shall invoke procedures, such as UE Context Modification, RRC Message Transfer, etc. The E2 node includes the IEs corresponding to one or more of parameters described below in the related interface messages. If the Target Primary Cell ID IE is missing in the RIC Control Request message, the E2 node will send a RIC Control Failure message.

| RAN Parameter ID | RAN Parameter | RAN Parameter Value Type | Key Flag | RAN Parameter Definition | Semantics Description |
| --- | --- | --- | --- | --- | --- |
| 1 | Target Primary Cell ID | STRUCTURE |  |  | Target Cell Global ID IE in TS 38.423 [15] Section 9.2.3.25 |
| 2 | >CHOICE Target Cell | STRUCTURE |  |  | Target Cell IE in TS 38.423 [15] Section 9.2.3.25 |
| 3 | >>NR Cell | STRUCTURE |  |  | NR IE in TS 38.423 [15] Section 9.2.3.25 |
| 4 | >>>NR CGI | ELEMENT | FALSE | NR CGI IE in TS 38.423 [15] Section 9.2.2.7 |  |
| 5 | >>E-UTRA Cell | STRUCTURE |  |  | E-UTRA IE in TS 38.423 [15] Section 9.2.3.25 |
| 6 | >>>E-UTRA CGI | ELEMENT | FALSE | E-UTRA CGI IE in TS 38.423 [15] Section 9.2.2.8 |  |
| 7 | List of PDU sessions for handover | LIST |  |  | PDU Session Resources To Be Setup List IE in TS 38.423 [15] Section 9.2.1.1 |
| 8 | >PDU session Item for handover | STRUCTURE |  |  | PDU Session Resources To Be Setup Item IE in TS 38.423 [15] Section 9.2.1.1 |
| 9 | >>PDU Session ID | ELEMENT | TRUE | PDU Session ID IE in TS 38.423 [15] Section 9.2.3.18 |  |
| 10 | >>List of QoS flows in the PDU session | LIST |  |  | QoS Flows To Be Setup List IE in TS 38.423 [15] Section 9.2.1.1 |
| 11 | >>>QoS flow Item | STRUCTURE |  |  | QoS Flow To Be Setup Item IE in TS 38.423 [15] Section 9.2.1.1 |
| 12 | >>>>QoS Flow Identifier | ELEMENT | TRUE | QoS Flow Identifier IE in TS 38.423 [15] Section 9.2.3.10 |  |
| 13 | List of DRBs for handover | LIST |  |  | DRB to Be Setup List IE in TS 38.473 [19] Section 9.2.2.1 |
| 14 | >DRB item for handover | STRUCTURE |  |  | DRB to Be Setup Item IE in TS 38.473 [19] Section 9.2.2.1 |
| 15 | >>DRB ID | ELEMENT | TRUE | DRB ID IE in TS 38.473 [19] Section 9.3.1.8 |  |
| 16 | >>List of QoS flows in the DRB | LIST |  |  | QoS Flows Information To Be Setup IE in TS 37.483 [21] Section 9.3.3.2 |
| 17 | >>>QoS flow Item | STRUCTURE |  |  | QoS Flow Item IE in TS 37.483 [21] Section 9.3.1.25 |
| 18 | >>>>QoS flow Identifier | ELEMENT | TRUE | QoS Flow Identifier IE in TS 37.483 [21] Section 9.3.1.25 |  |
| 19 | List of Secondary cells to be setup | LIST |  |  | Scell To Be Setup List IE in TS 38.473 [19] Section 9.2.2.1 |
| 20 | >Secondary cell Item to be setup | STRUCTURE |  |  | Scell To Be Setup Item Ies IE in TS 38.473 [19] Section 9.2.2.1 |
| 21 | >>Secondary cell ID | ELEMENT | FALSE | Scell ID IE in TS 38.473 [19] Section 9.2.2.1 |  |

#### 8.4.4.2	Conditional Handover Control

Upon receiving the RIC Control Request message, in the case of Xn/X2 or NG or inter-RAT conditional handovers, the E2 node shall invoke procedures, such as Handover Preparation related to UE Mobility Management, Bearer Context Modification, UE Context Modification, RRC Message Transfer, etc. involving the candidate target cells (note that there is only one Handover Preparation in the case of NG or inter-RAT handovers). In the case of intra-gNB or F1 conditional handover, the E2 node shall invoke procedures, such as UE Context Modification, RRC Message Transfer, etc. The E2 node includes the IEs corresponding to one or more of parameters described below in the related interface messages. If the Target Cell IE is missing in the RIC Control Request message, the E2 node will send a RIC Control Failure message.

| RAN Parameter ID | RAN Parameter | RAN Parameter Value Type | Key Param | RAN Parameter Definition | Semantics Description |
| --- | --- | --- | --- | --- | --- |
| 1 | List of Candidate target primary cells | LIST |  |  | Candidate SpCell List IE in TS 38.473 [19] Section 9.2.2.1 |
| 2 | >Candidate target primary cell item | STRUCTURE |  |  | Candidate SpCell Item IEs IE in TS 38.473 [19] Section 9.2.2.1 |
| 3 | >>CHOICE Target Cell | STRUCTURE |  |  | Target Cell IE in TS 38.423 [15] Section 9.2.3.25 |
| 4 | >>>NR | STRUCTURE |  |  | NR IE in TS 38.423 [15] Section 9.2.3.25 |
| 5 | >>>>NR CGI | ELEMENT | FALSE | NR CGI IE in TS 38.423 [15] Section 9.2.2.7 |  |
| 6 | >>>E-UTRA | STRUCTURE |  |  | E-UTRA IE in TS 38.423 [15] Section 9.2.3.25 |
| 7 | >>>>E-UTRA CGI | ELEMENT | FALSE | E-UTRA CGI IE in TS 38.423 [15] Section 9.2.2.8 |  |
| 8 | >>Estimated Arrival Probability | ELEMENT | FALSE |  |  |
| 9 | List of PDU sessions for handover | LIST |  |  | PDU Session Resources To Be Setup List IE in TS 38.423 [15] Section 9.2.1.1 |
| 10 | >PDU session Item for handover | STRUCTURE |  |  | PDU Session Resources To Be Setup Item IE in TS 38.423 [15] Section 9.2.1.1 |
| 11 | >>PDU Session ID | ELEMENT | TRUE | PDU Session ID IE in TS 38.423 [15] Section 9.2.3.18 |  |
| 12 | >>List of QoS flows in the PDU session | LIST |  |  | QoS Flows To Be Setup List IE in TS 38.423 [15] Section 9.2.1.1 |
| 13 | >>>QoS flow Item | STRUCTURE |  |  | QoS Flow To Be Setup Item IE in TS 38.423 [15] Section 9.2.1.1 |
| 14 | >>>>QoS Flow Identifier | ELEMENT | TRUE | QoS Flow Identifier IE in TS 38.423 [15] Section 9.2.3.10 |  |
| 15 | Candidate target primary cell global ID for NG handover | STRUCTURE |  |  | Target Cell ID IE in TS 38.413 [11] Section 9.3.1.73 |
| 16 | >CHOICE Target Cell | STRUCTURE |  |  | Target Cell IE in TS 38.423 [15] Section 9.2.3.25 |
| 17 | >>NR Cell | STRUCTURE |  |  | NR IE in TS 38.423 [15] Section 9.2.3.25 |
| 18 | >>>NR CGI | ELEMENT | FALSE | NR CGI IE in TS 38.423 [15] Section 9.2.2.7 |  |
| 19 | >>E-UTRA Cell | STRUCTURE |  |  | E-UTRA IE in TS 38.423 [15] Section 9.2.3.25 |
| 20 | >>>E-UTRA CGI | ELEMENT | FALSE | E-UTRA CGI IE in TS 38.423 [15] Section 9.2.2.8 |  |
| 21 | List of DRBs for handover | LIST |  |  | DRB To Be Setup List IE in TS 38.473 [19] Section 9.2.2.1 |
| 22 | >DRB item for handover | STRUCTURE |  |  | DRB To Be Setup Item IE in TS 38.473 [19] Section 9.2.2.1 |
| 23 | >>DRB ID | ELEMENT | TRUE | DRB ID IE in TS 38.473 [19] Section 9.3.1.8 |  |
| 24 | >>List of QoS flows in the DRB | LIST |  |  | Flows Mapped To DRB Item IE in TS 38.473 [19] Section 9.2.2.1 |
| 25 | >>>QoS flow Item | STRUCTURE |  |  |  |
| 26 | >>>>QoS flow Identifier | ELEMENT | TRUE | QoS Flow Identifier IE in TS 38.473 [19] Section 9.3.1.63 |  |
| 27 | List of Secondary cells to be setup | LIST |  |  | SCell To Be Setup List IE in TS 38.473 [19] Section 9.2.2.1 |
| 28 | >Secondary cell Item to be setup | STRUCTURE |  |  | SCell To Be Setup Item IEs IE in TS 38.473 [19] Section 9.2.2.1 |
| 29 | >>Secondary cell ID | ELEMENT | FALSE | SCell ID IE in TS 38.473 [19] Section 9.2.2.1 |  |

#### 8.4.4.3	DAPS Handover Control

Upon receiving the RIC Control Request message, in the case of Xn/X2 DAPS handovers, the E2 node shall invoke procedures, such as Handover Preparation related to UE Mobility Management, Bearer Context Modification, UE Context Modification, RRC Message Transfer, etc. In the case of intra-gNB or F1 DAPS handover, the E2 node shall invoke procedures, such as UE Context Modification, RRC Message Transfer, etc. The E2 node includes the IEs corresponding to one or more of parameters described below in the related interface messages. If the Target Primary Cell ID IE is missing in the RIC Control Request message, the E2 node will send a RIC Control Failure message.

| RAN Parameter ID | RAN Parameter | RAN Parameter Value Type | Key Param | RAN Parameter Definition | Semantics Description |
| --- | --- | --- | --- | --- | --- |
| 1 | Target Primary Cell ID | STRUCTURE |  |  | Target Cell Global ID IE in TS 38.423 [15] Section 9.2.3.25 |
| 2 | >CHOICE Target Cell | STRUCTURE |  |  | Target Cell IE in TS 38.423 [15] Section 9.2.3.25 |
| 3 | >>NR Cell | STRUCTURE |  |  | NR IE in TS 38.423 [15] Section 9.2.3.25 |
| 4 | >>>NR CGI | ELEMENT | FALSE | NR CGI IE in TS 38.423 [15] Section 9.2.2.7 |  |
| 5 | >>E-UTRA Cell | STRUCTURE |  |  | E-UTRA IE in TS 38.423 [15] Section 9.2.3.25 |
| 6 | >>>E-UTRA CGI | ELEMENT | FALSE | E-UTRA CGI IE in TS 38.423 [15] Section 9.2.2.8 |  |
| 7 | List of DRBs subject to DAPS handover | LIST |  |  | Source DRB to QoS Flow Mapping List IE in TS 38.423 [15] Section 9.2.1.17 |
| 8 | >DRB item for handover | STRUCTURE |  |  | DRB to QoS Flow Mapping Item IE in TS 38.423 [15] Section 9.2.1.15 |
| 9 | >>DRB ID | ELEMENT | TRUE | DRB ID IE in TS 38.423 [15] Section 9.2.3.33 |  |
| 10 | >>List of QoS flows in the DRB | LIST |  |  | QoS Flows List IE in TS 38.423 [15] Section 9.2.1.15 |
| 11 | >>>QoS flow Item | STRUCTURE |  |  | QoS Flow Item IE in TS 38.423 [15] Section 9.2.1.15 |
| 12 | >>>>QoS flow Identifier | ELEMENT | TRUE | QoS Flow Identifier IE in TS 38.423 [15] Section 9.2.3.10 |  |
| 13 | >>>>QoS flow mapping indication | ELEMENT | FALSE | QoS Flow Identifier IE in TS 38.423 [15] Section 9.2.3.79 |  |
| 14 | List of DRBs subject to handover without DAPS | LIST |  |  | Source DRB to QoS Flow Mapping List IE in TS 38.423 [15] Section 9.2.1.17 |
| 15 | >DRB item for handover | STRUCTURE |  |  | DRB to QoS Flow Mapping Item IE in TS 38.423 [15] Section 9.2.1.15 |
| 16 | >>DRB ID | ELEMENT | TRUE | DRB ID IE in TS 38.423 [15] Section 9.2.3.33 |  |
| 17 | >>List of QoS flows in the DRB | LIST |  |  | QoS Flows List IE in TS 38.423 [15] Section 9.2.1.15 |
| 18 | >>>QoS flow Item | STRUCTURE |  |  | QoS Flow Item IE in TS 38.423 [15] Section 9.2.1.15 |
| 19 | >>>>QoS flow Identifier | ELEMENT | TRUE | QoS Flow Identifier IE in TS 38.423 [15] Section 9.2.3.10 |  |
| 20 | >>>>QoS flow mapping indication | ELEMENT | FALSE | QoS Flow Identifier IE in TS 38.423 [15] Section 9.2.3.79 |  |
| 21 | List of PDU Sessions subject for handover | LIST |  |  | PDU Session Resources To Setup List IE in TS 38.423 [15] Section 9.2.1.1 |
| 22 | >PDU Session Item | STRUCTURE |  |  | PDU Session Resources To Be Setup Item IE in TS 38.423 [15] Section 9.2.1.1 |
| 23 | >>PDU Session ID | ELEMENT | TRUE | PDU Session ID IE in TS 38.423 [15] Section 9.2.1.1 |  |
| 24 | >>List of DRBs subject to DAPS handover | LIST |  |  | Source DRB to QoS Flow Mapping List IE in TS 38.423 [15] Section 9.2.1.17 |
| 25 | >>>DRB item for handover | STRUCTURE |  |  | DRB to QoS Flow Mapping Item IE in TS 38.423 [15] Section 9.2.1.15 |
| 26 | >>>>DRB ID | ELEMENT | TRUE | DRB ID IE in TS 38.423 [15] Section 9.2.3.33 |  |
| 27 | >>>>List of QoS flows in the DRB | LIST |  |  | QoS Flows List IE in TS 38.423 [15] Section 9.2.1.15 |
| 28 | >>>>>QoS flow Item | STRUCTURE |  |  | QoS Flow Item IE in TS 38.423 [15] Section 9.2.1.15 |
| 29 | >>>>>>QoS flow Identifier | ELEMENT | TRUE | QoS Flow Identifier IE in TS 38.423 [15] Section 9.2.3.10 |  |
| 30 | >>>>>>QoS flow mapping indication | ELEMENT | FALSE | QoS Flow Identifier IE in TS 38.423 [15] Section 9.2.3.79 |  |
| 31 | >>List of DRBs subject to handover without DAPS | LIST |  |  | Source DRB to QoS Flow Mapping List IE in TS 38.423 [15] Section 9.2.1.17 |
| 32 | >>>DRB item for handover | STRUCTURE |  |  | DRB to QoS Flow Mapping Item IE in TS 38.423 [15] Section 9.2.1.15 |
| 33 | >>>>DRB ID | ELEMENT | TRUE | DRB ID IE in TS 38.423 [15] Section 9.2.3.33 |  |
| 34 | >>>>List of QoS flows in the DRB | LIST |  |  | QoS Flows List IE in TS 38.423 [15] Section 9.2.1.15 |
| 35 | >>>>>QoS flow Item | STRUCTURE |  |  | QoS Flow Item IE in TS 38.423 [15] Section 9.2.1.15 |
| 36 | >>>>>>QoS flow Identifier | ELEMENT | TRUE | QoS Flow Identifier IE in TS 38.423 [15] Section 9.2.3.10 |  |
| 37 | >>>>>>QoS flow mapping Indication | ELEMENT | FALSE | QoS Flow Identifier IE in TS 38.423 [15] Section 9.2.3.79 |  |

### 8.4.5	Radio Access Control

#### 8.4.5.1	UE admission control

Upon receiving the RIC Control Request message, the E2 node shall invoke procedures related to UE admission control, such as PDU Session Management, Bearer Context Management, UE Context Management, RRC Message Transfer, etc. and include the IEs corresponding to one or more of parameters described below in the related interface messages.

| RAN Parameter ID | RAN Parameter | RAN Parameter Value Type | Key Flag |  | RAN Parameter Definition | Semantics Description |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Primary Cell ID | STRUCTURE |  |  |  | Target Cell Global ID IE in TS 38.423 [15] clause 9.2.3.25 |
| 2 | >CHOICE Primary Cell | STRUCTURE |  |  |  | Target Cell IE in TS 38.423 [15] clause 9.2.3.25 |
| 3 | >>NR Cell | STRUCTURE |  |  |  | NR IE in TS 38.423 [15] Section 9.2.3.25 |
| 4 | >>>NR CGI | ELEMENT | FALSE |  | NR CGI IE in TS 38.423 [15] Section 9.2.2.7 |  |
| 5 | >>E-UTRA Cell | STRUCTURE |  |  |  | E-UTRA IE in TS 38.423 [15] Section 9.2.3.25 |
| 6 | >>>E-UTRA CGI | ELEMENT | FALSE |  | E-UTRA CGI IE in TS 38.423 [15] Section 9.2.2.8 |  |
| 7 | List of PDU sessions for admission | LIST |  |  |  | PDU Session Resources To Be Setup List IE in TS 38.423 [15] Section 9.2.1.1 |
| 8 | >PDU session Item for admission | STRUCTURE |  |  |  | PDU Session Resources To Be Setup Item IE in TS 38.423 [15] Section 9.2.1.1 |
| 9 | >>PDU Session ID | ELEMENT | TRUE |  | PDU Session ID IE in TS 38.423 [15] Section 9.2.3.18 |  |
| 10 | >>List of DRBs for admission | LIST |  |  |  | DRB to Be Setup List IE in TS 38.473 [19] Section 9.2.2.1 |
| 11 | >>>DRB item for admission | STRUCTURE |  |  |  | DRB to Be Setup Item IE in TS 38.473 [19] Section 9.2.2.1 |
| 12 | >>>>DRB ID | ELEMENT | TRUE |  | DRB ID IE in TS 38.473 [19] Section 9.3.1.8 |  |
| 13 | >>>>List of QoS flows in the DRB | LIST |  |  |  | QoS Flows Information To Be Setup IE in TS 37.483 [21] Section 9.3.3.2 |
| 14 | >>>>>QoS flow Item | STRUCTURE |  |  |  | QoS Flow Item IE in TS 37.483 [21] Section 9.3.1.25 |
| 15 | >>>>>>QoS flow Identifier | ELEMENT | TRUE |  | QoS Flow Identifier IE in TS 37.483 [21] Section 9.3.1.25 |  |
| 16 | >>>>>>QoS flow mapping indication | ELEMENT | TRUE |  | QoS Flow Mapping Indication IE in TS 37.483 [21] Section 9.3.1.60 |  |
| 17 | >>>List of cell groups to be added | LIST |  |  |  | Cell Group To Add IE in TS 37.483 [21] Section 9.3.3.11 |  |
| 18 | >>>>Cell group item | STRUCTURE |  |  |  | Cell Group Item IE in TS 37.483 [21] Section 9.3.1.11 |  |
| 19 | >>>>>Cell Group ID | ELEMENT | TRUE | Cell Group ID IE in TS 37.483 [21] Section 9.3.1.11 |  |  |  |
| 20 | >>>SDAP Configuration | STRUCTURE |  |  |  | SDAP Configuration IE in TS 37.483 [21] Section 9.3.1.39 |  |
| 21 | >>>Default DRB | ELEMENT | FALSE | Default DRB IE in TS 37.483 [21] Section 9.3.1.39 |  |  |  |
| 22 | >>>PDCP Configuration | STRUCTURE |  |  |  | PDCP Configuration IE in TS 37.483 [21] Section 9.3.1.38 |  |
| 23 | >>>RLC mode | ELEMENT | FALSE | RLC mode IE in TS 37.483 [21] Section 9.3.1.38 |  |  |  |
| 24 | >>>PDCP Duplication | ELEMENT | FALSE | PDCP Duplication IE in TS 37.483 [21] Section 9.3.1.38 |  |  |  |
| 25 | >>>UL Data Split Threshold | ELEMENT | FALSE | UL Data Split Threshold IE in TS 37.483 [21] Section 9.3.1.43 |  |  |  |
| 26 | >>>PDCP Re-establishment | ELEMENT | FALSE | PDCP Re-establishment IE in TS 37.483 [21] Section 9.3.1.38 |  |  |  |
| 27 | >>>PDCP Data Recovery | ELEMENT | FALSE | PDCP Data Recovery IE in TS 37.483 [21] Section 9.3.1.38 |  |  |  |
| 28 | >>>Out-of-Order Delivery | ELEMENT | FALSE | Out of Order Delivery IE in TS 37.483 [21] Section 9.3.1.38 |  |  |  |
| 29 | >>>PDCP Status Report Indication | ELEMENT | FALSE | PDCP Status Report Indication IE in TS 37.483 [21] Section 9.3.1.38 |  |  |  |
| 30 | >>>Number of PDCP duplication | ELEMENT | FALSE | Additional PDCP duplication Information IE in TS 37.483 [21] Section 9.3.1.38 |  |  |  |
| 31 | >>>UL More than one RLC | STRUCTURE |  |  |  | moreThanOneRLC IE in TS 38.331 [22] Section 6 |  |
| 32 | >>>>Primary Path | STRUCTURE |  |  |  | primaryPath IE in TS 38.331 [22] Section 6 |  |
| 33 | >>>>>Cell Group ID | ELEMENT | TRUE | cellGroup IE in TS 38.331 [22] Section 6 |  |  |  |
| 34 | >>>>>Logical Channel ID | ELEMENT | FALSE | logicalChannel IE in TS 38.331 [22] Sec 6 |  |  |  |
| 35 | >>>UL More than two RLC | STRUCTURE |  |  |  | moreThanTwoRLC-DRB-r16 IE in TS 38.331 [22] Section 6 |  |
| 36 | >>>>Split Secondary Path | ELEMENT | FALSE | splitSecondaryPath IE in TS 38.331 [22] Sec 6 |  |  |  |
| 37 | >>>>Duplication State | ELEMENT | FALSE | duplicationState IE in TS 38.331 [22] Sec 6 |  |  |  |
| 38 | >>>DL More than one RLC | STRUCTURE |  |  |  | moreThanOneRLC IE in TS 38.331 [22] Section 6 |  |
| 39 | >>>>Primary Path | STRUCTURE |  |  |  | primaryPath IE in TS 38.331 [22] Section 6 |  |
| 49 | >>>>>Cell Group ID | ELEMENT | TRUE | cellGroup IE in TS 38.331 [22] Section 6 |  |  |  |
| 50 | >>>>>Logical Channel ID | ELEMENT | FALSE | logicalChannel IE in TS 38.331 [22] Sec 6 |  |  |  |
| 51 | >>>DL More than two RLC | STRUCTURE |  |  |  | moreThanTwoRLC-DRB-r16 IE in TS 38.331 [22] Section 6 |  |
| 52 | >>>>Split Secondary Path | ELEMENT | FALSE | splitSecondaryPath IE in TS 38.331 [22] Sec 6 |  |  |  |
| 53 | >>>>Duplication State | ELEMENT | FALSE | duplicationState IE in TS 38.331 [22] Sec 6 |  |  |  |
| 54 | List of Secondary cells to be setup | LIST |  |  |  | Scell To Be Setup List IE in TS 38.473 [19] Section 9.2.2.1 |
| 55 | >Secondary cell Item to be setup | STRUCTURE |  |  |  | Scell To Be Setup Item Ies IE in TS 38.473 [19] Section 9.2.2.1 |
| 56 | >>Secondary cell ID | ELEMENT | FALSE |  | Scell ID IE in TS 38.473 [19] Section 9.2.2.1 |  |

### 8.4.6	Dual Connectivity Control

#### 8.4.6.1	DC Secondary Node Addition Control

Upon receiving the RIC Control Request message, the E2 node shall invoke procedures related to DC Secondary Node Addition Control, such as Dual Connectivity Secondary Node Addition, UE Context Management, RRC Message Transfer, etc. and include the IEs corresponding to one or more of parameters described below in the related interface messages. If the Secondary Node ID is missing in the RIC Control Request message, the E2 node will send a RIC Control Failure.

| RAN Parameter ID | RAN Parameter | RAN Parameter Value Type | Key Flag | RAN Parameter Definition | Semantics Description |
| --- | --- | --- | --- | --- | --- |
| 1 | Secondary Node ID | STRUCTURE |  |  | Global NG-RAN Node ID IE in TS 38.423 [15] Section 9.2.2.3 |
| 2 | >CHOICE Secondary Node Type | STRUCTURE |  |  |  |
| 3 | >>Secondary Node gNB | STRUCTURE |  |  | Global ng-eNB ID IE in TS 38.423 [15] Section 9.2.2.1 |
| 4 | >>>PLMN Identity | ELEMENT | FALSE | PLMN Identity IE in TS 38.423 [15] Section 9.2.2.4 |  |
| 5 | >>>gNB ID | ELEMENT | FALSE | gNB ID IE in TS 38.423 [15] Section 9.2.2.1 |  |
| 6 | >>Secondary Node ng-eNB | STRUCTURE |  |  | Global ng-eNB ID IE in TS 38.423 [15] Section 9.2.2.2 |
| 7 | >>>PLMN Identity | ELEMENT | FALSE | PLMN Identity IE in TS 38.423 [15] Section 9.2.2.4 |  |
| 8 | >>>ng-eNB ID | ELEMENT | FALSE | Long Macro ng-eNB ID IE in TS 38.423 [15] Section 9.2.2.2 |  |
| 9 | >>Secondary Node en-gNB | STRUCTURE |  |  | en-gNB IE in TS 36.423 [17] Sec |
| 10 | >>>PLMN Identity | ELEMENT | FALSE | PLMN Identity IE in TS 36.423 [17] Section 9.2.4 |  |
| 11 | >>>en-gNB ID | ELEMENT | FALSE | Global en-gNB ID IE in TS 36.423 [17] Section 9.2.4 |  |
| 12 | List of PDU Sessions to be Added | LIST |  |  | PDU Session Resources To Be Added List IE in TS 38.423 [15] Section 9.1.2.1 |
| 13 | >PDU Session to be Added Item | STRUCTURE |  |  | PDU Session Resources To Be Added Item IE in TS 38.423 [15] Section 9.1.2.1 |
| 14 | >>PDU Session ID | ELEMENT | TRUE | PDU Session ID IE in TS 38.423 [15] Section 9.2.3.18 |  |
| 15 | >>PDU Session Resource Setup Info – SN-Terminated | STRUCTURE |  |  | PDU Session Resource Setup Info – SN Terminated IE in TS 38.423 [15] Section 9.2.1.5 |
| 16 | >>>QoS Flow To Be Setup List | LIST |  |  | QoS Flows To Be Setup List IE in TS 38.423 [15] Section 9.2.1.5 |
| 17 | >>>>QoS Flow Item | STRUCTURE |  |  | QoS Flows To Be Setup Item IE in TS 38.423 [15] Section 9.2.1.5 |
| 18 | >>>>>QoS Flow Identifier | ELEMENT | TRUE | QoS Flow Identifier IE in TS 38.423 [15] Section 9.2.3.10 |  |
| 19 | >>>>>QoS Flow Mapping Indication | ELEMENT | FALSE | QoS Flow Mapping Indication IE in TS 38.423 [15] Section 9.2.3.79 |  |
| 20 | >>>Default DRB allowed | ELEMENT | FALSE | Default DRB Allowed IE in TS 38.423 [15] Section 9.2.3.93 |  |
| 21 | >>PDU Session Resource Setup Info – MN-Terminated | STRUCTURE |  |  | PDU Session Resource Setup Info – SN Terminated IE in TS 38.423 [15] Section 9.2.1.7 |
| 22 | >>>DRBs To Be Setup List | STRUCTURE |  |  | DRBs To Be Setup List IE in TS 38.423 [15] Section 9.2.1.7 |
| 23 | >>>>DRB To Be Setup Item | STRUCTURE |  |  | DRBs To Be Setup Item IE in TS 38.423 [15] Section 9.2.1.7 |
| 24 | >>>>>DRB ID | ELEMENT | TRUE | DRB ID IE in TS 37.483 [21] Section 9.3.1.16 |  |
| 25 | >>>>>QoS Flow To Be Setup List | LIST |  |  | QoS Flows To Be Setup List IE in TS 38.423 [15] Section 9.2.1.5 |
| 26 | >>>>>>QoS Flow Item | STRUCTURE |  |  | QoS Flow Item IE in TS 38.423 [15] Section 9.2.3.10 |
| 27 | >>>>>>>QoS Flow Identifier | ELEMENT | TRUE | QoS Flow Identifier IE in TS 38.423 [15] Section 9.2.3.10 |  |
| 28 | >>>>>>>QoS Flow Mapping Indication | ELEMENT | FALSE | QoS Flow Mapping Indication IE in TS 38.423 [15] Section 9.2.3.79 |  |
| 29 | List of SN-terminated DRB IDs | LIST |  |  | Additional DRB IDs IE in TS 38.423 [15] Section 9.2.1.29 |
| 30 | >SN-Terminated DRB Item | STRUCTURE |  |  |  |
| 31 | >>DRB Identity | ELEMENT | TRUE | DRB ID IE in TS 38.473 [19] Section 9.3.1.8 |  |
| 32 | List of E-RABs to be Added | LIST |  |  | E-RABs To Be Added List IE in TS 36.423 [17] Section 9.1.4.1 |
| 33 | >E-RAB to be Added Item | STRUCTURE |  |  | E-RABs To Be Added Item IE in TS 36.423 [17] Section 9.1.4.1 |
| 34 | >>E-RAB ID | ELEMENT | TRUE | E-RAB ID IE in TS 36.423 [17] Section 9.2.23 |  |
| 35 | >>EN-DC Resource Configuration | STRUCTURE |  |  | EN-DC Resource Configuration IE in TS 36.423 [17] Section 9.2.108 |
| 36 | >>>PDCP at SgNB | ELEMENT | FALSE | PDCP at SgNB IE in TS 36.423 [17] Section 9.2.108 |  |
| 37 | >>>MCG Resources | ELEMENT | FALSE | MCG resources IE in TS 36.423 [17] Section 9.2.108 |  |
| 38 | >>>SCG Resources | ELEMENT | FALSE | SCG resources IE in TS 36.423 [17] Section 9.2.108 |  |

#### 8.4.6.2	DC Secondary Node Modification Control

Upon receiving the RIC Control Request message, the E2 node shall invoke procedures related to DC Secondary Node Modification Control, such as Dual Connectivity Secondary Node Modification, UE Context Management, RRC Message Transfer, etc. and include the IEs corresponding to one or more of parameters described below in the related interface messages.

| RAN Parameter ID | RAN Parameter | RAN Parameter Value Type | Key Flag | RAN Parameter Definition | Semantics Description |
| --- | --- | --- | --- | --- | --- |
| 1 | Secondary Node | STRUCTURE |  |  | Global NG-RAN Node ID IE in TS 38.423 [15] Section 9.2.2.3 |
| 2 | CHOICE Secondary Node Type | STRUCTURE |  |  |  |
| 3 | >Secondary Node gNB | STRUCTURE |  |  | Global gNB ID IE in TS 38.423 [15] Section 9.2.2.1 |
| 4 | >>PLMN Identity | ELEMENT | FALSE | PLMN Identity IE in TS 38.423 [15] Section 9.2.2.4 |  |
| 5 | >>gNB ID | ELEMENT | FALSE | gNB ID IE in TS 38.423 [15] Section 9.2.2.1 |  |
| 6 | >Secondary Node ng-eNB | STRUCTURE |  |  | Global ng-eNB ID IE in TS 38.423 [15] Section 9.2.2.2 |
| 7 | >>PLMN Identity | ELEMENT | FALSE | PLMN Identity IE in TS 38.423 [15] Section 9.2.2.4 |  |
| 8 | >>ng-eNB ID | ELEMENT | FALSE | Long Macro ng-eNB ID IE in TS 38.423 [15] Section 9.2.2.2 |  |
| 9 | >Secondary Node en-gNB | STRUCTURE |  |  | en-gNB IE in TS 36.423 [17] Sec |
| 10 | >>PLMN Identity | ELEMENT | FALSE | PLMN Identity IE in TS 36.423 [17] Section 9.2.4 |  |
| 11 | >>en-gNB ID | ELEMENT | FALSE | Global en-gNB ID IE in TS 36.423 [17] Section 9.2.4 |  |
| 12 | List of PDU Session Resources to be modified | LIST |  |  | PDU Session Resources To Be Modified List IE in TS 38.423 [15] Section 9.1.2.5 |
| 13 | >PDU Session Resource to be Modified Item | STRUCTURE |  |  | PDU Session Resources To Be Modified Item IE in TS 38.423 [15] Section 9.1.2.5 |
| 14 | >>PDU Session ID | ELEMENT | TRUE | PDU Session ID IE in TS 38.423 [15] Section 9.2.3.18 |  |
| 15 | >>PDU Session Resource Modification Info – SN-Terminated | STRUCTURE |  |  | PDU Session Resource Modification Info – SN Terminated IE in TS 38.423 [15] Section 9.2.1.9 |
| 16 | >>>QoS Flows To Be Setup List | LIST |  |  | QoS Flows To Be Setup List IE in TS 38.423 [15] Section 9.2.1.9 |
| 17 | >>>>QoS Flow Item | STRUCTURE |  |  | QoS Flows To Be Setup Item IE in TS 38.423 [15] Section 9.2.1.9 |
| 18 | >>>>>QoS Flow Identifier | ELEMENT | TRUE | QoS Flow Identifier IE in TS 38.423 [15] Section 9.2.3.10 |  |
| 19 | >>>>>QoS Flow Mapping Indication | ELEMENT | FALSE | QoS Flow Mapping Indication IE in TS 38.423 [15] Section 9.2.3.79 |  |
| 20 | >>>>>Redundant QoS Flow Indicator | ELEMENT | FALSE |  |  |
| 21 | >>>QoS Flows To Be Modified List | LIST |  |  | QoS Flows To Be Modified List IE in TS 38.423 [15] Section 9.2.1.9 |
| 22 | >>>>QoS Flow Item | STRUCTURE |  |  | QoS Flows To Be Modified Item IE in TS 38.423 [15] Section 9.2.1.9 |
| 23 | >>>>QoS Flow Identifier | ELEMENT | TRUE | QoS Flow Identifier IE in TS 38.423 [15] Section 9.2.3.10 |  |
| 24 | >>>>QoS Flow Mapping Indication | ELEMENT | FALSE | QoS Flow Mapping Indication IE in TS 38.423 [15] Section 9.2.3.79 |  |
| 25 | >>>>Redundant QoS Flow Indicator | ELEMENT | FALSE | Redundant QoS Flow Indicator IE in TS 38.423 [15] Section 9.2.3.118 |  |
| 26 | >>>QoS Flows To Be Released List | LIST |  |  | QoS Flows To Be Released List IE in TS 38.423 [15] Section 9.2.1.9 |
| 27 | >>>>QoS Flow Item | STRUCTURE |  |  | QoS Flow With Cause Item IE in TS 38.423 [15] Section 9.2.1.4 |
| 28 | >>>>QoS Flow Identifier | ELEMENT | TRUE | QoS Flow Identifier IE in TS 38.423 [15] Section 9.2.3.10 |  |
| 29 | >>>Default DRB allowed | ELEMENT | FALSE | Default DRB Allowed IE in TS 38.423 [15] Section 9.2.3.93 |  |
| 30 | >>PDU Session Resource Modification Info – MN-Terminated | STRUCTURE |  |  | PDU Session Resource Modification Info – MN Terminated IE in TS 38.423 [15] Section 9.2.1.11 |
| 31 | >>>DRBs to be setup list | LIST |  |  | DRBs To Be Setup List IE in TS 38.423 [15] Section 9.2.1.11 |
| 32 | >>>>DRBs to be setup item | STRUCTURE |  |  | DRBs To Be Setup Item IE in TS 38.423 [15] Section 9.2.1.11 |
| 33 | >>>>>DRB ID | ELEMENT | TRUE | DRB ID IE in TS 37.483 [21] Section 9.3.1.16 |  |
| 34 | >>>>>DRB QoS | STRUCTURE |  |  |  |
| 35 | >>>>>PDCP Duplication Activation | ELEMENT | TRUE | Duplication Activation IE in TS 37.483 [21] Section 9.2.3.71 |  |
| 36 | >>>>>QoS Flows Mapped to DRB List | LIST |  |  | QoS Flows Mapped to DRB List IE in TS 37.483 [21] Section 9.2.1.11 |
| 37 | >>>>>>QoS Flows Mapped to DRB Item | STRUCTURE |  |  | QoS Flows Mapped To DRB Item IE in TS 37.483 [21] Section 9.2.1.11 |
| 38 | >>>>>>>QoS Flow Identifier | ELEMENT | TRUE | QoS Flow Identifier IE in TS 38.423 [15] Section 9.2.3.10 |  |
| 39 | >>>>>>>QoS Flow Mapping Indication | ELEMENT | FALSE | QoS Flow Mapping Indication IE in TS 38.423 [15] Section 9.2.3.79 |  |
| 40 | >>>DRBs to be modified list | LIST |  |  | DRBs To Be Modified List IE in TS 37.483 [21] Section 9.2.1.11 |
| 41 | >>>>DRB to be modified item | STRUCTURE |  |  | DRBs To Be Modified Item IE in TS 37.483 [21] Section 9.2.1.11 |
| 42 | >>>>>DRB ID | ELEMENT | TRUE | DRB ID IE in TS 37.483 [21] Section 9.3.1.16 |  |
| 43 | >>>>>DRB QoS | STRUCTURE |  |  |  |
| 44 | >>>>>PDCP Duplication Configuration | ELEMENT | FALSE | PDCP Duplication Configuration IE in TS 37.483 [21] Section 9.2.3.86 |  |
| 45 | >>>>>PDCP Duplication Activation | ELEMENT | FALSE | Duplication Activation IE in TS 37.483 [21] Section 9.2.3.71 |  |
| 46 | >>>>>QoS Flows Mapped to DRB List | LIST |  |  | Flow Mapping Information IE in TS 37.483 [21] Section 9.3.1.26 |
| 47 | >>>>>>QoS Flows Mapped to DRB Item | STRUCTURE |  |  | QoS Flow Item IE in TS 37.483 [21] Section 9.3.1.25 |
| 48 | >>>>>>>QoS Flow Identifier | ELEMENT | TRUE | QoS Flow Identifier IE in TS 38.423 [15] Section 9.2.3.10 |  |
| 49 | >>>>>>>QoS Flow Mapping Indication | ELEMENT | FALSE | QoS Flow Mapping Indication IE in TS 38.423 [15] Section 9.2.3.79 |  |
| 50 | >>>DRBs to be released list | LIST |  |  | DRBs To Be Released List IE in TS 38.423 [15] Section 9.2.1.28 |
| 51 | >>>>DRB to be released item | STRUCTURE |  |  | DRBs To Be Setup Item IE in TS 38.423 [15] Section 9.2.1.11 |
| 52 | >>>>>DRB ID | ELEMENT | TRUE | DRB ID IE in TS 37.483 [21] Section 9.3.1.16 |  |
| 53 | List of PDU Session Resources to be deleted | LIST |  |  | PDU Session Resources To Be Released List IE in TS 38.423 [15] Section 9.1.2.5 |
| 54 | >PDU Session Resource to be deleted Item | STRUCTURE |  |  | PDU Session Resource To Be Released Item IE |
| 55 | >>PDU Session ID | ELEMENT | TRUE | PDU Session ID IE in TS 38.423 [15] Section 9.2.3.18 |  |
| 56 | List of E-RABs to be Modified | LIST |  |  | E-RABs To Be Modified List IE in TS 36.423 [17] Section 9.1.4.5 |
| 57 | >E-RAB to be Modified Item | STRUCTURE |  |  | E-RAB To Be Modified Item IE in TS 36.423 [17] Section 9.1.4.5 |
| 58 | >>E-RAB ID | ELEMENT | TRUE | E-RAB ID IE in TS 36.423 [17] Section 9.2.23 |  |
| 59 | >>EN-DC Resource Configuration | STRUCTURE |  |  | EN-DC Resource Configuration IE in TS 36.423 [17] Section 9.2.108 |
| 60 | >>>PDCP at SgNB | ELEMENT | FALSE | PDCP at SgNB IE in TS 36.423 [17] Section 9.2.108 |  |
| 61 | >>>MCG Resources | ELEMENT | FALSE | MCG resources IE in TS 36.423 [17] Section 9.2.108 |  |
| 62 | >>>SCG Resources | ELEMENT | FALSE | SCG resources IE in TS 36.423 [17] Section 9.2.108 |  |
| 63 | List of E-RABs to be Released | LIST |  |  | E-RABs To Be Released List IE in TS 36.423 [17] Section 9.1.4.5 |
| 64 | >E-RAB to be Released Item | STRUCTURE |  |  | E-RABs To Be Released Item IE in TS 36.423 [17] Section 9.1.4.5 |
| 65 | >>E-RAB ID | ELEMENT | TRUE | E-RAB ID IE in TS 36.423 [17] Section 9.2.23 |  |

#### 8.4.6.3	PSCell Change control for Secondary Cell Group

Upon receiving the RIC Control Request message, in the case of intra-SN PSCell change, the E2 node shall invoke procedures related to PSCell Change Control, such as Dual Connectivity Secondary Node Modification, UE Context Modification, RRC Message Transfer, etc. In the case of inter-SN PSCell change, the E2 node shall invoke procedures such as Dual Connectivity Secondary Node Release (MN/SN-initiated) or SN Change (only SN-initiated), Bearer Context Modification, UE Context Modification, RRC Message Transfer, etc. The E2 node then includes the IEs corresponding to one or more of parameters described below in the related interface messages.

| RAN Parameter ID | RAN Parameter | RAN Parameter Value Type | Key Flag | RAN Parameter Definition |  | Semantics Description |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Target PSCell ID | STRUCTURE |  |  |  | Target Cell Global ID IE in TS 38.423 [15] Section 9.2.3.25 |
| 2 | >CHOICE Target Cell | STRUCTURE |  |  |  | Target Cell IE in TS 38.423 [15] Section 9.2.3.25 |
| 3 | >>NR Cell | STRUCTURE |  |  |  | NR IE in TS 38.423 [15] Section 9.2.3.25 |
| 4 | >>>NR CGI | ELEMENT | FALSE | NR CGI IE in TS 38.423 [15] Section 9.2.2.7 |  |  |
| 5 | >>E-UTRA Cell | STRUCTURE |  |  |  | E-UTRA IE in TS 38.423 [15] Section 9.2.3.25 |
| 6 | >>>E-UTRA CGI | ELEMENT | FALSE | E-UTRA CGI IE in TS 38.423 [15] Section 9.2.2.8 |  |  |
| 7 | Secondary Node ID | STRUCTURE |  |  |  | Global NG-RAN Node ID IE in TS 38.423 [15] Section 9.2.2.3 |
| 8 | >CHOICE Secondary Node Type | STRUCTURE |  |  |  |  |
| 9 | >>Secondary Node gNB | STRUCTURE |  |  |  | Global gNB ID IE in TS 38.423 [15] Section 9.2.2.1 |
| 10 | >>>PLMN Identity | ELEMENT | FALSE | PLMN Identity IE in TS 38.423 [15] Section 9.2.2.4 |  |  |
| 11 | >>>gNB ID | ELEMENT | FALSE | gNB ID IE in TS 38.423 [15] Section 9.2.2.1 |  |  |
| 12 | >>Secondary Node ng-eNB | STRUCTURE |  |  |  | Global ng-eNB ID IE in TS 38.423 [15] Section 9.2.2.2 |
| 13 | >>>PLMN Identity | ELEMENT | FALSE | PLMN Identity IE in TS 38.423 [15] Section 9.2.2.4 |  |  |
| 14 | >>>ng-eNB ID | ELEMENT | FALSE | Long Macro ng-eNB ID IE in TS 38.423 [15] Section 9.2.2.2 |  |  |
| 15 | >>Secondary Node en-gNB | STRUCTURE |  |  |  | en-gNB IE in TS 36.423 [17] Sec |
| 16 | >>>PLMN Identity | ELEMENT | FALSE | PLMN Identity IE in TS 36.423 [17] Section 9.2.4 |  |  |
| 17 | >>>en-gNB ID | ELEMENT | FALSE | Global en-gNB ID IE in TS 36.423 [17] Section 9.2.4 |  |  |
| 18 | PDU Session SN Change Required List | LIST |  |  |  | PDU Session SN Change Required List IE in TS 38.423 [15] Section 9.1.2.11 |
| 19 | >PDU Session SN Change Required Item | STRUCTURE |  |  |  | PDU Session SN Change Required Item IE in TS 38.423 [15] Section 9.1.2.11 |
| 20 | >>PDU Session ID | ELEMENT | TRUE | PDU Session ID IE in TS 38.423 [15] Section 9.2.3.18 |  |  |
| 21 | >>List of QoS flows in the PDU session | LIST |  |  |  | QoS Flows To Be Setup List IE in TS 38.423 [15] Section 9.2.1.1 |
| 22 | >>>QoS flow Item | STRUCTURE |  |  |  | QoS Flow To Be Setup Item IE in TS 38.423 [15] Section 9.2.1.1 |
| 23 | >>>>QoS Flow Identifier | ELEMENT | TRUE |  | QoS Flow Identifier IE in TS 38.423 [15] Section 9.2.3.10 |  |
| 24 | List of DRBs for PSCell handover | LIST |  |  |  | DRB to Be Setup List IE in TS 38.473 [19] Section 9.2.2.1 |
| 25 | >DRB item for PSCell handover | STRUCTURE |  |  |  | DRB to Be Setup Item IE in TS 38.473 [19] Section 9.2.2.1 |
| 26 | >>DRB ID | ELEMENT | TRUE | DRB ID IE in TS 38.473 [19] Section 9.3.1.8 |  |  |
| 27 | >>List of QoS flows in the DRB | LIST |  |  |  | QoS Flows Information To Be Setup IE in TS 37.483 [21] Section 9.3.3.2 |
| 28 | >>>QoS flow Item | STRUCTURE |  |  |  | QoS Flow Item IE in TS 37.483 [21] Section 9.3.1.25 |
| 29 | >>>>QoS flow Identifier | ELEMENT | TRUE | QoS Flow Identifier IE in TS 37.483 [21] Section 9.3.1.25 |  |  |

#### 8.4.6.4	DC Secondary Node Change Control

Upon receiving the RIC Control Request message, the E2 node shall invoke procedures such as Dual Connectivity Secondary Node Release (MN/SN-initiated) or SN Change (only SN-initiated), Bearer Context Modification, UE Context Modification, RRC Message Transfer, etc. and includes the IEs corresponding to one or more of parameters described below in the related interface messages.

| RAN Parameter ID | RAN Parameter | RAN Parameter Value Type | Key Flag | RAN Parameter Definition |  | Semantics Description |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Target PSCell ID | STRUCTURE |  |  |  | Target Cell Global ID IE in TS 38.423 [15] Section 9.2.3.25 |  |
| 2 | >CHOICE Target Cell | STRUCTURE |  |  |  | Target Cell IE in TS 38.423 [15] Section 9.2.3.25 |  |
| 3 | >>NR Cell | STRUCTURE |  |  |  | NR IE in TS 38.423 [15] Section 9.2.3.25 |  |
| 4 | >>>NR CGI | ELEMENT | FALSE | NR CGI IE in TS 38.423 [15] Section 9.2.2.7 |  |  |  |
| 5 | >>E-UTRA Cell | STRUCTURE |  |  |  | E-UTRA IE in TS 38.423 [15] Section 9.2.3.25 |  |
| 6 | >>>E-UTRA CGI | ELEMENT | FALSE | E-UTRA CGI IE in TS 38.423 [15] Section 9.2.2.8 |  |  |  |
| 7 | Target Secondary Node ID | STRUCTURE |  |  |  | Target S-NG-RAN node ID IE in TS 38.423 [15] Section 9.1.2.11 |  |
| 8 | >CHOICE Target Secondary Node Type | STRUCTURE |  |  |  |  |  |
| 9 | >>Secondary Node gNB | STRUCTURE |  |  |  | Global gNB ID IE in TS 38.423 [15] Section 9.2.2.1 |  |
| 10 | >>>PLMN Identity | ELEMENT | FALSE | PLMN Identity IE in TS 38.423 [15] Section 9.2.2.4 |  |  |  |
| 11 | >>>gNB ID | ELEMENT | FALSE | gNB ID IE in TS 38.423 [15] Section 9.2.2.1 |  |  |  |
| 12 | >>Secondary Node ng-eNB | STRUCTURE |  |  |  | Global ng-eNB ID IE in TS 38.423 [15] Section 9.2.2.2 |  |
| 13 | >>>PLMN Identity | ELEMENT | FALSE | PLMN Identity IE in TS 38.423 [15] Section 9.2.2.4 |  |  |  |
| 14 | >>>ng-eNB ID | ELEMENT | FALSE | Long Macro ng-eNB ID IE in TS 38.423 [15] Section 9.2.2.2 |  |  |  |
| 15 | >>Secondary Node en-gNB | STRUCTURE |  |  |  | en-gNB IE in TS 36.423 [17] Sec |  |
| 16 | >>>PLMN Identity | ELEMENT | FALSE | PLMN Identity IE in TS 36.423 [17] Section 9.2.4 |  |  |  |
| 17 | >>>en-gNB ID | ELEMENT | FALSE | Global en-gNB ID IE in TS 36.423 [17] Section 9.2.4 |  |  |  |
| 18 | PDU Session SN Change Required List | LIST |  |  |  | PDU Session SN Change Required List IE in TS 38.423 [15] Section 9.1.2.11 |  |
| 19 | >PDU Session SN Change Required Item | STRUCTURE |  |  |  | PDU Session SN Change Required Item IE in TS 38.423 [15] Section 9.1.2.11 |  |
| 20 | >>PDU Session ID | ELEMENT | TRUE | PDU Session ID IE in TS 38.423 [15] Section 9.2.3.18 |  |  |  |
| 21 | >>List of QoS flows in the PDU session | LIST |  |  |  |  | QoS Flows To Be Setup List IE in TS 38.423 [15] Section 9.2.1.1 |
| 22 | >>>QoS flow Item | STRUCTURE |  |  |  |  | QoS Flow To Be Setup Item IE in TS 38.423 [15] Section 9.2.1.1 |
| 23 | >>>>QoS Flow Identifier | ELEMENT | TRUE |  | QoS Flow Identifier IE in TS 38.423 [15] Section 9.2.3.10 |  |  |
| 24 | List of DRBs for PSCell handover | LIST |  |  |  | DRB to Be Setup List IE in TS 38.473 [19] Section 9.2.2.1 |  |
| 25 | >DRB item for PSCell handover | STRUCTURE |  |  |  | DRB to Be Setup Item IE in TS 38.473 [19] Section 9.2.2.1 |  |
| 26 | >>DRB ID | ELEMENT | TRUE | DRB ID IE in TS 38.473 [19] Section 9.3.1.8 |  |  |  |
| 27 | >>List of QoS flows in the DRB | LIST |  |  |  | QoS Flows Information To Be Setup IE in TS 37.483 [21] Section 9.3.3.2 |  |
| 28 | >>>QoS flow Item | STRUCTURE |  |  |  | QoS Flow Item IE in TS 37.483 [21] Section 9.3.1.25 |  |
| 29 | >>>>QoS flow Identifier | ELEMENT | TRUE | QoS Flow Identifier IE in TS 37.483 [21] Section 9.3.1.25 |  |  |  |

### 8.4.7	Carrier Aggregation Control

#### 8.4.7.1	Secondary cell Addition control

Upon receiving the RIC Control Request message, the E2 node shall invoke procedures related to Secondary cell Addition Control, such as UE Context Management, RRC Message Transfer, etc. and include the IEs corresponding to one or more of parameters described below in the related interface messages.

| RAN Parameter ID | RAN Parameter | RAN Parameter Value Type | Key Flag | RAN Parameter Definition | Semantics Description |
| --- | --- | --- | --- | --- | --- |
| 1 | List of secondary cells to be setup | LIST |  |  | Scell To Be Setup List IE in TS 38.473 [19] Section 9.2.2.1 |
| 2 | >Secondary cell to be setup Item | STRUCTURE |  |  | Scell To Be Setup Item Ies IE in TS 38.473 [19] Section 9.2.2.1 |
| 3 | >>Scell Cell Global ID | ELEMENT | FALSE | Scell ID IE in TS 38.473 [19] Section 9.2.2.1 |  |
| 4 | >>Scell UL Configured | ELEMENT | FALSE | Cell UL Configured IE in TS 38.473 [19] Section 9.3.1.33 |  |
| 5 | List of DRBs to be served using SCells | LIST |  |  | DRB to Be Setup List IE in TS 38.473 [19] Section 9.2.2.1 |
| 6 | >DRB item | STRUCTURE |  |  | DRB to Be Setup Item IE in TS 38.473 [19] Section 9.2.2.1 |
| 7 | >>DRB ID | ELEMENT | TRUE | DRB ID IE in TS 38.473 [19] Section 9.3.1.8 |  |
| 8 | >>List of QoS flows in the DRB | LIST |  |  | QoS Flows Information To Be Setup IE in TS 37.483 [21] Section 9.3.3.2 |
| 9 | >>>QoS flow Item | STRUCTURE |  |  | QoS Flow Item IE in TS 37.483 [21] Section 9.3.1.25 |
| 10 | >>>>QoS flow Identifier | ELEMENT | TRUE | QoS Flow Identifier IE in TS 37.483 [21] Section 9.3.1.25 |  |

#### 8.4.7.2	Secondary cell Modification control

Upon receiving the RIC Control Request message, the E2 node shall invoke procedures related to Secondary cell Modification Control, such as UE Context Management, RRC Message Transfer, etc. and include the IEs corresponding to one or more of parameters described below in the related interface messages.

| RAN Parameter ID | RAN Parameter | RAN Parameter Value Type | Key Flag | RAN Parameter Definition | Semantics Description |
| --- | --- | --- | --- | --- | --- |
| 1 | List of secondary cells to be setup | LIST |  |  | Scell To Be Setup List IE in TS 38.473 [19] Section 9.2.2.1 |
| 2 | >Secondary cell to be setup Item | STRUCTURE |  |  | Scell To Be Setup Item Ies IE in TS 38.473 [19] Section 9.2.2.1 |
| 3 | >>Scell Global ID | ELEMENT | FALSE | Scell ID IE in TS 38.473 [19] Section 9.2.2.1 |  |
| 4 | >>Scell UL Configured | ELEMENT | FALSE | Cell UL Configured IE in TS 38.473 [19] Section 9.3.1.33 |  |
| 5 | List of Scells to be removed | LIST |  |  | SCell To Be Removed List IE in TS 38.473 [19] Section 9.2.2.7 |
| 6 | >Scell to be removed Item | STRUCTURE |  |  | SCell to Be Removed Item IEs IE in TS 38.473 [19] Section 9.2.2.7 |
| 7 | >>Scell Global ID | ELEMENT | FALSE | Scell ID IE in TS 38.473 [19] Section 9.2.2.1 |  |

#### 8.5.2.6	DRB Split Ratio Control

##### 8.5.2.6.1	Control action

The RAN parameters associated with this policy type are listed in Section 8.4.2.6 Table 1.

##### 8.5.2.6.2	Offset

| RAN Parameter ID | RAN Parameter | RAN Parameter Value Type | Key Flag | RAN Parameter Definition | Semantics Description |
| --- | --- | --- | --- | --- | --- |
| 101 | Uplink PDCP Data Split Threshold offset | ELEMENT | FALSE | INTEGER | UL Data Split Threshold IE in TS 37.483 [21] Section 9.3.1.43 |
| 102 | Downlink PDCP Data Split | ELEMENT | FALSE | INTEGER | Downlink PDCP Data Split IE in 8.4.2.6 |

********** END OF MODIFIED SECTION**********

********** NEXT MODIFIED SECTION **********

********** END OF MODIFIED SECTION**********

********** NEXT MODIFIED SECTION **********

********** END OF MODIFIED SECTION**********