| CR Number | Date | Specification Base Version |
| --- | --- | --- |
| QCOM-20191028-01 | 11th November 2019 | GTI v2.5 |
| Author |  | Change type |
| Qualcomm |  | 2. Modification |
| CR Title, |  |  |
| Add CSI-RS/TRS config to default test configuration |  |  |
| Sections/Clauses affected |  |  |
| 4.1.2.2 |  |  |
| Fill in the following sections if the CR is proposing modifications. Otherwise, leave blank. |  |  |
| Summary of change |  |  |
| 1. Add CSI-RS/TRS config to default test configuration in 4.1.2.2 |  |  |
| Reason for change |  |  |
| Correct/optimize test cases |  |  |
| Consequence if the change is not agreed |  |  |
| The test cases will fail to reflect test requirement |  |  |
| Any other supporting information |  |  |
| Give any additional information not covered above |  |  |

Test Environment

Cell Configuration

Test Frequencies

Table 4-1: Test Frequencies for NR TDD

| Operating Band | Frequency Configuration | Occupied Bandwidth | Range (F_low – F_high) |
| --- | --- | --- | --- |
| n41 | f1 | 100 MHz | 2515M - 2615M |
|  | f2 | 80 MHz | 2515M - 2595M |
|  | f3 | 60 MHz | 2515M - 2575M |
|  | f4 | 100 MHz | 2575M - 2675M |
| n79 | f1 | 100 MHz | 4800M - 4900M |

Table 4-2: Test Frequencies for E-UTRA TDD

| Operating Band | Frequency Configuration | Occupied Bandwidth | Range (F_low – F_high) |
| --- | --- | --- | --- |
| 34 | f1 | 15 MHz | 2010M – 2025M |
| 39 | f1 | 20 MHz | 1880M - 1900M |
|  | f2 | 10 MHz | 1900M - 1910M |
|  | f3 | 15 MHz | 1900M - 1915M |
|  | f4 | 10 MHz | 1905M - 1915M |
| 40 | f1 | 20 MHz | 2325M - 2345M |
|  | f2 | 20 MHz | 2345M - 2365M |
| 41 | f1 | 20 MHz | 2615M - 2635M |
|  | f2 | 20 MHz | 2635M - 2655M |
|  | f3 | 20 MHz | 2655M - 2675M |

Table 4-3: Test Frequencies for E-UTRA FDD

| Operating Band | Frequency Configuration | Occupied Bandwidth | Range (F_low – F_high) |
| --- | --- | --- | --- |
| 3 | f1 | 10 MHz | UL: 1710M - 1720M DL: 1805M - 1815M |
|  | f2 | 15 MHz | UL: 1720M - 1735M DL: 1815M - 1830M |
|  | f3 | 20 MHz | UL: 1710M - 1730M DL: 1805M - 1825M |

Table 4-4: Band combinations for NSA option3 (EN-DC, two bands)

| Band Combinations | E-UTRA Band | NR Band | Note |
| --- | --- | --- | --- |
| DC_3A_n41A | 3A | n41A | Inter-band EN-DC |
| DC_39A_n41A | 39A | n41A | Inter-band EN-DC |
| DC_3A_n79A | 3A | N79A | Inter-band EN-DC |
| DC_39A_n79A | 39A | N79A | Inter-band EN-DC |

Table 4-5: Test Frequencies for E-UTRA FDD

| GSM Band | Bandwidth | f1(Mid) | f2(High) | f3(Low) |
| --- | --- | --- | --- | --- |
| Band 8 (GSM 900) | 25MHz | 20 (UL:894MHz/ DL:939MHz) | 110 (UL:912MHz/ DL:957MHz) | 5 (UL:891MHz/ DL:936MHz) |
| Band 3 (DCS 1800) | 75MHz | 590 (UL:1725.8MHz/ DL:1820.8MHz) | 700 (UL:1747.8MHz/ DL:1842.8MHz) | 515 (UL:1710.8MHz/ DL:1805.8MHz) |

Default Configuration

If not explicitly specified in the test case prose, the following Cell Configuration parameters shall be used for NR cells in the test cases

Table 4-4: Default Parameters

| Parameters | Value | Note |
| --- | --- | --- |
| MCC | 460 |  |
| MNC | 00 |  |
| NR Frame Structure for n41 | Uplink-Downlink Switch Period: 5ms | DD DD DD DS UU |
|  | Uplink-Downlink Switch Period: 3ms+2ms | DD DS UU DD DD |
| NR Frame Structure for n79 | Uplink-Downlink Switch Period: 2.5 ms | DSUUU DSUUU |
| Special Frame for n41 | DL : GP : UL = 6 : 4 : 4 | DDDDDDGGGGUUUU |
| Special Frame for n79 | DL : GP : UL = 10 : 2 : 2 | DDDDDDDDDDGGUU |
| CP Length | Normal CP |  |
| PRACH Format | Format 0 |  |
| PRACH Period | 10ms |  |
| PUCCH Format | Format 0 / Format 1 |  |
|  | Format 2 / Format 3 |  |
| PBCH SCS | 30kHz |  |
| PBCH Period | 20ms |  |
| PDCCH Symbols | 1 symbol |  |
| PDCCH Format | Format 0_1/Format 1_1 |  |
| PDSCH DMRS | Mapping type A & Type1 |  |
| PUSCH DMRS | Mapping type A & Type1 |  |
| PUSCH Transmission | The codebook-based transmission mode |  |
| UL Power Control | ON |  |
| HARQ | ON |  |
| SRS | NR SRS Switching |  |
| MIMO | NSA: NR 1T4R, LTE 1T4R or 1T2R |  |
|  | SA: 2T4R |  |
| UE Maximum TX Power | NSA: 26 dBm |  |
|  | SA: 26 dBm |  |
| Waveform | Uplink : CP-OFDM |  |
|  |  | Downlink: CP-OFDM |  |  |  |
|  |  | Subcarrier indexes in the PRB used for CSI-RS |  | k0 = 3 for CSI-RS resource 1,2,3,4 |  |
|  |  | OFDM symbols in the PRB used for CSI-RS |  | l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 |  |
|  |  | Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |  |
|  |  | CDM Type |  | ‘No CDM’ for CSI-RS resource 1,2,3,4 |  |
|  |  | Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |  |
| CSI-RS for tracking |  | CSI-RS periodicity |  | 15 kHz SCS: 20 for CSI-RS resource 1,2,3,4 30 kHz SCS: 40 for CSI-RS resource 1,2,3,4 |  |
|  |  | CSI-RS offset |  | 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |  |
|  |  | Frequency Occupation |  | Start PRB 0 Number of PRB = BWP size |  |
|  |  | QCL info |  | TCI state #0 |  |
|  |  | Subcarrier indexes in the PRB used for CSI-RS |  | k0 = 4 |  |
|  |  | OFDM symbols in the PRB used for CSI-RS |  | l0 = 12 |  |
|  |  | Number of CSI-RS ports (X) |  | Same as number of transmit antenna |  |
|  |  | CDM Type |  | ‘FD-CDM2’ |  |
| NZP CSI-RS for CSI acquisition |  | Density (ρ) |  | 1 |  |
|  |  | CSI-RS periodicity |  | 15 kHz SCS: 20 30 kHz SCS: 40 |  |
|  |  | CSI-RS offset |  | 0 |  |
|  |  | Frequency Occupation |  | Start PRB 0 Number of PRB = BWP size |  |
|  |  | QCL info |  | TCI state #1 |  |